# Upstream Fisheries Impact Assessment Quantitative Methods

This appendix summarizes fisheries impact assessment quantitative methods related to upstream temperature, spawning habitat weighted usable area, redd dewatering, redd scour/entombment, the SALMOD model, rearing flows, rearing habitat weighted usable area, juvenile stranding, and upstream migration of salmon and sturgeon adults. Other fisheries impact assessment quantitative methods are discussed in various other appendices.

## Upstream Temperature Methods

### Introduction

For the Sacramento River and American River, the water temperature analysis was completed utilizing daily modeled water temperature outputs from the HEC-5Q model, in addition to monthly modeled water temperature outputs from the Reclamation Temperature Model for the Feather River. Because there would be negligible or no differences in instream flows between each alternative and the NAA in the Trinity, Stanislaus, and San Joaquin Rivers and in Clear Creek, and because water temperatures can only be affected by differences in flow between alternatives, no temperature modeling was needed to conclude that there would be no temperature-related effects on aquatic resources in these waterways.

There were multiple methods used in this effects analysis to determine whether there would be effects of the Project on aquatic resources. The methods vary by river, race/species, and life stage (Table 11B-1). The first analysis evaluated the results of physical water temperature models that overlapped fish presence in space and time to assess potential water temperature-related effects to aquatic resources. The second analysis determined the frequency and magnitude either above one or more water temperature index values or outside one or more water temperature index ranges for each life stage, race/species, and location. The third and fourth methods involved an evaluation of water temperature-related mortality in the Sacramento River using the Martin and Anderson Egg Mortality Models for winter-run Chinook salmon and SALMOD for all races of Chinook salmon.

No water temperature analyses were conducted for the Trinity River, Stanislaus River, San Joaquin River, and Clear Creek because preliminary review of the CalSim II flow outputs indicated that there were negligible differences in flows between the NAA and all alternatives in these waterways (Appendix 5B2, River Operations). The only water temperature model inputs affected by the alternatives would be flow. Therefore, because difference in flows would be negligible, difference in water temperatures would be negligible.

Table 11B-1. Water Temperature Analysis Methods Used in Each River, Species, and Life Stage

| Life stage(s) | Method Used |  |  |  |
| --- | --- | --- | --- | --- |
|  | Physical Model Output Characterization | Water Temperature Index Value/Range Analysis | Martin and Anderson Egg Mortality Models | SALMOD – Temperature-Related Mortality |
| Sacramento River |  |  |  |  |
| Winter-run Chinook Salmon |  |  |  |  |
| Spawning, egg incubation, and alevins | X | X | X | X |
| Fry and juvenile rearing | X | X |  | X |
| Juvenile emigration | X | X |  |  |
| Adult immigration | X | X |  |  |
| Adult holding | X | X |  |  |
| Spring-run Chinook Salmon |  |  |  |  |
| Spawning, egg incubation, and alevins | X | X |  | X |
| Fry and juvenile rearing | X | X |  | X |
| Juvenile emigration | X | X |  |  |
| Adult immigration | X | X |  |  |
| Adult holding | X | X |  |  |
| Fall-/Late Fall-run Chinook Salmon |  |  |  |  |
| Spawning, egg incubation, and alevins | X | X |  | X |
| Fry and juvenile rearing | X | X |  | X |
| Juvenile emigration | X | X |  |  |
| Adult immigration | X | X |  |  |
| Adult holding | X | X |  |  |
| Steelhead |  |  |  |  |
| Spawning, egg incubation, and alevins | X | X |  |  |
| Kelt emigration | X | X |  |  |
| Juvenile rearing | X | X |  |  |
| Smolt emigration (not migrant parr) | X | X |  |  |
| Adult immigration | X | X |  |  |
| Adult holding | X | X |  |  |
| Green Sturgeon |  |  |  |  |
| Spawning and egg incubation | X | X |  |  |
| Pre- and post-spawn adult holding | X | X |  |  |
| Post-spawn emigration | X | X |  |  |
| Larval to Juvenile rearing and emigration | X | X |  |  |
| Adult immigration | X | X |  |  |
| White Sturgeon |  |  |  |  |
| Spawning and egg incubation | X | X |  |  |
| Juvenile rearing and emigration | X | X |  |  |
| Adult immigration and holding | X | X |  |  |
| Pacific Lamprey |  |  |  |  |
| Spawning and egg incubation | X | X |  |  |
| Ammocoete rearing and emigration | X | X |  |  |
| River Lamprey |  |  |  |  |
| Spawning and egg incubation | X | X |  |  |
| Ammocoete rearing and emigration | X | X |  |  |
| Hardhead |  |  |  |  |
| Non-spawning life stages | X | X |  |  |
| Spawning | X | X |  |  |
| Sacramento Hitch |  |  |  |  |
| Spawning | X | X |  |  |
| Sacramento Splittail |  |  |  |  |
| Spawning | X | X |  |  |
| Striped Bass |  |  |  |  |
| Spawning, embryo incubation, and initial rearing | X | X |  |  |
| Larvae, fry, and juvenile rearing and emigration | X | X |  |  |
| American Shad |  |  |  |  |
| Spawning, embryo incubation, and initial rearing | X | X |  |  |
| Larvae, fry, and juvenile rearing and emigration | X | X |  |  |
| Largemouth Bass |  |  |  |  |
| Spawning | X | X |  |  |
| Feather River |  |  |  |  |
| Winter-run Chinook Salmon |  |  |  |  |
| Non-natal rearing | X | X |  |  |
| Spring-run Chinook Salmon |  |  |  |  |
| Spawning, egg incubation, and alevins | X | X |  |  |
| Fry and juvenile rearing | X | X |  |  |
| Juvenile emigration | X | X |  |  |
| Adult immigration | X | X |  |  |
| Adult holding | X | X |  |  |
| Fall-run Chinook Salmon |  |  |  |  |
| Spawning, egg incubation, and alevins | X | X |  |  |
| Fry and juvenile rearing | X | X |  |  |
| Juvenile emigration | X | X |  |  |
| Adult immigration | X | X |  |  |
| Adult holding | X | X |  |  |
| Steelhead |  |  |  |  |
| Spawning, egg incubation, and alevins | X | X |  |  |
| Kelt emigration | X | X |  |  |
| Juvenile rearing | X | X |  |  |
| Smolt emigration | X | X |  |  |
| Adult immigration | X | X |  |  |
| Adult holding | X | X |  |  |
| Green Sturgeon |  |  |  |  |
| Spawning, egg incubation | X | X |  |  |
| Pre- and post-spawn adult holding | X | X |  |  |
| Post-spawn emigration | X | X |  |  |
| Larval to Juvenile rearing and emigration | X | X |  |  |
| Adult immigration | X | X |  |  |
| White Sturgeon |  |  |  |  |
| Spawning and egg incubation | X | X |  |  |
| Juvenile rearing and emigration | X | X |  |  |
| Adult immigration and holding | X | X |  |  |
| Pacific Lamprey |  |  |  |  |
| Spawning and egg incubation | X | X |  |  |
| Ammocoete rearing and emigration | X | X |  |  |
| River Lamprey |  |  |  |  |
| Spawning and egg incubation | X | X |  |  |
| Ammocoete rearing and emigration | X | X |  |  |
| Hardhead |  |  |  |  |
| Non-spawning life stages | X | X |  |  |
| spawning | X | X |  |  |
| Sacramento Hitch |  |  |  |  |
| Spawning | X | X |  |  |
| Sacramento Splittail |  |  |  |  |
| Spawning | X | X |  |  |
| Striped Bass |  |  |  |  |
| Spawning, embryo incubation, and initial rearing | X | X |  |  |
| Larvae, fry, and juvenile rearing and emigration | X | X |  |  |
| American Shad |  |  |  |  |
| Spawning, embryo incubation, and initial rearing | X | X |  |  |
| Larvae, fry, and juvenile rearing and emigration | X | X |  |  |
| Largemouth Bass |  |  |  |  |
| Spawning | X | X |  |  |
| American River |  |  |  |  |
| Winter-run Chinook Salmon |  |  |  |  |
| Non-natal rearing | X | X |  |  |
| Steelhead |  |  |  |  |
| Spawning, egg incubation, and alevins | X | X |  |  |
| Kelt emigration | X | X |  |  |
| Juvenile rearing | X | X |  |  |
| Smolt emigration | X | X |  |  |
| Adult immigration | X | X |  |  |
| Adult holding | X | X |  |  |
| Fall-run Chinook Salmon |  |  |  |  |
| Spawning, egg incubation, and alevins | X | X |  |  |
| Fry and juvenile rearing | X | X |  |  |
| Juvenile emigration | X | X |  |  |
| Adult immigration | X | X |  |  |
| Adult holding | X | X |  |  |
| Pacific Lamprey |  |  |  |  |
| Spawning and egg incubation | X | X |  |  |
| Ammocoete rearing and emigration | X | X |  |  |
| River Lamprey |  |  |  |  |
| Spawning and egg incubation | X | X |  |  |
| Ammocoete rearing and emigration | X | X |  |  |
| Hardhead |  |  |  |  |
| Non-spawning life stages | X | X |  |  |
| Spawning | X | X |  |  |
| Sacramento Hitch |  |  |  |  |
| Spawning | X | X |  |  |
| Sacramento Splittail |  |  |  |  |
| Spawning | X | X |  |  |
| Striped Bass |  |  |  |  |
| Spawning, embryo incubation, and initial rearing | X | X |  |  |
| Larvae, fry, and juvenile rearing and emigration | X | X |  |  |
| American Shad |  |  |  |  |
| Spawning, embryo incubation, and initial rearing | X | X |  |  |
| Larvae, fry, and juvenile rearing and emigration | X | X |  |  |
| Largemouth Bass |  |  |  |  |
| Spawning | X | X |  |  |

### Detailed Methods

#### Physical Model Output Characterization

Patterns in water temperatures at key locations within the Sacramento, Feather, and American rivers were evaluated for each month that a life stage of each race/species was present and were summarized at the beginning of the water temperature section for each impact statement. The purpose of this characterization was to identify whether there were any locations, months, or water year types in which differences in water temperatures between the NAA and each alternative could potentially cause an effect. It included an evaluation between the NAA and each alternative of exceedance plots of mean monthly water temperature by month and comparisons of exceedance values, long-term averages, and average water temperatures by month and water year type, all of which is reported in Appendix 6C, River Temperature Modeling. If a specific result appeared concerning based on best professional judgment, the month, water year type, and location with the concerning result was flagged as requiring close examination in the results of the remaining water temperature evaluation. In addition, specifics of the month, water year type, and location with the concerning result were closely reviewed to determine the cause of the result and to determine whether the modeled effect could be avoided during real time operations.

#### Water Temperature Threshold Analysis

This analysis determined the frequency and magnitude of exceedance above one or more water temperature index values or outside one or more index ranges obtained from the scientific literature and USEPA guidance (USEPA 2003) for each race/species and life stage at multiple locations within the Sacramento River (Table 11B-2), Feather River (Table 11B-3), and American River (Table 11B-4). These index values and index ranges typically characterize the suitable, optimal, acceptable, and observed temperature range needed for survival, growth, or presence. The list of index values for salmonids and green sturgeon was originally compiled to assess potential upstream water temperature-related effects for the California WaterFix Section 7 consultation (NMFS 2016). The list of index values and ranges for other species were primarily taken from the 2017 Sites DEIR/DEIS (Sites Project Authority and US Bureau of Reclamation 2017), Appendix 12D, Water Temperature Index Value Selection Rationale, with supplemental information taken from the scientific literature as necessary.

For fish species not listed under the ESA or CESA, the frequency of exceedance above one or more water temperature index values or outside one or more index ranges was evaluated. For ESA-/CESA-listed fish species, both the frequency and magnitude of exceedance above water temperature index values was evaluated. Although sufficient information is available using only the frequency of exceedance to assess potential significant impacts or adverse effects for NEPA/CEQA purposes, NMFS has previously requested an analysis of both the frequency and magnitude of exceedance for Section 7 purposes because it provides additional information used in their jeopardy/adverse modification opinion. Therefore, this enhanced analysis was conducted for listed salmonids (plus fall-/late fall-run Chinook salmon) and green sturgeon for the Sites Reservoir Biological Assessment. Thus, the results of this analysis were available for this EIR/EIS.

Because USEPA (2003) criteria are provided as seven-day average daily maximum (7DADM) and water temperature model outputs are monthly means, an additional conversion step was performed to convert 7DADM values into monthly means, which involved first calculating daily mean and maximum values from historical stream gage data for multiple locations in the Sacramento, Feather, and American Rivers obtained from the California Data Exchange Center web site (cdec.ca.gov). The 7DADM was calculated for each day using the mean of that day and the preceding 6 days. Next, the difference between 7DADM and mean daily values was calculated for each day. Finally, for each location, the mean monthly difference between 7DADM and mean daily values was calculated. This difference was used as a conversion value to adjust water temperature index values. These conversion values are presented by month in Table 11B-5, Table 11B-6, and Table 11B-7 for the Sacramento, Feather, and American Rivers, respectively. No conversions were necessary for index values and index ranges that did not use USEPA 7DADM guidance.

The index value/range analysis consisted of three steps. First, for the NAA and each alternative, the total number of days (Sacramento and American Rivers) or months (Feather River) across the 82-year modeling period with a modeled temperature that exceeded a given index value or was outside a given index range in Table 11B-2, Table 11B-3, and Table 11B-4 was divided by the total number of days months for each month of the year and water year type to provide the frequency of exceedance above the index value or occurrence outside the index range. The difference in frequency of exceedance or occurrence outside the range between NAA and each alternative was then calculated for each month and water year type.

Second, for listed species (plus fall-/late fall-run Chinook salmon) only, the magnitude of exceedance above a temperature index value was calculated. For all days (Sacramento and American Rivers) or months (Feather River) that the modeled temperature exceeded a given temperature index value as shown in Table 11B-2, Table 11B-3, and Table 11B-4, the cumulative degrees exceeded were summed as a degree-day or a degree-month total by month and water year type across the 82-year modeling period and divided by the total number of days or months, respectively, that the index value was exceeded, to provide the average daily/monthly magnitude of exceedance for those days/months that exceeded the index temperature. The difference in average daily/monthly magnitude of exceedance between NAA and each alternative was then calculated for each month and water year type. Combined, these calculations provided a magnitude and frequency of exceedance above a given temperature index value.

The final step identified in which months and water year types there would be a biologically meaningful effect. This differed between listed and non-listed species. For listed species (plus fall-/late fall-run Chinook salmon), this step evaluated both frequency and magnitude combined. A biologically meaningful effect was defined as the months and water year types in which water temperature results met two criteria: (1) the difference in frequency of exceedance between NAA and an alternative was greater than 5%, and (2) the difference in average daily exceedance was greater than 0.5°F. The 5% criterion was based on best professional judgment of fisheries biologists from NMFS, CDFW, DWR, and Reclamation. The 0.5°F criterion was based on: (1) a review of the water temperature-related mortality rates for steelhead eggs and juveniles (D. Swank pers. comm.), and (2) a reasonable water temperature differential that could be resolved through real-time reservoir operations. The 0.5°F value was applied to all species/races and life stages although it was based on data for steelhead eggs and juveniles. For those months and water year types that met these two conditions, a thorough review was conducted to determine whether these patterns were persistent across multiple years and whether the differences could be alleviated during real time operations (i.e., the results are due to a model artifact when in reality, the system would not be operated in this way). Further, when results from a month and/or water year type met these two criteria, exceedance plots were reviewed to determine whether the results may be due to one or two outliers. If this was found to be the case, it was concluded that the effect was not persistent enough to be biologically relevant.

For non-listed species, the final step involved an evaluation of only the frequency of exceedance above a water temperature index value or occurrence outside a water temperature index range. A biologically meaningful effect was defined as the months and water year types in which the difference between NAA and an alternative in frequency of exceedance above a water temperature index value or occurrence outside a water temperature index range was greater than 5%. As with listed species, a thorough review was conducted to determine whether these patterns were persistent across multiple years and whether the differences could be alleviated during real time operations. Further, when results from a month and/or water year type met the criterion, exceedance plots were reviewed to determine whether the results may be due to one or two outliers. If this was found to be the case, it was concluded that the effect was not persistent enough to be biologically relevant.

Table 11B-2. Water Temperature Index Values and Index Ranges Used for Water Temperature Index Value/Range Analyses, Sacramento River

| Species | Life Stage | Period | Location | Index Value/Range (°F) |  |  | Sources/Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean Daily | 7DADM1 |  |  |
| Winter-run Chinook Salmon | Spawning, Egg Incubation, and Alevins | Apr-Oct | Keswick |  |  | 55.4 | USEPA 2003 |
|  |  |  | Clear Creek |  |  | 55.4 | USEPA 2003 |
|  |  |  | Balls Ferry |  |  | 55.4 | USEPA 2003 |
|  |  |  | Bend Bridge |  |  | 55.4 | USEPA 2003 |
|  |  |  | Red Bluff |  |  | 55.4 | USEPA 2003 |
|  | Fry and Juvenile Rearing and Emigration | Jul-Mar | Keswick |  |  | 61 | USEPA 2003; core juvenile rearing2 |
|  |  |  | Clear Creek |  |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | Balls Ferry |  |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | Bend Bridge |  |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | Red Bluff |  |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | Hamilton City |  |  | 64 | USEPA 2003; non-core juvenile rearing3 |
|  | Adult Immigration | Dec-Aug | Keswick |  |  | 68 | USEPA 2003 |
|  |  |  | Bend Bridge |  |  | 68 | USEPA 2003 |
|  |  |  | Red Bluff |  |  | 68 | USEPA 2003 |
|  | Adult Holding | Jan-Aug | Keswick |  |  | 61 | USEPA 2003 |
|  |  |  | Balls Ferry |  |  | 61 | USEPA 2003 |
|  |  |  | Red Bluff |  |  | 61 | USEPA 2003 |
| Spring-run Chinook Salmon | Spawning, Egg Incubation, and Alevins | Aug-Dec | Keswick |  |  | 55.4 | USEPA 2003 |
|  |  |  | Clear Creek |  |  | 55.4 | USEPA 2003 |
|  |  |  | Balls Ferry |  |  | 55.4 | USEPA 2003 |
|  |  |  | Bend Bridge |  |  | 55.4 | USEPA 2003 |
|  |  |  | Red Bluff |  |  | 55.4 | USEPA 2003 |
|  | Fry and Juvenile Rearing and Emigration | Year-round | Keswick |  |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | Clear Creek |  |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | Balls Ferry |  |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | Bend Bridge |  |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | Red Bluff |  |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | Hamilton City |  |  | 64 | USEPA 2003; non-core juvenile rearing |
|  | Adult Immigration | Mar-Sep | Keswick |  |  | 68 | USEPA 2003 |
|  |  |  | Bend Bridge |  |  | 68 | USEPA 2003 |
|  |  |  | Red Bluff |  |  | 68 | USEPA 2003 |
|  | Adult Holding | Apr-Sep | Keswick |  |  | 61 | USEPA 2003 |
|  |  |  | Balls Ferry |  |  | 61 | USEPA 2003 |
|  |  |  | Red Bluff |  |  | 61 | USEPA 2003 |
| Fall-run Chinook Salmon | Spawning, Egg Incubation, and Alevins | Sep-Jan | Keswick |  |  | 55.4 | USEPA 2003 |
|  |  |  | Clear Creek |  |  | 55.4 | USEPA 2003 |
|  |  |  | Balls Ferry |  |  | 55.4 | USEPA 2003 |
|  |  |  | Bend Bridge |  |  | 55.4 | USEPA 2003 |
|  |  |  | Red Bluff |  |  | 55.4 | USEPA 2003 |
|  | Fry and Juvenile Rearing and Emigration | Dec-Jun | Keswick |  |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | Clear Creek |  |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | Balls Ferry |  |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | Bend Bridge |  |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | Red Bluff |  |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | Hamilton City |  |  | 64 | USEPA 2003; non-core juvenile rearing |
|  | Adult Immigration | Jul-Dec | Keswick |  |  | 68 | USEPA 2003 |
|  |  |  | Bend Bridge |  |  | 68 | USEPA 2003 |
|  |  |  | Red Bluff |  |  | 68 | USEPA 2003 |
|  | Adult Holding | Jul-Aug | Keswick |  |  | 61 | USEPA 2003 |
|  |  |  | Balls Ferry |  |  | 61 | USEPA 2003 |
|  |  |  | Red Bluff |  |  | 61 | USEPA 2003 |
| Late Fall-run Chinook Salmon | Spawning, Egg Incubation, and Alevins | Dec-Jun | Keswick |  |  | 55.4 | USEPA 2003 |
|  |  |  | Clear Creek |  |  | 55.4 | USEPA 2003 |
|  |  |  | Balls Ferry |  |  | 55.4 | USEPA 2003 |
|  |  |  | Bend Bridge |  |  | 55.4 | USEPA 2003 |
|  |  |  | Red Bluff |  |  | 55.4 | USEPA 2003 |
|  | Fry and Juvenile Rearing and Emigration | Mar-Jan | Keswick |  |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | Clear Creek |  |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | Balls Ferry |  |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | Bend Bridge |  |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | Red Bluff |  |  | 64 | USEPA 2003; non-core juvenile rearing |
|  |  |  | Hamilton City |  |  | 64 | USEPA 2003; non-core juvenile rearing |
|  | Adult Immigration | Nov-Apr | Keswick |  |  | 68 | USEPA 2003 |
|  |  |  | Bend Bridge |  |  | 68 | USEPA 2003 |
|  |  |  | Red Bluff |  |  | 68 | USEPA 2003 |
| Steelhead | Spawning, Egg Incubation, and Alevins | Nov-Apr | Keswick | 53 |  |  | McCullough et al. 2001 |
|  |  |  |  | 56 |  |  | NMFS 2009 |
|  |  |  | Clear Creek | 53 |  |  | McCullough et al. 2001 |
|  |  |  |  | 56 |  |  | NMFS 2009 |
|  |  |  | Balls Ferry | 53 |  |  | McCullough et al. 2001 |
|  |  |  |  | 56 |  |  | NMFS 2009 |
|  |  |  | Bend Bridge | 53 |  |  | McCullough et al. 2001 |
|  |  |  |  | 56 |  |  | NMFS 2009 |
|  |  |  | Red Bluff | 53 |  |  | McCullough et al. 2001 |
|  |  |  |  | 56 |  |  | NMFS 2009 |
|  | Kelt Emigration | Feb-May | Keswick |  |  | 68 | USEPA 2003 |
|  |  |  |  | 70 |  |  | Average of studies cited in Richter and Kolmes 2005 (for upper end of suboptimal range) |
|  |  |  | Bend Bridge |  |  | 68 | USEPA 2003 |
|  |  |  |  | 70 |  |  | Average of studies cited in Richter and Kolmes 2005 (for upper end of suboptimal range) |
|  |  |  | Red Bluff |  |  | 68 | USEPA 2003 |
|  |  |  |  | 70 |  |  | Average of studies cited in Richter and Kolmes 2005 (for upper end of suboptimal range) |
|  | Juvenile Rearing | Year-round | Keswick | 63 |  |  | Intermediate value of ranges of optimal growth from Grabowski 1973; Hokanson et al. 1977; Wurtsbaugh and Davis 1977; Myrick and Cech 2005; and Beakes et al. 2014 |
|  |  |  |  |  |  | 69 | Sullivan et al. 2000 |
|  |  |  | Clear Creek | 63 |  |  | Intermediate value of ranges of optimal growth from Grabowski 1973; Hokanson et al. 1977; Wurtsbaugh and Davis 1977; Myrick and Cech 2005; and Beakes et al. 2014 |
|  |  |  |  |  |  | 69 | Sullivan et al. 2000 |
|  |  |  | Balls Ferry | 63 |  |  | Intermediate value of ranges of optimal growth from Grabowski 1973; Hokanson et al. 1977; Wurtsbaugh and Davis 1977; Myrick and Cech 2005; and Beakes et al. 2014 |
|  |  |  |  |  |  | 69 | Sullivan et al. 2000 |
|  |  |  | Bend Bridge | 63 |  |  | Intermediate value of ranges of optimal growth from Grabowski 1973; Hokanson et al. 1977; Wurtsbaugh and Davis 1977; Myrick and Cech 2005; and Beakes et al. 2014 |
|  |  |  |  |  |  | 69 | Sullivan et al. 2000 |
|  |  |  | Red Bluff | 63 |  |  | Intermediate value of ranges of optimal growth from Grabowski 1973; Hokanson et al. 1977; Wurtsbaugh and Davis 1977; Myrick and Cech 2005; and Beakes et al. 2014 |
|  |  |  |  |  |  | 69 | Sullivan et al. 2000 |
|  | Smoltification | Jan-Mar | Keswick | 54 |  |  | Zaugg and Wagner 1973; Adams et al. 1975; Zaugg 1981; Hoar 1988 |
|  |  |  | Clear Creek | 54 |  |  | Zaugg and Wagner 1973; Adams et al. 1975; Zaugg 1981; Hoar 1988 |
|  |  |  | Balls Ferry | 54 |  |  | Zaugg and Wagner 1973; Adams et al. 1975; Zaugg 1981; Hoar 1988 |
|  |  |  | Bend Bridge | 54 |  |  | Zaugg and Wagner 1973; Adams et al. 1975; Zaugg 1981; Hoar 1988 |
|  |  |  | Red Bluff | 54 |  |  | Zaugg and Wagner 1973; Adams et al. 1975; Zaugg 1981; Hoar 1988 |
|  | Smolt Emigration (excludes migrant parr) | Nov-Jun | Keswick |  |  | 61 | USEPA 2003 |
|  |  |  |  |  |  | 64 | USEPA 2003 |
|  |  |  | Clear Creek |  |  | 61 | USEPA 2003 |
|  |  |  |  |  |  | 64 | USEPA 2003 |
|  |  |  | Balls Ferry |  |  | 61 | USEPA 2003 |
|  |  |  |  |  |  | 64 | USEPA 2003 |
|  |  |  | Bend Bridge |  |  | 61 | USEPA 2003 |
|  |  |  |  |  |  | 64 | USEPA 2003 |
|  |  |  | Red Bluff |  |  | 61 | USEPA 2003 |
|  |  |  |  |  |  | 64 | USEPA 2003 |
|  | Adult Immigration | Aug-Mar | Keswick |  |  | 68 | USEPA 2003 |
|  |  |  |  | 70 |  |  | Average of studies cited in Richter and Kolmes 2005 (for upper end of suboptimal range) |
|  |  |  | Bend Bridge |  |  | 68 | USEPA 2003 |
|  |  |  |  | 70 |  |  | Average of studies cited in Richter and Kolmes 2005 (for upper end of suboptimal range) |
|  |  |  | Red Bluff |  |  | 68 | USEPA 2003 |
|  |  |  |  | 70 |  |  | Average of studies cited in Richter and Kolmes 2005 (for upper end of suboptimal range) |
|  | Adult Holding | Sep-Nov | Keswick |  |  | 61 | USEPA 2003 |
|  |  |  | Balls Ferry |  |  | 61 | USEPA 2003 |
|  |  |  | Red Bluff |  |  | 61 | USEPA 2003 |
| Green Sturgeon | Spawning and Embryo Incubation | Mar-Jul | Bend Bridge | 63 |  |  | Upper end of optimal range for embryonic development (Van Eenennaam 2005) |
|  |  |  | Red Bluff | 63 |  |  |  |
|  |  |  | Hamilton City | 63 |  |  |  |
|  | Non-Spawning Adult Presence (Immigration, Pre- and Post-Spawn Holding) | Aug-Feb | Bend Bridge | 66 |  |  | Assumes that adults are at least as tolerant to temperatures as larvae and juveniles |
|  |  |  |  | 73 |  |  | Houston 1988; Erickson et al. 2002 |
|  |  |  | Red Bluff | 66 |  |  | Assumes that adults are at least as tolerant to temperatures as larvae and juveniles |
|  |  |  |  | 73 |  |  | Houston 1988; Erickson et al. 2002 |
|  |  |  | Hamilton City | 66 |  |  | Assumes that adults are at least as tolerant to temperatures as larvae and juveniles |
|  |  |  |  | 73 |  |  | Houston 1988; Erickson et al. 2002 |
|  |  | Year-round | Knights Landing | 66 |  |  | Assumes that adults are at least as tolerant to temperatures as larvae and juveniles |
|  |  |  |  | 73 |  |  | Houston 1988; Erickson et al. 2002 |
|  | Larval to Juvenile Rearing and Emigration | Year-round | Bend Bridge | 66 |  |  | Upper end of optimal range for bioenergetics performance of Age 0/1 sturgeon with full or reduced food supply (Mayfield and Cech 2004) |
|  |  |  | Red Bluff | 66 |  |  |  |
|  |  |  | Hamilton City | 66 |  |  |  |
| White Sturgeon | Spawning and Embryo Incubation | Feb-May | Hamilton City | 61 |  |  | Optimal egg incubation range upper limit (Israel et al. 2011) |
|  |  |  |  | 68 |  |  | Embryo hatching upper limit (Israel et al. 2011) |
|  | Juvenile Rearing and Emigration | Year-round | Hamilton City | 66 |  |  | Stress observed in juvenile white sturgeon above 66°F (Israel et al. 2011) |
|  | Adult Immigration and Holding | Nov-May | Hamilton City | 77 |  |  | Upper limit of suitable water temperatures for adults (Israel et al. 2011) |
| Pacific Lamprey | Spawning and Egg Incubation | Apr-Aug | Keswick | 50-64 |  |  | High survival and low occurrence of embryonic developmental abnormalities observed in this range (Meeuwig et al. 2002, 2005) |
|  |  |  | Red Bluff | 50-64 |  |  |  |
|  | Ammocoete Rearing and Emigration | Year-round | Keswick | 72 |  |  | Significant decrease in survival and increase in developmental abnormalities observed above 72°F (Meeuwig et al. 2002, 2005) |
|  |  |  | Red Bluff | 72 |  |  |  |
| River Lamprey | Spawning and Egg Incubation | Feb-Jul | Keswick | 50-64 |  |  | High survival and low occurrence of embryonic developmental abnormalities observed in this range (Meeuwig et al. 2002, 2005) |
|  |  |  | Red Bluff | 50-64 |  |  |  |
|  | Ammocoete Rearing and Emigration | Year-round | Keswick | 72 |  |  | Significant decrease in survival and increase in developmental abnormalities observed above 72°F (Meeuwig et al. 2002, 2005) |
|  |  |  | Red Bluff | 72 |  |  |  |
| Hardhead | Spawning | Apr-Jun | Keswick | 59-64 |  |  | Optimal range (Wang 1986) |
|  |  |  | Red Bluff | 59-64 |  |  |  |
|  | Non-spawning Life Stages | Year-round | Keswick | 65-82 |  |  | Widest observed range (Cech et al. 1990, Moyle 2002, Southern California Edison Company 2007) |
|  |  |  | Red Bluff | 65-82 |  |  |  |
| Sacramento Hitch | Spawning | Mar-Jul | Red Bluff | 57-79 |  |  | Moyle 2002 |
|  |  |  | Butte City | 57-79 |  |  | Moyle 2002 |
| Sacramento Splittail | Spawning | Feb-May | Hamilton City | 45-75 |  |  | Observed range of suitable water temperatures (Moyle et al. 2004) |
| Striped Bass | Spawning, Embryo Incubation, and Initial Rearing | Apr-Jun | Butte City | 59-68 |  |  | Optimal range (Moyle 2002) |
|  | Larvae, Fry, and Juvenile Rearing and Emigration | Year-round | Butte City | 61-71 |  |  | Optimal range (Fay et al. 1983) |
| American Shad | Spawning, Embryo Incubation, and Initial Rearing | Apr-Jun | Red Bluff | 60-70 |  |  | Optimal range (Bell 1986, Painter et al. 1980, Leggett and Whitney 1972, Painter et al. 1979, Rich 1987) |
|  |  |  | Butte City | 60-70 |  |  |  |
|  | Larvae, Fry, and Juvenile Rearing and Emigration | Year-round | Red Bluff | 63-77 |  |  | Optimal range (Moyle 2002) |
|  |  |  | Butte City | 63-77 |  |  |  |
| Largemouth Bass | Spawning | Mar-Jun | Keswick | 54-75 |  |  | Acceptable range for spawning and incubation (Moyle 2002) |
|  |  |  | Red Bluff | 54-75 |  |  |  |

1 7DADM = Seven Day Average Daily Maximum

2 Core = “moderate to high density” (USEPA 2003)

3 Non-core = “low to moderate density” (USEPA 2003)

Table 11B-3. Water Temperature Index Values and Index Ranges Used for Water Temperature Index Value/Range Analyses, Feather River

| Species | Life Stage | Period | Location | Index Value/Range (°F) |  | Source/Note |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean Monthly | 7DADM1 |  |
| Winter-run Chinook Salmon | Non-Natal Rearing | Jul-Mar | LFC above Thermalito |  | 64 | USEPA 2003; non-core juvenile rearing2 |
|  |  |  | HFC at Gridley |  | 64 | USEPA 2003; non-core juvenile rearing |
| Spring-run Chinook Salmon | Spawning, Egg Incubation, and Alevins | Sep-Feb | LFC below Fish Dam |  | 55.4 | USEPA 2003 |
|  |  |  | HFC below Thermalito |  | 55.4 | USEPA 2003 |
|  | Fry and Juvenile Rearing and Emigration | Nov-Jun | LFC below Fish Dam |  | 61 | USEPA 2003; core juvenile rearing3 |
|  |  |  | HFC below Thermalito |  | 64 | USEPA 2003; non-core juvenile rearing |
|  | Adult Immigration | Mar-Jun | LFC below Fish Dam |  | 68 | USEPA 2003 |
|  |  |  | HFC below Thermalito |  | 68 | USEPA 2003 |
|  | Adult Holding | Apr-Sep | LFC below Fish Dam |  | 61 | USEPA 2003 |
|  |  |  | HFC below Thermalito |  | 61 | USEPA 2003 |
| Fall-run Chinook Salmon | Spawning, Egg Incubation, and Alevins | Oct-Feb | LFC below Fish Dam |  | 55.4 | USEPA 2003 |
|  |  |  | HFC below Thermalito |  | 55.4 | USEPA 2003 |
|  | Fry and Juvenile Rearing and Emigration | Nov-May | LFC below Fish Dam |  | 61 | USEPA 2003; core juvenile rearing |
|  |  |  | HFC below Thermalito |  | 64 | USEPA 2003; non-core juvenile rearing |
|  | Adult Immigration | Aug-Dec | LFC below Fish Dam |  | 68 | USEPA 2003 |
|  |  |  | HFC below Thermalito |  | 68 | USEPA 2003 |
|  | Adult Holding | Aug-Dec | LFC below Fish Dam |  | 61 | USEPA 2003 |
|  |  |  | HFC below Thermalito |  | 61 | USEPA 2003 |
| Steelhead | Spawning, Egg Incubation, and Alevins | Dec-May | LFC below Fish Dam | 53 |  | McCullough et al. 2001 |
|  |  |  | HFC below Thermalito | 53 |  | McCullough et al. 2001 |
|  | Kelt Emigration | Feb-May | LFC below Fish Dam |  | 68 | USEPA 2003 |
|  |  |  |  | 70 |  | Average of studies cited in Richter and Kolmes 2005 (for upper end of suboptimal range) |
|  |  |  | HFC below Thermalito |  | 68 | USEPA 2003 |
|  |  |  |  | 70 |  | Average of studies cited in Richter and Kolmes 2005 (for upper end of suboptimal range) |
|  | Juvenile Rearing | Year-round | LFC below Fish Dam | 63 |  | Intermediate value of ranges of optimal growth from Grabowski 1973; Hokanson et al. 1977; Wurtsbaugh and Davis 1977; Myrick and Cech 2005; and Beakes et al. 2014 |
|  |  |  |  |  | 69 | USEPA 2003 |
|  |  |  | HFC below Thermalito | 63 |  | Intermediate value of ranges of optimal growth from Grabowski 1973; Hokanson et al. 1977; Wurtsbaugh and Davis 1977; Myrick and Cech 2005; and Beakes et al. 2014 |
|  |  |  |  |  | 69 | USEPA 2003 |
|  | Smoltification | Jan-Mar | LFC below Fish Dam | 54 |  | Zaugg and Wagner 1973; Adams et al. 1975; Zaugg 1981; Hoar 1988 |
|  |  |  | HFC below Thermalito | 54 |  | Zaugg and Wagner 1973; Adams et al. 1975; Zaugg 1981; Hoar 1988 |
|  | Smolt Emigration | Dec-Jun | LFC below Fish Dam |  | 61 | USEPA 2003 |
|  |  |  | HFC below Thermalito |  | 64 | USEPA 2003 |
|  | Adult Immigration | Aug-Mar | LFC below Fish Dam |  | 68 | USEPA 2003 |
|  |  |  |  | 70 |  | Average of studies cited in Richter and Kolmes 2005 (for upper end of suboptimal range) |
|  |  |  | HFC below Thermalito |  | 68 | USEPA 2003 |
|  |  |  |  | 70 |  | Average of studies cited in Richter and Kolmes 2005 (for upper end of suboptimal range) |
|  | Adult Holding | Sep-Nov | LFC below Fish Dam |  | 61 | USEPA 2003 |
|  |  |  | HFC below Thermalito |  | 61 | USEPA 2003 |
| Green Sturgeon | Spawning and Embryo Incubation | Mar-Jul | LFC below Fish Dam | 63 |  | Upper end of optimal range for embryonic development (Van Eenennaam 2005) |
|  |  |  | HFC below Thermalito | 63 |  |  |
|  |  |  | HFC at Gridley | 63 |  |  |
|  | Non-Spawning Adult Presence (Immigration, Pre- and Post-Spawn Holding) | Aug-Nov | LFC below Fish Dam | 66 |  | Assumes that adults are at least as tolerant to temperatures as larvae and juveniles |
|  |  |  |  | 73 |  | Houston 1988; Erickson et al. 2002 |
|  |  |  | HFC below Thermalito | 66 |  | Assumes that adults are at least as tolerant to temperatures as larvae and juveniles |
|  |  |  |  | 73 |  | Houston 1988; Erickson et al. 2002 |
|  |  |  | HFC at Gridley | 66 |  | Assumes that adults are at least as tolerant to temperatures as larvae and juveniles |
|  |  |  |  | 73 |  | Houston 1988; Erickson et al. 2002 |
|  | Larval to Juvenile Rearing and Emigration | Year-round | LFC below Fish Dam | 66 |  | Upper end of optimal range for bioenergetics performance of Age 0/1 sturgeon with full or reduced food supply (Mayfield and Cech 2004) |
|  |  |  | HFC below Thermalito | 66 |  |  |
|  |  |  | HFC at Gridley | 66 |  |  |
| White Sturgeon | Spawning and Embryo Incubation | Feb-May | LFC below Fish Dam | 61 |  | Optimal egg incubation range upper limit (Israel et al. 2011) |
|  |  |  | HFC below Thermalito | 61 |  |  |
|  |  |  | HFC at Mouth | 61 |  |  |
|  | Juvenile Rearing and Emigration | Year-round | LFC below Fish Dam | 66 |  | Stress observed in juvenile white sturgeon above 66°F (Israel et al. 2011) |
|  |  |  | HFC below Thermalito | 66 |  |  |
|  |  |  | HFC at Mouth | 66 |  |  |
|  | Adult Immigration and Holding | Nov-May | LFC below Fish Dam | 77 |  | Upper limit of suitable water temperatures for adult (Israel et al. 2011) |
|  |  |  | HFC below Thermalito | 77 |  |  |
|  |  |  | HFC at Mouth | 77 |  |  |
| Pacific Lamprey | Spawning and Egg Incubation | Apr-Aug | LFC below Fish Dam | 50-64 |  | High survival and low occurrence of embryonic developmental abnormalities observed in this range (Meeuwig et al. 2002, 2005) |
|  |  |  | HFC below Thermalito | 50-64 |  |  |
|  |  |  | HFC at Mouth | 50-64 |  |  |
|  | Ammocoete Rearing and Emigration | Year-round | LFC below Fish Dam | 72 |  | Significant decrease in survival and increase in developmental abnormalities observed above 72°F (Meeuwig et al. 2002, 2005) |
|  |  |  | HFC below Thermalito | 72 |  |  |
|  |  |  | HFC at Mouth | 72 |  |  |
| River Lamprey | Spawning and Egg Incubation | Feb-Jul | LFC below Fish Dam | 50-64 |  | High survival and low occurrence of embryonic developmental abnormalities observed in this range (Meeuwig et al. 2002, 2005) |
|  |  |  | HFC below Thermalito | 50-64 |  |  |
|  |  |  | HFC at Mouth | 50-64 |  |  |
|  | Ammocoete Rearing and Emigration | Year-round | LFC below Fish Dam | 72 |  | Significant decrease in survival and increase in developmental abnormalities observed above 72°F (Meeuwig et al. 2002, 2005) |
|  |  |  | HFC below Thermalito | 72 |  |  |
|  |  |  | HFC at Mouth | 72 |  |  |
| Hardhead | Spawning | Apr-Jun | LFC below Fish Dam | 59-64 |  | Optimal range (Wang 1986) |
|  |  |  | HFC below Thermalito | 59-64 |  |  |
|  |  |  | HFC at Mouth | 59-64 |  |  |
|  | Non-Spawning Life Stages | Year-round | LFC below Fish Dam | 65-82 |  | Widest observed range (Cech et al. 1990, Moyle 2002, Southern California Edison Company 2007) |
|  |  |  | HFC below Thermalito | 65-82 |  |  |
|  |  |  | HFC at Mouth | 65-82 |  |  |
| Sacramento Hitch | Spawning | Mar-Jul | LFC below Fish Dam | 57-79 |  | Moyle 2002 |
|  |  |  | HFC below Thermalito | 57-79 |  |  |
| Sacramento Splittail | Spawning | Feb-May | HFC at Mouth | 45-75 |  | Observed range of suitable water temperatures (Moyle et al. 2004) |
| Striped Bass | Spawning, Embryo Incubation, and Initial Rearing | Apr-Jun | HFC below Thermalito | 59-68 |  | Optimal range (Moyle 2002) |
|  |  |  | HFC at Mouth | 59-68 |  |  |
|  | Larvae, Fry, and Juvenile Rearing and Emigration | Year-round | HFC below Thermalito | 61-71 |  | Optimal range (Fay et al. 1983) |
|  |  |  | HFC at Mouth | 61-71 |  |  |
| American Shad | Spawning, Embryo Incubation, and Initial Rearing | Apr-Jun | HFC below Thermalito | 60-70 |  | Optimal range (Bell 1986, Painter et al. 1980, Leggett and Whitney 1972, Painter et al. 1979, Rich 1987) |
|  |  |  | HFC at Mouth | 60-70 |  |  |
|  | Larvae, Fry, and Juvenile Rearing and Emigration | Jul - Nov | HFC below Thermalito | 63-77 |  | Optimal range (Moyle 2002) |
|  |  |  | HFC at Mouth | 63-77 |  |  |
| Largemouth Bass | Spawning | Mar-Jun | HFC below Thermalito | 54-75 |  | Acceptable range for spawning and incubation (Moyle 2002) |
|  |  |  | HFC at Mouth | 54-75 |  |  |

1 7DADM = Seven Day Average Daily Maximum

2 Core = “moderate to high density” (USEPA 2003)

3 Non-core = “low to moderate density” (USEPA 2003)

Table 11B-4. Water Temperature Index Values and Index Ranges Used for Water Temperature Index Value/Range Analyses, American River

| Species | Life Stage | Period | Location | Index Value/Range (°F) |  | Source/Note |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean Daily | 7DADM1 |  |
| Winter-run Chinook Salmon | Non-Natal Rearing | Jul-Apr | Watt Ave |  | 64 | USEPA 2003; non-core location2 |
| Fall-run Chinook Salmon | Spawning, Egg Incubation, and Alevins | Oct-Feb | Below Nimbus |  | 55.4 | USEPA 2003 |
|  |  |  | Watt Ave |  | 55.4 | USEPA 2003 |
|  | Fry and Juvenile Rearing and Emigration | Jan-May | Below Nimbus |  | 61 | USEPA 2003; core juvenile rearing3 |
|  |  |  | Watt Ave |  | 64 | USEPA 2003; non-core juvenile rearing |
|  | Adult Immigration | Sep-Dec | Below Nimbus |  | 68 | USEPA 2003 |
|  |  |  | Watt Ave |  | 68 | USEPA 2003 |
|  | Adult Staging | Jul-Dec | Below Nimbus |  | 61 | USEPA 2003 |
|  |  |  | Watt Ave |  | 61 | USEPA 2003 |
| Steelhead | Spawning, Egg Incubation, and Alevins | Dec-May | Below Nimbus | 53 |  | McCullough et al. 2001 |
|  |  |  | Watt Ave | 53 |  | McCullough et al. 2001 |
|  | Kelt Emigration | Feb-May | Below Nimbus |  | 68 | USEPA 2003 |
|  |  |  |  | 70 |  | Average of studies cited in Richter and Kolmes 2005 (for upper end of suboptimal range) |
|  |  |  | Watt Ave |  | 68 | USEPA 2003 |
|  |  |  |  | 70 |  | Average of studies cited in Richter and Kolmes 2005 (for upper end of suboptimal range) |
|  | Juvenile Rearing | Year-round | Below Nimbus | 63 |  | Intermediate value of ranges of optimal growth from Grabowski 1973; Hokanson et al. 1977; Wurtsbaugh and Davis 1977; Myrick and Cech 2005; and Beakes et al. 2014 |
|  |  |  |  |  | 69 | Sullivan et al. 2000 |
|  |  |  | Watt Ave | 63 |  | Intermediate value of ranges of optimal growth from Grabowski 1973; Hokanson et al. 1977; Wurtsbaugh and Davis 1977; Myrick and Cech 2005; and Beakes et al. 2014 |
|  |  |  |  |  | 69 | Sullivan et al. 2000 |
|  | Smoltification | Jan-Mar | Below Nimbus | 54 |  | Zaugg and Wagner 1973; Adams et al. 1975; Zaugg 1981; Hoar 1988 |
|  |  |  | Watt Ave | 54 |  | Zaugg and Wagner 1973; Adams et al. 1975; Zaugg 1981; Hoar 1988 |
|  | Smolt Emigration | Dec-Jun | Below Nimbus |  | 61 | USEPA 2003; core location |
|  |  |  | Watt Ave |  | 64 | USEPA 2003; non-core location |
|  | Adult Immigration | Oct-Apr | Below Nimbus |  | 68 | USEPA 2003 |
|  |  |  |  | 70 |  | Average of studies cited in Richter and Kolmes 2005 (for upper end of suboptimal range) |
|  |  |  | Watt Ave |  | 68 | USEPA 2003 |
|  |  |  |  | 70 |  | Average of studies cited in Richter and Kolmes 2005 (for upper end of suboptimal range) |
|  | Adult Holding | Oct-Nov | Below Nimbus |  | 61 | USEPA 2003 |
|  |  |  | Watt Ave |  | 61 | USEPA 2003 |
| Pacific Lamprey | Spawning and Egg Incubation | Mar-Jul | Below Nimbus | 50-64 |  | High survival and low occurrence of embryonic developmental abnormalities observed in this range (Meeuwig et al. 2002, 2005) |
|  |  |  | Watt Ave | 50-64 |  |  |
|  |  |  | Mouth | 50-64 |  |  |
|  | Ammocoete Rearing and Emigration | Year-round | Below Nimbus | 72 |  | Significant decrease in survival and increase in developmental abnormalities observed above 72°F (Meeuwig et al. 2002, 2005) |
|  |  |  | Watt Ave | 72 |  |  |
|  |  |  | Mouth | 72 |  |  |
| River Lamprey | Spawning and Egg Incubation | Feb-Jul | Below Nimbus | 50-64 |  | High survival and low occurrence of embryonic developmental abnormalities observed in this range (Meeuwig et al. 2002, 2005) |
|  |  |  | Watt Ave | 50-64 |  |  |
|  |  |  | Mouth | 50-64 |  |  |
|  | Ammocoete Rearing and Emigration | Year-round | Below Nimbus | 72 |  | Significant decrease in survival and increase in developmental abnormalities observed above 72°F (Meeuwig et al. 2002, 2005) |
|  |  |  | Watt Ave | 72 |  |  |
|  |  |  | Mouth | 72 |  |  |
| Hardhead | Spawning | April - June | Below Nimbus | 59-64 |  | Optimal range (Wang 1986) |
|  | Non-Spawning Life Stages | Year-round | Watt Ave | 65-82 |  | Widest observed range (Cech et al. 1990, Moyle 2002, Southern California Edison Company 2007) |
| Sacramento Hitch | Spawning | Mar-July | Below Nimbus | 57-79 |  | Moyle 2002 |
|  |  |  | Watt Ave | 57-79 |  |  |
| Sacramento Splittail | Spawning | Feb-May | Mouth | 45-75 |  | Observed range of suitable water temperatures (Moyle et al. 2004) |
| Striped Bass | Spawning, Embryo Incubation, and Initial Rearing | Apr-Jun | Watt Ave | 59-68 |  | Optimal range (Moyle 2002) |
|  |  |  | Mouth | 59-68 |  |  |
|  | Larvae, Fry, and Juvenile Rearing and Emigration | Year-round | Watt Ave | 61-71 |  | Optimal range (Fay et al. 1983) |
|  |  |  | Mouth | 61-71 |  |  |
| American Shad | Spawning, Embryo Incubation, and Initial Rearing | Apr-Jun | Watt Ave | 60-70 |  | Optimal range (Bell 1986, Painter et al. 1980, Leggett and Whitney 1972, Painter et al. 1979, Rich 1987) |
|  |  |  | Mouth | 60-70 |  |  |
|  | Larvae, Fry, and Juvenile Rearing and Emigration | Jul-Nov | Watt Ave | 63-77 |  | Optimal range (Moyle 2002) |
|  |  |  | Mouth | 63-77 |  |  |
| Largemouth Bass | Spawning | Mar-Jun | Watt Ave | 54-75 |  | Acceptable range for spawning and incubation (Moyle 2002) |

1 7DADM = Seven Day Average Daily Maximum

2 Non-core = “low to moderate density” (USEPA 2003)

3 Core = “moderate to high density” (USEPA 2003)

Table 11B-5. Conversion Factors (°F) for USEPA (2003) Seven-Day Average Daily Maximum (7DADM) Water Temperature Index Values to Monthly Mean, Sacramento River1.

| Month | Keswick | Clear Creek | Balls Ferry | Bend Bridge | Red Bluff | Wilkins Slough2 |
| --- | --- | --- | --- | --- | --- | --- |
| January | -0.36 | -1.01 | -0.75 | -0.67 | -0.86 | 0.0 |
| February | -0.28 | -1.11 | -0.86 | -0.62 | -0.97 | -0.3 |
| March | -0.17 | -1.29 | -0.94 | -0.66 | -1.23 | -0.3 |
| April | -0.25 | -1.66 | -1.47 | -0.95 | -1.55 | -0.6 |
| May | -0.36 | -1.73 | -2.18 | -1.59 | -1.47 | -1.4 |
| June | -0.32 | -1.55 | -2.25 | -1.87 | -0.96 | -1.2 |
| July | -0.36 | -1.41 | -2.18 | -2.01 | -0.90 | -1.3 |
| August | -0.43 | -1.74 | -2.06 | -1.61 | -0.94 | -1.3 |
| September | -0.30 | -2.00 | -1.76 | -1.16 | -1.70 | -2.0 |
| October | -0.25 | -1.73 | -1.25 | -0.91 | -1.83 | -1.4 |
| November | -0.38 | -1.37 | -1.10 | -0.99 | -1.53 | -1.3 |
| December | -0.82 | -1.42 | -1.30 | -1.24 | -1.48 | -1.0 |

1	Based on historical data from 2003-2014 for all sites except Wilkins Slough, which is based on historical data from November 2012 through June 2015. For a given location and month, values in this table were added to 7DADM index values in Table 11B-2 such that actual values used in the evaluation for each month were lower than those listed in Table 11B-2.

2	Because there is no flow gage at Hamilton City, Wilkins Slough data were used to calculate the conversion factor for Hamilton City

Table 11B-6. Conversion Factors (°F) for USEPA (2003) Seven-Day Average Daily Maximum (7DADM) Water Temperature Index Values to Monthly Mean, Feather River1,2.

| Month | RM 66.3 (Downstream of Hatchery) | RM 58.7 (Downstream of Afterbay Outlet) | RM 25.5 (Shanghai Bend) |
| --- | --- | --- | --- |
| January | -0.76 | -0.52 | -0.45 |
| February | -0.83 | -0.56 | -0.58 |
| March | -0.93 | -0.60 | -0.60 |
| April | -0.88 | -0.78 | -1.06 |
| May | -1.06 | -0.87 | -1.34 |
| June | -1.10 | -1.37 | -1.74 |
| July | -1.82 | -1.41 | -1.30 |
| August | -2.08 | -1.37 | -1.04 |
| September | -2.16 | -1.58 | -1.48 |
| October | -1.36 | -1.20 | -1.51 |
| November | -0.92 | -1.15 | -1.45 |
| December | -0.94 | -0.78 | -0.96 |

1 	Based on historical data from 2002-2014. For a given location and month, values in this table were added to 7DADM index values in Table 11B-3 such that actual values used in the evaluation were lower than those listed in Table 11B-3.

2 	RM 66.3 conversion factors were used for both locations in the LFC (below Fish Dam and above Thermalito); RM58.7 conversion factors were used for the HFC below Thermalito Afterbay Outlet; RM 25.5 conversion factors were used for the HFC at Gridley Bridge.

Table 11B-7. Conversion Factors (°F) for USEPA (2003) Seven-Day Average Daily Maximum (7DADM) Maximum Water Temperature Index Values to Monthly Mean, American River1.

| Month | Below Nimbus Dam | Watt Ave |
| --- | --- | --- |
| January | -0.44 | -1.01 |
| February | -0.15 | -1.05 |
| March | -0.25 | -1.29 |
| April | -0.40 | -1.72 |
| May | -0.60 | -2.05 |
| June | -0.44 | -2.55 |
| July | -0.50 | -3.17 |
| August | -0.70 | -3.11 |
| September | -0.59 | -2.52 |
| October | -0.60 | -2.01 |
| November | -0.80 | -1.65 |
| December | -0.77 | -1.26 |

1	Based on historical data from 2003-2014. For a given location and month, values in this table were added to 7DADM index values in Table 11B-4 such that actual values used in the evaluation were lower than those listed in Table 11B-4.

#### Winter-Run Chinook Salmon Egg Mortality Analysis based on Martin et al. (2017)

##### Background

The dissolved oxygen content of the water passing through the gravel substrate and sustaining Winter-Run Chinook Salmon eggs is positively correlated with temperature; warm, anoxic conditions result in egg mortality. This analysis attempted to isolate the thermal component of egg mortality from other components such as density-dependent mortality and redd dewatering. Both the Martin et al. (2017) model described in this section and the Anderson (2018) model (described below in Section 11B.1.2.4, Winter-Run Chinook Salmon Egg Mortality Analysis based on Anderson (2018)) begin by modeling a redd's lifetime by counting the days required to cross a known cumulative degree-days threshold, and both estimate mortality as a linear, increasing function of temperature past a known temperature threshold, but each model uses a different set of assumptions to implement this conceptual model. The methods were applied to a set of simulated redds and the results were summarized on a seasonal level for comparison of mortality outcomes between DCR 2015 Without and With Project scenario HEC5Q model runs.

Martin et al. (2017) identified a discrepancy between laboratory and field estimates of egg mortality and proposed a mechanism based on differing flow velocities in the laboratory and field environments. They then outlined a model for estimating temperature-dependent egg mortality in the field and fit its parameters to Sacramento River Winter-Run Chinook Salmon population data collected between 1996 and 2015 (Martin et al., 2017).

##### Mortality Calculations

The first step in the Martin et al. (2017) model is to estimate a redd's date of emergence. Individual eggs within the redd hatch but stay within the gravel substrate of the redd and become alevins. These alevins later depart the redd in the emergence stage. The redd's estimated date of emergence is intended to represent the point in the average egg's life span where it leaves the gravel substrate of the redd.

The Martin et al. (2017) model estimates the date of emergence using a linear relationship between water temperature (T, in °F) and maturation: Rate of maturation = 0.00058 * T - 0.018 (Zeug et al. 2012). For each simulated redd, the Zeug et al. (2012) equation was applied to daily temperatures starting the day after redd creation until the cumulative sum of daily maturation rates is greater than one. The day on which this occurs is considered the date of emergence for the redd.

Daily survival is then calculated for every day of the redd's lifespan. Below a temperature threshold of 11.9°C, no temperature-dependent mortality is recorded, and the survival is 1. For each degree C above the threshold, 0.024 is subtracted from the daily survival. The product of the natural exponents of daily survivals is the total survival, and one minus survival is the estimated mortality fraction for that simulated redd.

In summary, the Martin et al. (2017) model uses the Zeug et al. (2012) equation to estimate date of emergence, then estimates daily mortality for each day of the redd's lifespan using a linear relationship.

##### Spatiotemporal Distribution of Simulated Winter-Run Chinook Salmon Eggs

The Martin et al. (2017) model was applied to HEC5Q Sacramento River temperature results using the same spatiotemporal distribution of redds in each year. The distribution is the averaged location and timing of redds counted in California Department of Fish and Wildlife Winter-Run aerial survey data from 2007 to 2014. Simulated redds were created and subjected to mortality calculations. All simulated redds’ mortalities were combined in a sum, weighted by the spatiotemporal distribution, to estimate the total seasonal mortality fraction.

No assumption was made regarding the total number of redds, as density-dependent mortality is not considered in this calculation; results indicate only the percentage of the total seasonal Winter-Run Chinook Salmon egg population in the upper Sacramento River that is estimated to have succumbed to temperature-dependent mortality. Because a large percentage of modeled redds survived into October and the HEC5Q simulation ends at September of 2003, temperature-dependent egg mortality was only estimated for the 1922–2002 water years.

Tables 11B-8 and 11B-9 indicate the river miles and dates for which simulated redds were created as well as the proportion of the total Winter-Run Chinook Salmon egg population which each location or time represents. The same temporal distribution was assumed for all locations.

Table 11B-8. Spatial Distribution of Simulated Redds Used in the Martin et al. (2017) Model of Winter-Run Chinook Salmon Egg Mortality

| River Reach | River Mile | Mean Percentage (2007-2014) |
| --- | --- | --- |
| Keswick to A.C.I.D. Dam. | 298 | 46.4% |
| A.C.I.D. Dam to Highway 44 Bridge | 296 | 46.1% |
| Highway 44 Br. to Airport Rd. Br. | 284 | 6.7% |
| Airport Rd. Br. to Balls Ferry Br. | 275 | 0.3% |
| Balls Ferry Br. to Battle Creek. | 271 | 0.2% |
| Battle Creek to Jellys Ferry Br. | 266 | 0.2% |
| Jellys Ferry Br. to Bend Bridge | 257 | 0.1% |
| Bend Bridge to Red Bluff Diversion Dam | 242 | 0.0% |

Table 11B-9. Temporal Distribution of Simulated Redds Used in the Martin et al. (2017) Model of Winter-Run Chinook Salmon Egg Mortality

| Date (month/day) | Mean Percentage (2007-2014) |
| --- | --- |
| 5/15 | 5.4% |
| 6/1 | 5.9% |
| 6/9 | 7.8% |
| 6/16 | 13.3% |
| 6/24 | 16.0% |
| 7/1 | 15.9% |
| 7/9 | 14.2% |
| 7/16 | 10.4% |
| 7/24 | 6.7% |
| 8/1 | 3.1% |
| 8/16 | 1.4% |

#### Winter-Run Chinook Salmon Egg Mortality Analysis based on Anderson (2018)

Anderson (2018) developed a model that built on Martin et al.'s (2017) findings but differed in two key assumptions. While Martin et al. (2017) applied mortality to each day of a redd's lifespan from birth past hatching to emergence, Anderson (2018) used a short critical period instead. Using field data from 2002 through 2015, a critical period just before hatching was found to provide the best fit (Anderson 2018). This analysis used a critical period of five days in length, following the implementation of the Anderson (2018) model on the SacPAS website (http://www.cbr.washington.edu/sacramento/fishmodel/).

Instead of using the Zeug et al. (2012) equation to estimate date of emergence, the Anderson (2018) model uses a different equation to estimate date of hatching. Like the Zeug et al. (2012) equation, daily temperatures are correlated to daily maturation and a cumulative sum of daily maturation is calculated until maturation crosses a known threshold. The date on which this occurs is the hatching date, and in this implementation of the Anderson (2018) model the five days before hatching are the days on which mortality is estimated.

The daily equation was calibrated as by Alderdice and Velsen (1978): ln(Daily development rate) = ln(k) + b * ( ln(T - c) ), where k = 0.08646, b = 1.23473, c = -2.26721, and temperature is measured in °C. The day on which the cumulative sum of daily development rate passes 100 is considered the redd hatching date.

Like the Martin et al. (2017) model, the Anderson (2018) model assumes a linear relationship between mortality and temperature, with zero mortality below a threshold. The threshold was set identical to the Martin et al. (2017) model at 11.9 C, while the slope is not 0.024 but 0.5. This is unsurprising; calibration to substantially the same dataset will naturally result in a much higher slope, or a much larger mortality impact per degree C above the threshold, for a model that only applies mortality to five days instead of the full lifespan of the redd. The same formulae for adding up daily survivals and finding a total mortality estimate were used as for the Martin et al. (2017) model, as described above in Mortality Calculations. The same spatiotemporal redd weighting was applied as the Martin et al. (2017) model; see description above in Spatiotemporal Distribution of Simulated Winter-Run Chinook Salmon Eggs.

## Spawning Habitat Weighted Usable Area

### Introduction

Weighted usable area (WUA) analysis provides estimates of the amount of suitable spawning and rearing habitat of fishes available in rivers and streams at various levels of flow. WUA is computed as the surface area of physical habitat available weighted by its suitability. Habitat suitability is determined from field studies of the distributions of redds or rearing juveniles with respect to flow velocities, depths, and substrate or cover characteristics in the river. These data are used in model simulations (PHABSIM) that estimate the availability of suitable habitat in a portion of the river at a given flow. WUA curves showing suitable habitat availability versus flow are generated from the simulations.

For this RDEIR/SDEIS, spawning WUA was estimated for winter-run, spring-run, fall-run, and late fall–run Chinook salmon and California Central Valley steelhead in the Sacramento, Feather and American Rivers. Fry and juvenile rearing WUA were estimated only for the Sacramento River because no acceptable rearing WUA curves are available for the Feather and American Rivers. Spawning and rearing WUA was estimated for the No Action Alternative (NAA) and Alternatives 1A, 1B, 2, and 3 (hereinafter referred to as Alternatives 1–3) from CALSIM II flow data for each month of the 82-year period of record.

### Sacramento River

The WUA curves used for Chinook salmon and steelhead spawning habitat in the Sacramento River were obtained from two U.S. Fish and Wildlife Service reports (U.S. Fish and Wildlife Service 2003a, 2006). Modeling assumptions used to derive spawning WUA curves include that the suitability of physical habitat for salmon and steelhead spawning is largely a function of substrate particle size, water depth, and flow velocity. The race- or species-specific suitability of the habitat with respect to these variables is determined by cataloguing conditions at active redds and is used to develop habitat suitability criteria (HSC) for each race or species of fish. Hydraulic modeling is then used to estimate the amount of habitat available for different HSC levels at different river flows, and the results are used to develop spawning habitat WUA curves (Bovee et al. 1998). The WUA curves and tables are used to look up the amount of spawning WUA available at different flows during the spawning periods of the race or species.

U.S. Fish and Wildlife Service (2003a) provides WUA curves and tables for spawning winter-run, fall-run, and late fall–run Chinook salmon and steelhead for three segments of the Sacramento River encompassing the reach from Keswick Dam to Battle Creek (Figure 11B-1). The WUA tables were updated in 2006 (U.S. Fish and Wildlife Service 2006). No WUA curves were developed for spring-run Chinook salmon, but as discussed later, the fall-run curves were used to quantify spring-run spawning habitat. Also as further discussed below, the HSC used to develop the steelhead WUA curve for Sacramento River spawning were obtained from investigations of steelhead redds in the American River (U.S. Fish and Wildlife Service 2003b) because few steelhead redds were observed in the Sacramento River. Figure 11B-2 through Figure 11B-5 show the flow versus spawning WUA results for winter-run, fall-run, late fall–run and steelhead in the three river segments (Segment 6 = Keswick to Anderson-Cottonwood Irrigation District [ACID] Dam, Segment 5 = ACID Dam to Cow Creek, and Segment 4 = Cow Creek to Battle Creek) as provided by U.S. Fish and Wildlife Service (2006). Figure 11B-6 shows spawning WUA results for fall-run in an additional downstream segment (Segment 3 = Battle Creek to Red Bluff Diversion Dam) because spawning for fall run occurs further downstream than it does for the other salmon runs. Note that for Segment 6, separate WUA curves were developed for periods when the ACID Dam boards were installed (April through October) and for when the boards were out because installation of the boards affects water depths and velocities for some of the sampling transects used to develop the curves.

Because several tributaries enter the Sacramento River between Keswick Dam and Battle Creek, flows generally differ among the segments. For the U.S. Fish and Wildlife Service studies, Sacramento River flows were measured directly at the sampling transects and were estimated as the sum of Keswick Dam flow releases and tributary gauge readings upstream of the transects. To estimate WUA for the effects analysis, the segment flows were estimated using Sacramento River CALSIM II flows at Keswick Dam, the Clear Creek confluence, and Battle Creek. Keswick Dam flows were used for Segment 5 upstream of the Clear Creek confluence. For Segment 6, the WUA curves for the months when the ACID Dam boards are installed (April through October) were used with the Keswick Dam flows for those months and the WUA curves for the months when the ACID Dam boards are out were used with the flows for the rest of the year. To evaluate the relative importance of results from the three segments for each race or species in the effects analysis, the typical spawning distributions of the salmon with respect to the segments (Table 11B-10) was estimated from aerial redd surveys conducted by California Department of Fish and Wildlife (ICF 2016). All the races other than fall-run primarily spawn upstream of the Battle Creek confluence, and most fall-run spawning occurs upstream of the RBDD. Little is known about steelhead spawning locations in the Sacramento River, although it was assumed for this analysis that, because of constraints on water temperature and other habitat features, individuals spawn between Keswick Dam and RBDD, where nearly all Chinook salmon spawn (Table 11B-10).

Table 11B-10. Distributions of Spawning Redds among WUA River Segments as Percent of Total in the Sacramento River for Chinook Salmon Runs.

| Segment | Description | River Miles | Winter-run | Spring-run | Fall-run | Late fall–run |
| --- | --- | --- | --- | --- | --- | --- |
| 6 | Keswick to ACIDa | 302-298.5 | 45.0% | 12.4% | 16.3% | 67.6% |
| 5 | ACID to Cow Creek | 298.5-280 | 54.6% | 66.0% | 25.9% | 12.7% |
| 4 | Cow Creek to Battle Creek | 280-271 | 0.4% | 12.8% | 18.4% | 9.2% |
| 3 | Battle Creek to RBDD | 271-243 | 0.0% | 4.9% | 22.8% | 4.3% |
| — | Downstream of RBDD | — | 0.0% | 4.0% | 16.6% | 6.2% |

a ACID = Anderson-Cottonwood Irrigation District

Figure 11B-1. Segments 2–6 of the Sacramento River Used in U.S. Fish and Wildlife Service Studies to Determine Spawning and Rearing Weighted Usable Area (WUA) (flows in the figure are the average flows at the upstream boundary of each segment for October 1974 to September 1993). Source: U.S. Fish and Wildlife Service 2003a.

Figure 11B-2. Spawning WUA curves for Winter-Run Chinook Salmon in the Sacramento River, Segments 4 to 6. ACID = Anderson-Cottonwood Irrigation District.

Figure 11B-3. Spawning WUA Curves for Fall-Run Chinook Salmon in the Sacramento River, Segments 4 to 6. The fall-run curves were also used to quantify spring-run Chinook salmon WUA, as discussed in the text. ACID = Anderson-Cottonwood Irrigation District.

Figure 11B-4. Spawning WUA Curves for Late Fall–Run Chinook Salmon in the Sacramento River, Segments 4 to 6. ACID = Anderson-Cottonwood Irrigation District.

Figure 11B-5. Spawning WUA curves for Steelhead in the Sacramento River, Segments 4 to 6. ACID = Anderson-Cottonwood Irrigation District.

Figure 11B-6. Spawning WUA Curve for Fall–Run Chinook Salmon in the Sacramento River, Segment 3.

Because there are no spring-run Chinook salmon WUA curves in the U.S. Fish and Wildlife Service documentation, the previous practice (described below) has been to use fall-run Chinook salmon WUA curves to model spring-run habitat. Two models that currently produce spawning WUA outputs for spring-run Chinook salmon, SALMOD and SacEFT, derive the spring-run WUA results using the fall-run Chinook salmon spawning WUA curves as surrogates (Bartholow 2004; ESSA Technologies 2011). Mark Gard, who led the U.S. Fish and Wildlife Service studies that produced the Sacramento River WUA curves, has endorsed this practice (Gard pers. comm.). However, this practice introduces uncertainty to the spring-run Chinook salmon results. Although fall-run spawning WUA curves were used as surrogates for spring-run spawning, CALSIM II flows for the months of spring-run spawning, not those of fall-run spawning, were used to compute the spring-run WUA results.

A potential limitation of the Sacramento River spawning WUA curves for steelhead is the use of previously obtained American River steelhead HSC used in developing the curves (U.S. Fish and Wildlife Service 2003b). HSC data were not collected by U.S. Fish and Wildlife Service for steelhead in the Sacramento River because very few steelhead redds were observed and because the steelhead redds could not be distinguished from those of resident rainbow trout. The validity of this substitution could not be tested and is uncertain (U.S. Fish and Wildlife Service 2003a).

A further limitation of the WUA curves presented above, as of all such habitat-based studies, is that they assume the channel characteristics of the river during the time of field data collection by U.S. Fish and Wildlife Service (1995–1999), such as proportions of mesohabitat types, have remained in dynamic equilibrium to the present time and would continue to do so through the life of the Project. If the channel characteristics substantially change, the shape of the curve may no longer be applicable.

Differences in the mean spawning WUA under Alternatives 1–3 and NAA were examined for the months of the spawning period under each water year type and all water year types combined. Differences in mean spawning WUA of greater than 5% were flagged in the results tables to highlight results with the largest differences.

### Feather River

The relationships between instream flows and Chinook salmon and steelhead spawning habitat availability (WUA) in the Feather River developed by Payne and Allen (2004) were used in this effects analysis to determine effects of changing flows on Chinook salmon (spring-run and fall-run) and steelhead spawning WUA in the lower Feather River (Figure 11B-7 and Figure 11B-8). Although spring-run, fall-run and steelhead spawn in both the upper Feather River between the Fish Barrier Dam and Thermalito Afterbay Outlet and the lower Feather River downstream of the Thermalito Afterbay Outlet, Alternatives 1–3 would have no effect on flow in the upper river (Low Flow Channel), so differences in WUA between Alternatives 1–3 and the NAA were estimated only for the lower river (High Flow Channel).

Differences in the mean spawning WUA under Alternatives 1–3 and the NAA and were examined for the months of the spawning period for each species and race under each water year type and all water year types combined. Differences in mean spawning WUA of greater than 5% were flagged in the results tables to highlight the largest differences.

Figure 11B-7. Spawning WUA for Chinook Salmon (spring-run and fall-run) in the Feather River below Thermalito Afterbay Outlet.

Figure 11B-8. Spawning WUA for California Central Valley Steelhead in the Feather River below Thermalito Afterbay Outlet.

### American River

The WUA curves used for fall-run Chinook salmon and steelhead spawning habitat in the American River were obtained from Bratovich et al. (2107), which provides spawning WUA curves for fall-run and steelhead in eight reaches of the American River. The eight reaches lie within the approximately 10-mile river reach from Nimbus Dam downstream to Riverbend Side Channel, where most salmon and steelhead spawning occurs. Figure 11B-9 and Figure 11B-10 show composite WUA curves for flow versus spawning that combine the WUA results for the eight reaches. For this effects analysis, CALSIM II flows at Nimbus Dam were used to compute fall-run and steelhead spawning WUA from the composite WUA curves and tables.

Differences in the mean spawning WUA under Alternatives 1–3 and the NAA were examined for the months of the spawning period under each water year type and all water year types combined. Differences in mean spawning WUA of greater than 5% were flagged in the results tables to highlight results with the largest differences.

Figure 11B-9. Composite Spawning WUA for Fall-Run Chinook Salmon in the American River.

Figure 11B-10. Composite Spawning WUA for Steelhead in the American River.

## Redd Dewatering

Redd dewatering occurs when the water level drops below the depth of the redds or drops low enough to produce depth and flow velocity conditions that are inadequate to sustain incubating eggs or alevins in the redds. The redd dewatering analyses for the Sacramento, Feather, and American Rivers are based on the maximum reduction in flow from the initial flow, or spawning flow, that occurs over the duration of an egg cohort. The duration of a cohort in a redd includes egg incubation and alevin development to emergence from the gravel. Based on technical assistance from NMFS, cohort duration was estimated as 3 months for the four Chinook salmon races (fall-run, spring-run, winter-run, and late fall–run) and steelhead. The minimum flow of the egg cohort period is referred to herein as the dewatering flow. If flows during the 3 months subsequent to spawning are all greater than the spawning flow, no dewatering is assumed to occur. The analysis assumes that in Sacramento River, for which USRDOM daily time-step flow data are available, a new egg cohort begins each day of the spawning period. The spawning period is assumed to end 3 months prior to the end of the full spawning and incubation period. No daily time-step flow data were available for the Feather or American Rivers, so CALSIM II data, which has a monthly time-step, were used for redd dewatering analyses in these rivers. These analyses assume a new egg cohort begins each month of the spawning period. It should be noted that the use of monthly time-step flow estimates likely underestimates redd dewatering rates, but this potential bias is expected to affect all Project scenarios equally.

### Sacramento River

Flows from the spring-run spawning period and spawning distribution (Table 11B-11) were used to look up the percent of spring-run redds dewatered. The percentage of redds in the Sacramento River lost to dewatering was estimated using tables in U.S. Fish and Wildlife (USFWS) (2006) that relate spawning and dewatering flows to percent reductions in species-specific spawning habitat weighted usable area (WUA). See Table 11B-12 through Table 11B-19. USFWS (2006) developed the dewatering tables for winter-run, fall-run, late fall–run Chinook salmon and steelhead, but not for spring-run Chinook salmon. Therefore, as was done for the WUA curves, the fall-run salmon tables (Table 11B-14 and Table 11B-15) were used to estimate spring-run redd dewatering. The validity of substituting the fall-run tables for spring-run is discussed below in Section 11B.7.1, Sacramento River. Note that separate tables were developed for periods when the Anderson-Cottonwood Irrigation District (ACID) Dam boards are installed (April through October) and for when the boards are out because installation of the boards affects water levels for some of the sampling transects used to produce the tables.

The field studies used for USFWS (2006) were conducted in the Sacramento River between Keswick Dam and Battle Creek at the same locations as the spawning WUA studies discussed in Section 11B.2, Spawning Habitat Weighted Usable Area. USRDOM flow data are available for three locations in the Keswick Dam to Battle Creek river section: Keswick Dam (River Mile [RM] 302), the Sacramento River at Clear Creek (RM 289), and the Sacramento River at Battle Creek (RM 271). In contrast to the WUA studies, a single relationship for flows was developed for the entire river section, but the flows used to estimate redd dewatering in the current analysis were those that best matched the longitudinal distribution of the redds of the different salmon runs in the river as estimated from aerial redd surveys conducted by California Department of Fish and Wildlife from 2003 through 2019 (Table 11B-11). The redd distributions of steelhead in the Sacramento River is poorly known but is expected to be similar to that of spring-run. Therefore, Keswick Dam flows were used for winter-run and late fall–run, Sacramento River at Clear Creek flows were used for spring-run and steelhead, and Sacramento River at Battle Creek flows were used for fall-run. Redd dewatering was computed for these flows under Alternatives 1–3 and the No Action Alternative (NAA).

Table 11B-11. Average 2003–2019 Distributions of Spawning Redds of Chinook Salmon runs in the Sacramento River as Percent of Total, from Aerial Redd Surveys by California Department of Fish and Wildlife.

| Description | River Miles | Winter-run | Spring-run | Fall-run | Late fall–run |
| --- | --- | --- | --- | --- | --- |
| Keswick to ACIDa | 302–298 | 44.6% | 12.8% | 19.5% | 71.3% |
| ACID to Highway 44 | 298–296 | 38.8% | 33.9% | 6.6% | 5.2% |
| Highway 44 to Airport Rd. | 296–284 | 15.7% | 29.7% | 14.7% | 3.9% |
| Airport Rd. to Balls Ferry Br. | 284–275 | 0.6% | 11.1% | 19.4% | 8.9% |
| Balls Ferry Br. To Battle Creek | 275–271 | 0.2% | 7.4% | 12.5% | 5.9% |
| Battle Creek to Jellys Ferry Br. | 271–266 | 0.1% | 1.5% | 15.2% | 3.1% |
| Jellys Ferry Br. to Bend Bridge | 266–257 | 0.1% | 2.6% | 8.0% | 1.2% |
| Bend Bridge to RBDDb | 257–242 | 0.0% | 0.8% | 4.2% | 0.6% |

a ACID = Anderson-Cottonwood Irrigation District

b RBDD = Red Bluff Diversion Dam

Table 11B-12. Percent Redd Dewatered Look-up Table for Winter-Run Chinook Salmon with ACID Dam Boards Out (the percent of redds dewatered are looked up at the intersection of the “Spawning Flow” columns and “Dewatering Flow” rows)

|  | Spawning Flow |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dewatering Flow |  | 3,500 | 3,750 | 4,000 | 4,250 | 4,500 | 4,750 | 5,000 | 5,250 | 5,500 | 6,000 | 6,500 | 7,000 | 7,500 | 8,000 | 9,000 | 10,000 | 11,000 |
|  | 3,250 | 0.8 | 1.5 | 2.2 | 3 | 3.9 | 4.9 | 5.8 | 7 | 8.2 | 11 | 13.8 | 16.7 | 19.7 | 22.6 | 28.8 | 34.8 | 39.4 |
|  | 3,500 |  | 0.6 | 1 | 1.4 | 2 | 2.7 | 3.4 | 4.2 | 5.1 | 7.2 | 9.5 | 12.1 | 14.7 | 17.4 | 23.4 | 29.5 | 34.3 |
|  | 3,750 |  |  | 0.2 | 0.5 | 0.8 | 1.2 | 1.6 | 2.1 | 2.8 | 4.3 | 6.1 | 8.3 | 10.6 | 13.1 | 18.9 | 25.1 | 30 |
|  | 4,000 |  |  |  | 0.2 | 0.4 | 0.7 | 1 | 1.4 | 2 | 3.2 | 4.7 | 7.6 | 8.9 | 11.3 | 16.9 | 23.1 | 27.9 |
|  | 4,250 |  |  |  |  | 0.1 | 0.3 | 0.5 | 0.8 | 1.2 | 2.2 | 3.4 | 5.9 | 7 | 9.1 | 14.3 | 20.3 | 25 |
|  | 4,500 |  |  |  |  |  | 0.2 | 0.3 | 0.6 | 0.8 | 1.7 | 2.6 | 3.9 | 5.5 | 7.6 | 12.2 | 17.8 | 22.3 |
|  | 4,750 |  |  |  |  |  |  | 0.1 | 0.3 | 0.5 | 1.2 | 1.9 | 2.9 | 4.3 | 5.8 | 10.2 | 15.5 | 19.8 |
|  | 5,000 |  |  |  |  |  |  |  | 0.2 | 0.4 | 0.9 | 1.5 | 2.4 | 3.5 | 4.8 | 8.7 | 13.8 | 17.9 |
|  | 5,250 |  |  |  |  |  |  |  |  | 0.2 | 0.6 | 1.1 | 1.8 | 2.7 | 3.8 | 7 | 11.8 | 15.7 |
|  | 5,500 |  |  |  |  |  |  |  |  |  | 0.3 | 0.8 | 1.4 | 2.1 | 3 | 5.8 | 10.3 | 14.1 |
|  | 6,000 |  |  |  |  |  |  |  |  |  |  | 0.2 | 0.6 | 1.1 | 1.7 | 3.7 | 7.7 | 10.9 |
|  | 6,500 |  |  |  |  |  |  |  |  |  |  |  | 0.1 | 0.4 | 0.8 | 2.2 | 5.5 | 8.4 |
|  | 7,000 |  |  |  |  |  |  |  |  |  |  |  |  | 0.2 | 0.4 | 1.2 | 3.5 | 5.6 |
|  | 7,500 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.2 | 0.7 | 2.6 | 4.3 |
|  | 8,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.3 | 1.9 | 3.2 |
|  | 9,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.2 | 1.8 |
|  | 10,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.4 |
|  | 11,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 17,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 19,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 21,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 27,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 11B-12 (cont.)

|  | Spawning Flow |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dewatering Flow |  | 12,000 | 13,000 | 14,000 | 15,000 | 17,000 | 19,000 | 21,000 | 23,000 | 25,000 | 27,000 | 29,000 | 31,000 |
|  | 3,250 | 43.2 | 46.2 | 49.1 | 51.4 | 55 | 57.6 | 59.9 | 62.6 | 64.7 | 68.9 | 73.3 | 77.3 |
|  | 3,500 | 38.3 | 41.5 | 44.6 | 47.1 | 51 | 53.6 | 56.1 | 58.8 | 61.1 | 65.4 | 70.2 | 74.5 |
|  | 3,750 | 34.1 | 37.5 | 40.6 | 43.2 | 47.2 | 50 | 52.5 | 55.4 | 57.7 | 62.3 | 67.4 | 72 |
|  | 4,000 | 32.1 | 35.5 | 38.6 | 41.2 | 45.4 | 48.2 | 50.7 | 53.6 | 56.1 | 60.8 | 66.1 | 70.8 |
|  | 4,250 | 29.1 | 32.5 | 35.5 | 38.2 | 42.4 | 45.3 | 47.8 | 50.8 | 53.4 | 58.3 | 63.8 | 68.8 |
|  | 4,500 | 26.3 | 29.6 | 32.6 | 35.3 | 39.6 | 42.5 | 45.1 | 48.2 | 51 | 56 | 61.7 | 66.9 |
|  | 4,750 | 23.7 | 26.9 | 29.9 | 32.7 | 37 | 40 | 42.7 | 45.9 | 48.8 | 54 | 59.9 | 65.4 |
|  | 5,000 | 21.6 | 24.7 | 27.7 | 30.4 | 34.8 | 37.9 | 40.6 | 43.8 | 44.1 | 52.3 | 58.4 | 64.1 |
|  | 5,250 | 19.4 | 22.4 | 25.4 | 28.2 | 32.7 | 35.8 | 38.6 | 41.9 | 45.2 | 50.7 | 57 | 62.8 |
|  | 5,500 | 17.6 | 20.6 | 23.5 | 26.2 | 30.7 | 33.9 | 36.8 | 40.1 | 43.5 | 49 | 55.5 | 61.5 |
|  | 6,000 | 14 | 16.7 | 19.4 | 22 | 26.4 | 29.6 | 32.6 | 35.9 | 39.6 | 45.4 | 52.2 | 58.5 |
|  | 6,500 | 11.2 | 13.6 | 16.2 | 18.8 | 23.1 | 26.2 | 29.3 | 32.7 | 36.5 | 42.6 | 49.7 | 56.4 |
|  | 7,000 | 7.9 | 10.1 | 12.4 | 14.8 | 19 | 22.3 | 25.6 | 29.2 | 33.3 | 39.7 | 47.2 | 54.1 |
|  | 7,500 | 6.3 | 8.1 | 10.2 | 12.4 | 16.3 | 19.7 | 23 | 26.7 | 31 | 37.6 | 45.3 | 52.5 |
|  | 8,000 | 4.9 | 6.6 | 8.6 | 10.5 | 14.3 | 17.7 | 21.1 | 25 | 29.3 | 36.1 | 44.1 | 51.4 |
|  | 9,000 | 3 | 4.4 | 6 | 7.8 | 11.4 | 14.7 | 18.3 | 22.1 | 26.6 | 33.6 | 41.9 | 49.5 |
|  | 10,000 | 1.3 | 2.3 | 3.7 | 5.3 | 8.6 | 11.8 | 15.4 | 19.3 | 23.8 | 30.6 | 39.7 | 47.5 |
|  | 11,000 | 0.6 | 1.2 | 2.2 | 3.5 | 6.4 | 9.5 | 13.2 | 17.1 | 21.7 | 28.5 | 37.6 | 45.6 |
|  | 12,000 |  | 0.2 | 0.9 | 1.8 | 4.1 | 7 | 10.5 | 14.7 | 19.3 | 26.3 | 35.7 | 43.8 |
|  | 13,000 |  |  | 0.4 | 1 | 2.8 | 5.3 | 8.7 | 13 | 17.5 | 24.5 | 34 | 42.3 |
|  | 14,000 |  |  |  | 0.4 | 1.6 | 4.2 | 7.5 | 11.8 | 16.2 | 23 | 32.6 | 41 |
|  | 15,000 |  |  |  |  | 0.9 | 2.8 | 5.9 | 10.6 | 14.9 | 21.8 | 31.5 | 40.1 |
|  | 17,000 |  |  |  |  |  | 1.3 | 3.9 | 7.8 | 11.8 | 18.3 | 28.1 | 36.9 |
|  | 19,000 |  |  |  |  |  |  | 1.4 | 4 | 7.1 | 13 | 22.5 | 31.7 |
|  | 21,000 |  |  |  |  |  |  |  | 1.3 | 3.6 | 9.2 | 18.7 | 28 |
|  | 23,000 |  |  |  |  |  |  |  |  | 1.4 | 6.2 | 15.4 | 24.6 |
|  | 25,000 |  |  |  |  |  |  |  |  |  | 0 | 8.3 | 15.2 |
|  | 27,000 |  |  |  |  |  |  |  |  |  |  | 1.6 | 3.6 |
|  | 29,000 |  |  |  |  |  |  |  |  |  |  |  | 0.6 |

Table 11B-13. Percent Redd Dewatered Look-up Table for Winter-Run Chinook Salmon with ACID Dam Boards In (the percent of redds dewatered are looked up at the intersection of the “Spawning Flow” columns and “Dewatering Flow” rows)

|  | Spawning Flow |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dewatering Flow |  | 3,500 | 3,750 | 4,000 | 4,250 | 4,500 | 4,750 | 5,000 | 5,250 | 5,500 | 6,000 | 6,500 | 7,000 | 7,500 | 8,000 | 9,000 | 10,000 | 11,000 |
|  | 3,250 | 1.2 | 2.2 | 3.1 | 4.1 | 5.2 | 6.4 | 7.5 | 8.8 | 10.2 | 13 | 16 | 18.9 | 21.9 | 24.7 | 30.5 | 35.9 | 40.1 |
|  | 3,500 |  | 0.9 | 1.4 | 2 | 2.7 | 3.6 | 4.4 | 5.3 | 6.3 | 8.5 | 11 | 13.6 | 16.2 | 18.9 | 24.7 | 30.4 | 34.8 |
|  | 3,750 |  |  | 0.4 | 0.8 | 0.2 | 1.7 | 2.2 | 2.8 | 3.5 | 5.1 | 7 | 9.3 | 11.7 | 14.2 | 19.9 | 25.9 | 30.5 |
|  | 4,000 |  |  |  | 0.4 | 0.7 | 1.1 | 1.4 | 1.9 | 2.5 | 3.8 | 5.4 | 7.5 | 9.8 | 12.2 | 17.7 | 23.7 | 28.3 |
|  | 4,250 |  |  |  |  | 0.3 | 0.5 | 0.8 | 1.1 | 1.5 | 2.6 | 3.9 | 5.6 | 7.6 | 9.7 | 15 | 20.7 | 25.2 |
|  | 4,500 |  |  |  |  |  | 0.3 | 0.5 | 0.8 | 1.1 | 1.9 | 2.9 | 4.3 | 5.9 | 7.9 | 12.6 | 18.1 | 22.4 |
|  | 4,750 |  |  |  |  |  |  | 0.2 | 0.4 | 0.7 | 1.3 | 2.1 | 3.1 | 4.5 | 6.1 | 10.5 | 15.7 | 20 |
|  | 5,000 |  |  |  |  |  |  |  | 0.3 | 0.5 | 1 | 1.6 | 2.5 | 3.7 | 5 | 9 | 14 | 18.1 |
|  | 5,250 |  |  |  |  |  |  |  |  | 0.3 | 0.7 | 1.2 | 1.9 | 2.9 | 3.9 | 7.3 | 11.9 | 15.9 |
|  | 5,500 |  |  |  |  |  |  |  |  |  | 0.4 | 0.9 | 1.5 | 2.3 | 3.2 | 6.1 | 10.5 | 14.3 |
|  | 6,000 |  |  |  |  |  |  |  |  |  |  | 0.3 | 0.7 | 1.3 | 1.9 | 4 | 8 | 11.3 |
|  | 6,500 |  |  |  |  |  |  |  |  |  |  |  | 0.2 | 0.5 | 1 | 2.4 | 5.8 | 8.8 |
|  | 7,000 |  |  |  |  |  |  |  |  |  |  |  |  | 0.3 | 0.5 | 1.4 | 3.8 | 6.1 |
|  | 7,500 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.3 | 0.9 | 2.9 | 4.8 |
|  | 8,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.4 | 2.1 | 3.7 |
|  | 9,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.3 | 2.4 |
|  | 10,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.9 |
|  | 11,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 17,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 19,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 21,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 27,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 11B-13 (cont.)

|  | Spawning Flow |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dewatering Flow |  | 12,000 | 13,000 | 14,000 | 15,000 | 17,000 | 19,000 | 21,000 | 23,000 | 25,000 | 27,000 | 29,000 | 31,000 |
|  | 3,250 | 43.4 | 46 | 48.4 | 50.3 | 53.5 | 56 | 58.9 | 62.4 | 65.4 | 69.5 | 73.7 | 77.2 |
|  | 3,500 | 38.5 | 41.1 | 43.9 | 46.1 | 49.6 | 52.3 | 55.3 | 58.8 | 61.9 | 65.9 | 69.9 | 73.5 |
|  | 3,750 | 34.4 | 37.3 | 40 | 42.4 | 46.1 | 49 | 52.1 | 55.7 | 58.8 | 62.8 | 66.7 | 70.2 |
|  | 4,000 | 32.2 | 35.3 | 38 | 40.4 | 44.2 | 47.2 | 50.3 | 53.9 | 57 | 61.1 | 65 | 68.5 |
|  | 4,250 | 29.2 | 32.2 | 34.9 | 37.4 | 41.4 | 44.4 | 47.5 | 51.2 | 54.4 | 58.5 | 62.3 | 65.7 |
|  | 4,500 | 26.3 | 29.3 | 32 | 34.6 | 38.6 | 41.7 | 45 | 48.7 | 52 | 56 | 59.8 | 63.2 |
|  | 4,750 | 23.7 | 26.7 | 29.5 | 32.1 | 36.3 | 39.5 | 42.8 | 46.6 | 49.9 | 53.9 | 57.6 | 61.1 |
|  | 5,000 | 21.7 | 24.6 | 27.4 | 29.9 | 34.2 | 37.4 | 40.8 | 44.6 | 48 | 51.9 | 55.7 | 59.1 |
|  | 5,250 | 19.5 | 22.5 | 25.2 | 27.9 | 32.2 | 35.6 | 39 | 42.8 | 46.4 | 50.3 | 54.1 | 57.5 |
|  | 5,500 | 17.9 | 20.7 | 23.5 | 26.1 | 30.5 | 33.9 | 37.4 | 41.2 | 44.8 | 48.7 | 52.4 | 55.8 |
|  | 6,000 | 14.5 | 17.1 | 19.8 | 22.3 | 26.8 | 30.2 | 33.7 | 37.5 | 41.3 | 45.1 | 48.8 | 52.2 |
|  | 6,500 | 11.8 | 14.3 | 16.8 | 19.3 | 23.7 | 27.2 | 30.7 | 34.7 | 38.4 | 42.3 | 45.9 | 49.3 |
|  | 7,000 | 8.7 | 10.9 | 13.3 | 15.7 | 20.1 | 23.7 | 27.5 | 31.5 | 35.4 | 39.4 | 42.9 | 46.2 |
|  | 7,500 | 7 | 9 | 11.2 | 13.5 | 17.7 | 21.4 | 25.2 | 29.3 | 33.2 | 37.2 | 40.7 | 44 |
|  | 8,000 | 5.7 | 7.6 | 9.7 | 11.8 | 15.9 | 19.6 | 23.5 | 27.7 | 31.6 | 35.7 | 39.1 | 42.4 |
|  | 9,000 | 4 | 5.6 | 7.4 | 9.4 | 13.3 | 16.9 | 20.8 | 24.9 | 28.7 | 32.8 | 36.3 | 39.6 |
|  | 10,000 | 2.2 | 3.6 | 5.2 | 7 | 10.5 | 14 | 17.7 | 18.6 | 25.4 | 28.9 | 32.6 | 35.8 |
|  | 11,000 | 1.1 | 2 | 3.1 | 4.6 | 7.6 | 10.5 | 13.8 | 17.4 | 20.6 | 23.5 | 26.7 | 29.4 |
|  | 12,000 |  | 0.5 | 1.2 | 2.2 | 4.2 | 6.4 | 9.1 | 12.1 | 14.6 | 16.8 | 19.1 | 21.1 |
|  | 13,000 |  |  | 0.5 | 1.1 | 2.6 | 4.4 | 6.7 | 9.2 | 11.7 | 13.5 | 15.3 | 17 |
|  | 14,000 |  |  |  | 0.5 | 1.7 | 3.5 | 5.5 | 8.2 | 10.1 | 11.7 | 13.4 | 14.9 |
|  | 15,000 |  |  |  |  | 0.7 | 2.1 | 3.9 | 6.8 | 8.6 | 10.1 | 11.6 | 13 |
|  | 17,000 |  |  |  |  |  | 0.9 | 2.5 | 4.9 | 6.5 | 7.7 | 9.1 | 10.4 |
|  | 19,000 |  |  |  |  |  |  | 1 | 2.5 | 3.6 | 4.4 | 5.5 | 6.6 |
|  | 21,000 |  |  |  |  |  |  |  | 0.9 | 1.6 | 2.1 | 3 | 4 |
|  | 23,000 |  |  |  |  |  |  |  |  | 0.4 | 0.6 | 1.1 | 1.9 |
|  | 25,000 |  |  |  |  |  |  |  |  |  | 0.3 | 0.9 | 1.6 |
|  | 27,000 |  |  |  |  |  |  |  |  |  |  | 0.3 | 0.7 |
|  | 29,000 |  |  |  |  |  |  |  |  |  |  |  | 0.3 |

Table 11B-14. Percent Redd Dewatered Look-up Table for Fall-Run Chinook Salmon (Also Used for the Spring-Run Chinook Salmon Analysis) with ACID Dam Boards Out (the percent of redds dewatered are looked up at the intersection of the “Spawning Flow” columns and “Dewatering Flow” rows)

|  | Spawning Flow |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dewatering Flow |  | 3,500 | 3,750 | 4,000 | 4,250 | 4,500 | 4,750 | 5,000 | 5,250 | 5,500 | 6,000 | 6,500 | 7,000 | 7,500 | 8,000 | 9,000 | 10,000 | 11,000 |
|  | 3,250 | 1 | 2 | 3.4 | 4.8 | 6.6 | 8.4 | 10.6 | 12.9 | 15.3 | 20.6 | 26.2 | 31.7 | 37 | 41.5 | 50.2 | 56.3 | 60.4 |
|  | 3,500 |  | 1 | 2.1 | 3.2 | 4.6 | 6.2 | 8.1 | 10.1 | 12.2 | 17 | 22.2 | 27.4 | 29.2 | 37 | 45.9 | 52.8 | 57.3 |
|  | 3,750 |  |  | 0.9 | 1.6 | 2.6 | 3.9 | 5.5 | 7.3 | 9.2 | 13.6 | 18.4 | 23.1 | 28 | 32.4 | 41.5 | 48.7 | 53.6 |
|  | 4,000 |  |  |  | 0.9 | 1.7 | 2.8 | 4.1 | 5.7 | 7.3 | 11.4 | 15.8 | 20.3 | 24.8 | 29 | 38 | 45.7 | 50.7 |
|  | 4,250 |  |  |  |  | 0.8 | 1.6 | 2.7 | 4 | 5.4 | 8.9 | 13 | 17.2 | 21.6 | 25.8 | 34.9 | 42.8 | 48 |
|  | 4,500 |  |  |  |  |  | 0.8 | 1.7 | 2.8 | 4 | 6.9 | 10.4 | 14.2 | 18.2 | 22.1 | 30.9 | 38.8 | 44.2 |
|  | 4,750 |  |  |  |  |  |  | 0.8 | 1.6 | 2.5 | 4.8 | 7.6 | 10.8 | 14.2 | 17.6 | 25.8 | 33.2 | 38.8 |
|  | 5,000 |  |  |  |  |  |  |  | 0.7 | 1.3 | 3.2 | 5.6 | 8.6 | 11.6 | 14.7 | 22.6 | 30.2 | 36 |
|  | 5,250 |  |  |  |  |  |  |  |  | 0.7 | 2.1 | 4.2 | 6.8 | 9.4 | 12.3 | 19.8 | 27.2 | 33.1 |
|  | 5,500 |  |  |  |  |  |  |  |  |  | 1.4 | 3.2 | 5.4 | 7.7 | 10.3 | 17.6 | 24.9 | 31 |
|  | 6,000 |  |  |  |  |  |  |  |  |  |  | 1.2 | 2.8 | 4.6 | 6.4 | 12.9 | 19.7 | 25.8 |
|  | 6,500 |  |  |  |  |  |  |  |  |  |  |  | 1.3 | 2.6 | 4.2 | 9.8 | 15.6 | 21.1 |
|  | 7,000 |  |  |  |  |  |  |  |  |  |  |  |  | 0.9 | 2 | 6.6 | 11.8 | 17.3 |
|  | 7,500 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.8 | 4.4 | 9.1 | 14.1 |
|  | 8,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.6 | 6.6 | 11.5 |
|  | 9,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.2 | 5.5 |
|  | 10,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.9 |
|  | 11,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 17,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 19,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 21,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 27,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 11B-14 (cont.)

|  | Spawning Flow |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dewatering Flow |  | 12,000 | 13,000 | 14,000 | 15,000 | 17,000 | 19,000 | 21,000 | 23,000 | 25,000 | 27,000 | 29,000 | 31,000 |
|  | 3,250 | 62.9 | 63.7 | 65.3 | 66.4 | 66.8 | 65.7 | 67.8 | 71.3 | 74.5 | 80.4 | 87.3 | 92 |
|  | 3,500 | 60.1 | 61.1 | 63 | 64.2 | 64.9 | 63.8 | 66 | 69.5 | 73 | 79.1 | 86.2 | 91 |
|  | 3,750 | 56.9 | 58.3 | 60.3 | 61.8 | 62.7 | 61.7 | 64 | 67.7 | 71.4 | 77.7 | 84.9 | 89.6 |
|  | 4,000 | 54.3 | 55.9 | 58.2 | 59.9 | 61.2 | 60.2 | 62.7 | 66.5 | 70.4 | 77.1 | 84.1 | 88.8 |
|  | 4,250 | 51.8 | 53.6 | 56 | 58.1 | 59.6 | 58.8 | 61.3 | 65 | 68.5 | 75.7 | 83.1 | 87.8 |
|  | 4,500 | 48.3 | 50.2 | 52.8 | 55.1 | 57.1 | 56.4 | 59 | 62.7 | 66.2 | 73.3 | 81.8 | 86.5 |
|  | 4,750 | 43.3 | 45.6 | 48.6 | 51.4 | 54 | 53.7 | 56.6 | 60.4 | 64.5 | 71.7 | 80.3 | 85 |
|  | 5,000 | 40.6 | 43 | 46.1 | 49.1 | 52.2 | 52.2 | 55.2 | 59.1 | 63.3 | 70.6 | 79.4 | 84.1 |
|  | 5,250 | 37.7 | 40.2 | 43.5 | 46.5 | 50 | 50.2 | 53.5 | 57.4 | 60.7 | 68 | 78.2 | 83 |
|  | 5,500 | 35.8 | 38.4 | 41.7 | 44.8 | 48.3 | 48.8 | 52.3 | 56.1 | 60.1 | 67.5 | 77.3 | 82 |
|  | 6,000 | 30.9 | 33.8 | 37.3 | 40.6 | 45 | 45.8 | 49.5 | 53.2 | 57.2 | 65 | 75.4 | 80 |
|  | 6,500 | 26.5 | 29.2 | 32.7 | 36.1 | 41 | 42.4 | 46.5 | 50.4 | 54.8 | 63 | 73.3 | 77.7 |
|  | 7,000 | 22.8 | 25.8 | 29.3 | 32.9 | 38.3 | 40 | 44.4 | 48.3 | 52.9 | 61.3 | 71.8 | 76.1 |
|  | 7,500 | 20 | 23.2 | 26.9 | 30.7 | 36.4 | 38.2 | 42.8 | 46.8 | 51.9 | 60.5 | 70.9 | 75.3 |
|  | 8,000 | 17.2 | 20.9 | 24.9 | 28.9 | 34.9 | 36.6 | 41.3 | 45.4 | 50.5 | 59.3 | 70.2 | 74.7 |
|  | 9,000 | 10.6 | 14.4 | 18.4 | 22.5 | 29.2 | 31.9 | 37.4 | 41.8 | 47.7 | 57 | 68.2 | 72.6 |
|  | 10,000 | 4.5 | 7.7 | 12 | 16.4 | 23.5 | 26.9 | 33 | 38.5 | 44.5 | 54.1 | 65.9 | 70.5 |
|  | 11,000 | 2.7 | 5.3 | 9 | 13.6 | 21.4 | 24.8 | 30.2 | 35.3 | 41.8 | 51.6 | 63.7 | 68.4 |
|  | 12,000 |  | 1.6 | 4.7 | 9 | 16.8 | 20.6 | 27 | 32.9 | 39.8 | 50 | 62.3 | 67.2 |
|  | 13,000 |  |  | 1.6 | 4.8 | 12.2 | 16.9 | 24.4 | 31.3 | 38.1 | 48.4 | 60.8 | 65.9 |
|  | 14,000 |  |  |  | 2.6 | 9.5 | 14.8 | 22.1 | 28.9 | 36.2 | 46.8 | 59.5 | 64.7 |
|  | 15,000 |  |  |  |  | 5.3 | 11.1 | 18.5 | 26.2 | 33.5 | 44.6 | 57.6 | 63.1 |
|  | 17,000 |  |  |  |  |  | 4.1 | 11.3 | 18.5 | 26.1 | 37.8 | 51.5 | 57.9 |
|  | 19,000 |  |  |  |  |  |  | 4.6 | 10.8 | 18.8 | 30.4 | 44.2 | 51.1 |
|  | 21,000 |  |  |  |  |  |  |  | 4.2 | 11.7 | 23.9 | 38.4 | 46.3 |
|  | 23,000 |  |  |  |  |  |  |  |  | 6.7 | 17.8 | 31.2 | 38.9 |
|  | 25,000 |  |  |  |  |  |  |  |  |  | 2.3 | 6.4 | 10.7 |
|  | 27,000 |  |  |  |  |  |  |  |  |  |  | 1.8 | 5.3 |
|  | 29,000 |  |  |  |  |  |  |  |  |  |  |  | 2.2 |

Table 11B-15. Percent Redd Dewatered Look-up Table for Fall-Run Chinook Salmon (Also Used for the Spring-Run Chinook Salmon Analysis) with ACID Dam Boards In (the percent of redds dewatered are looked up at the intersection of the “Spawning Flow” columns and “Dewatering Flow” rows)

|  | Spawning Flow |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dewatering Flow |  | 3,500 | 3,750 | 4,000 | 4,250 | 4,500 | 4,750 | 5,000 | 5,250 | 5,500 | 6,000 | 6,500 | 7,000 | 7,500 | 8,000 | 9,000 | 10,000 | 11,000 |
|  | 3,250 | 1.0 | 2.0 | 3.3 | 4.7 | 6.2 | 7.8 | 9.7 | 11.7 | 13.6 | 17.8 | 22.2 | 26.3 | 30.2 | 33.4 | 39.5 | 43.5 | 46.0 |
|  | 3,500 |  | 1.0 | 2.0 | 3.1 | 4.4 | 5.7 | 7.4 | 9.2 | 10.9 | 14.8 | 18.8 | 22.8 | 23.9 | 29.8 | 36.2 | 40.8 | 43.6 |
|  | 3,750 |  |  | 0.9 | 1.6 | 2.5 | 3.6 | 5.1 | 6.7 | 8.3 | 11.9 | 15.6 | 19.3 | 23.0 | 26.2 | 32.8 | 37.7 | 40.9 |
|  | 4,000 |  |  |  | 0.9 | 1.7 | 2.6 | 3.8 | 5.3 | 6.6 | 10.0 | 13.5 | 16.9 | 20.4 | 23.5 | 30.1 | 35.4 | 38.7 |
|  | 4,250 |  |  |  |  | 0.8 | 1.5 | 2.5 | 3.7 | 5.0 | 7.8 | 11.1 | 14.4 | 17.8 | 20.9 | 27.5 | 33.1 | 36.6 |
|  | 4,500 |  |  |  |  |  | 0.8 | 1.6 | 2.6 | 3.7 | 6.0 | 8.9 | 11.9 | 15.0 | 17.8 | 24.4 | 29.9 | 33.6 |
|  | 4,750 |  |  |  |  |  |  | 0.8 | 1.6 | 2.4 | 4.3 | 6.6 | 9.1 | 11.8 | 14.3 | 20.3 | 25.7 | 29.5 |
|  | 5,000 |  |  |  |  |  |  |  | 0.7 | 1.3 | 2.9 | 4.9 | 7.2 | 9.6 | 11.9 | 17.7 | 23.1 | 26.9 |
|  | 5,250 |  |  |  |  |  |  |  |  | 0.6 | 1.9 | 3.5 | 5.6 | 7.7 | 9.7 | 15.3 | 20.4 | 24.1 |
|  | 5,500 |  |  |  |  |  |  |  |  |  | 1.2 | 2.7 | 4.4 | 6.2 | 8.1 | 13.5 | 18.5 | 22.3 |
|  | 6,000 |  |  |  |  |  |  |  |  |  |  | 1.0 | 2.3 | 3.7 | 5.1 | 9.8 | 14.5 | 18.3 |
|  | 6,500 |  |  |  |  |  |  |  |  |  |  |  | 1.1 | 2.1 | 3.3 | 7.4 | 11.5 | 15.0 |
|  | 7,000 |  |  |  |  |  |  |  |  |  |  |  |  | 0.7 | 1.6 | 5.0 | 8.6 | 12.1 |
|  | 7,500 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.6 | 3.4 | 6.7 | 9.9 |
|  | 8,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.0 | 4.9 | 8.1 |
|  | 9,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.6 | 3.8 |
|  | 10,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.2 |
|  | 11,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 17,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 19,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 21,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 27,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 11B-15 (cont.)

|  | Spawning Flow |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dewatering Flow |  | 12,000 | 13,000 | 14,000 | 15,000 | 17,000 | 19,000 | 21,000 | 23,000 | 25,000 | 27,000 | 29,000 | 31,000 |
|  | 3,250 | 47.6 | 48.0 | 49.3 | 50.5 | 52.0 | 52.5 | 55.1 | 57.6 | 57.4 | 59.0 | 61.1 | 63.3 |
|  | 3,500 | 45.5 | 46.0 | 47.4 | 48.8 | 50.4 | 50.8 | 53.4 | 55.9 | 55.7 | 57.2 | 59.3 | 61.6 |
|  | 3,750 | 43.1 | 43.9 | 45.5 | 47.0 | 48.7 | 49.1 | 51.8 | 54.3 | 54.1 | 55.6 | 57.6 | 59.8 |
|  | 4,000 | 41.2 | 42.2 | 43.8 | 45.5 | 47.5 | 47.9 | 50.5 | 53.1 | 52.9 | 54.5 | 56.3 | 58.5 |
|  | 4,250 | 39.2 | 4.0 | 42.1 | 43.9 | 46.0 | 46.4 | 49.0 | 51.3 | 50.8 | 52.5 | 54.4 | 56.5 |
|  | 4,500 | 36.4 | 37.6 | 39.4 | 41.4 | 43.6 | 43.9 | 46.4 | 48.7 | 47.8 | 49.1 | 51.6 | 53.7 |
|  | 4,750 | 32.6 | 34.0 | 36.1 | 38.3 | 40.8 | 41.1 | 43.6 | 45.7 | 44.9 | 46.0 | 48.3 | 50.3 |
|  | 5,000 | 30.0 | 31.2 | 33.2 | 35.3 | 37.6 | 37.6 | 39.8 | 41.7 | 40.5 | 41.3 | 43.2 | 45.1 |
|  | 5,250 | 27.1 | 28.2 | 29.9 | 31.8 | 33.9 | 33.5 | 35.4 | 36.8 | 34.6 | 35.0 | 37.4 | 39.0 |
|  | 5,500 | 25.3 | 26.4 | 28.0 | 29.7 | 31.5 | 31.0 | 32.7 | 33.8 | 31.7 | 31.9 | 33.6 | 35.1 |
|  | 6,000 | 21.5 | 22.7 | 24.4 | 26.2 | 28.2 | 27.5 | 29.0 | 29.8 | 27.1 | 27.1 | 28.7 | 29.8 |
|  | 6,500 | 18.3 | 19.5 | 21.1 | 23.0 | 25.2 | 24.7 | 26.4 | 27.1 | 24.4 | 24.2 | 25.3 | 26.3 |
|  | 7,000 | 15.6 | 17.0 | 18.7 | 20.7 | 23.2 | 22.8 | 24.5 | 25.1 | 22.4 | 22.1 | 23.2 | 24.0 |
|  | 7,500 | 13.7 | 15.3 | 17.1 | 19.3 | 21.9 | 21.5 | 23.3 | 23.9 | 21.3 | 21.0 | 21.9 | 22.7 |
|  | 8,000 | 11.8 | 13.7 | 15.7 | 17.9 | 20.7 | 20.2 | 21.9 | 22.4 | 19.8 | 19.4 | 20.5 | 21.4 |
|  | 9,000 | 7.2 | 9.2 | 11.3 | 13.6 | 16.8 | 16.8 | 18.9 | 19.6 | 17.2 | 16.8 | 17.9 | 18.5 |
|  | 10,000 | 3.0 | 4.9 | 7.2 | 9.8 | 13.3 | 13.8 | 16.2 | 17.4 | 14.9 | 14.5 | 15.9 | 16.7 |
|  | 11,000 | 1.9 | 3.4 | 5.4 | 8.2 | 12.1 | 12.2 | 14.5 | 15.6 | 13.3 | 12.8 | 14.1 | 15.0 |
|  | 12,000 |  | 1.0 | 2.8 | 5.4 | 9.4 | 10.0 | 12.5 | 14.0 | 11.9 | 11.5 | 12.9 | 13.9 |
|  | 13,000 |  |  | 1.0 | 3.0 | 6.9 | 8.1 | 11.1 | 13.1 | 11.0 | 10.7 | 12.1 | 13.1 |
|  | 14,000 |  |  |  | 1.8 | 5.4 | 7.0 | 9.8 | 11.8 | 10.0 | 9.9 | 11.4 | 12.4 |
|  | 15,000 |  |  |  |  | 2.8 | 4.8 | 7.7 | 10.2 | 8.6 | 8.7 | 10.4 | 11.5 |
|  | 17,000 |  |  |  |  |  | 1.8 | 5.0 | 7.5 | 6.5 | 6.8 | 8.5 | 10.0 |
|  | 19,000 |  |  |  |  |  |  | 2.3 | 4.8 | 4.6 | 5.0 | 6.9 | 8.4 |
|  | 21,000 |  |  |  |  |  |  |  | 1.9 | 2.0 | 2.6 | 4.7 | 6.6 |
|  | 23,000 |  |  |  |  |  |  |  |  | 0.7 | 1.6 | 3.6 | 5.7 |
|  | 25,000 |  |  |  |  |  |  |  |  |  | 1.2 | 3.0 | 5.0 |
|  | 27,000 |  |  |  |  |  |  |  |  |  |  | 1.2 | 3.3 |
|  | 29,000 |  |  |  |  |  |  |  |  |  |  |  | 1.5 |

Table 11B-16. Percent Redd Dewatered Look-up Table for Late Fall–Run Chinook Salmon with ACID Dam Boards Out (the percent of redds dewatered are looked up at the intersection of the “Spawning Flow” columns and “Dewatering Flow” rows)

|  | Spawning Flow |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dewatering Flow |  | 3,500 | 3,750 | 4,000 | 4,250 | 4,500 | 4,750 | 5,000 | 5,250 | 5,500 | 6,000 | 6,500 | 7,000 | 7,500 | 8,000 | 9,000 | 10,000 | 11,000 |
|  | 3,250 | 0.9 | 1.5 | 2.6 | 3.6 | 4.9 | 6.3 | 8 | 9.8 | 11.7 | 15.9 | 20.1 | 24.1 | 28 | 31.5 | 37.8 | 42.7 | 45.6 |
|  | 3,500 |  | 0.9 | 1.6 | 2.4 | 3.4 | 4.5 | 6 | 7.6 | 9.3 | 13.1 | 17.1 | 21 | 24.9 | 28.2 | 35 | 40.2 | 43.3 |
|  | 3,750 |  |  | 0.8 | 1.1 | 2 | 2.9 | 4.1 | 5.5 | 7 | 10.5 | 14.2 | 17.8 | 21.6 | 25 | 32 | 37.5 | 40.7 |
|  | 4,000 |  |  |  | 0.7 | 1.2 | 2 | 3 | 4.2 | 5.5 | 8.8 | 12.1 | 15.6 | 19.2 | 22.5 | 29.5 | 35.3 | 38.7 |
|  | 4,250 |  |  |  |  | 0.6 | 1.1 | 1.9 | 3 | 4.1 | 6.9 | 10 | 13.4 | 16.9 | 20.1 | 27.3 | 33.3 | 36.8 |
|  | 4,500 |  |  |  |  |  | 0.6 | 1.2 | 2.1 | 3.1 | 5.5 | 8.3 | 11.3 | 14.6 | 17.7 | 24.8 | 30.8 | 34.5 |
|  | 4,750 |  |  |  |  |  |  | 0.6 | 1.3 | 2 | 4 | 6.3 | 9 | 11.8 | 14.7 | 21.5 | 27.6 | 31.5 |
|  | 5,000 |  |  |  |  |  |  |  | 0.5 | 1 | 2.6 | 4.6 | 7 | 9.6 | 12.2 | 18.9 | 25.2 | 29.3 |
|  | 5,250 |  |  |  |  |  |  |  |  | 0.5 | 1.8 | 3.5 | 5.6 | 7.9 | 10.4 | 16.9 | 23.1 | 27.4 |
|  | 5,500 |  |  |  |  |  |  |  |  |  | 1.3 | 2.7 | 4.6 | 6.7 | 8.9 | 15.3 | 21.5 | 25.8 |
|  | 6,000 |  |  |  |  |  |  |  |  |  |  | 0.9 | 2.3 | 3.8 | 5.5 | 11.2 | 17.1 | 21.7 |
|  | 6,500 |  |  |  |  |  |  |  |  |  |  |  | 1 | 2.1 | 3.5 | 8.3 | 13.4 | 17.6 |
|  | 7,000 |  |  |  |  |  |  |  |  |  |  |  |  | 0.8 | 1.8 | 5.9 | 10.4 | 14.4 |
|  | 7,500 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.7 | 3.9 | 7.9 | 11.5 |
|  | 8,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.2 | 5.5 | 8.9 |
|  | 9,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.7 | 3.9 |
|  | 10,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.2 |
|  | 11,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 17,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 19,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 21,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 27,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 11B-16 (cont.)

|  | Spawning Flow |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dewatering Flow |  | 12,000 | 13,000 | 14,000 | 15,000 | 17,000 | 19,000 | 21,000 | 23,000 | 25,000 | 27,000 | 29,000 | 31,000 |
|  | 3,250 | 47.8 | 48.9 | 50.6 | 52.6 | 55.5 | 57.5 | 61.6 | 67.3 | 73.5 | 79.8 | 86.6 | 91.1 |
|  | 3,500 | 45.6 | 46.8 | 48.6 | 50.7 | 53.6 | 55.5 | 59.6 | 65.4 | 71.5 | 78.3 | 85.4 | 90.1 |
|  | 3,750 | 43.3 | 44.6 | 46.5 | 48.6 | 51.5 | 53.3 | 57.4 | 63.3 | 69.6 | 76.6 | 83.9 | 88.5 |
|  | 4,000 | 41.5 | 42.8 | 44.8 | 46.9 | 49.9 | 51.8 | 55.9 | 61.8 | 68.3 | 75.6 | 82.9 | 87.6 |
|  | 4,250 | 39.7 | 41.1 | 43.1 | 45.3 | 48.4 | 50.2 | 54.3 | 60.2 | 66.6 | 74.2 | 81.7 | 86.5 |
|  | 4,500 | 37.5 | 38.9 | 41 | 43.3 | 46.5 | 48.3 | 52.4 | 58.1 | 64.5 | 72.2 | 80.2 | 85 |
|  | 4,750 | 34.6 | 36.6 | 38.5 | 40.9 | 44.2 | 46 | 50.1 | 55.3 | 62.4 | 70.2 | 78.4 | 83.3 |
|  | 5,000 | 32.6 | 34.3 | 36.7 | 39.1 | 42.6 | 44.5 | 48.6 | 54.2 | 60.8 | 68.9 | 77.3 | 82.3 |
|  | 5,250 | 30.8 | 32.5 | 34.9 | 37.5 | 41.1 | 42.9 | 47 | 52.6 | 58.9 | 67 | 76 | 81.1 |
|  | 5,500 | 29.4 | 31.2 | 33.2 | 36.1 | 39.7 | 41.6 | 45.7 | 51.2 | 57.7 | 65.9 | 74.9 | 80 |
|  | 6,000 | 25.5 | 27.5 | 29.9 | 32.6 | 36.4 | 38.3 | 42.3 | 47.7 | 54.1 | 62.7 | 72.1 | 77.3 |
|  | 6,500 | 21.7 | 23.8 | 26.4 | 29.1 | 33.1 | 35.1 | 39.2 | 44.5 | 50.9 | 59.7 | 69.1 | 74 |
|  | 7,000 | 18.6 | 20.7 | 23.2 | 26.1 | 30.3 | 32.4 | 36.4 | 41.6 | 48 | 57 | 66.6 | 71.6 |
|  | 7,500 | 16 | 18.4 | 21.1 | 24 | 28.3 | 30.4 | 34.5 | 39.6 | 46.3 | 55.4 | 65.2 | 70.3 |
|  | 8,000 | 13.3 | 16 | 18.9 | 21.9 | 26.3 | 28.3 | 32.5 | 37.6 | 44.3 | 53.7 | 63.7 | 69 |
|  | 9,000 | 7.8 | 10.5 | 13.6 | 16.7 | 21.5 | 23.7 | 28.1 | 33.2 | 40.2 | 50 | 60.5 | 65.9 |
|  | 10,000 | 3.1 | 5.6 | 8.8 | 12.1 | 17 | 19.6 | 24 | 29.8 | 36.7 | 46.7 | 57.4 | 62.9 |
|  | 11,000 | 2.3 | 4.1 | 6.7 | 10 | 15.2 | 17.4 | 21.8 | 26.9 | 34 | 44.2 | 55.1 | 60.7 |
|  | 12,000 |  | 1.2 | 3.4 | 6.5 | 11.7 | 14.2 | 18.7 | 24.5 | 31.8 | 42.2 | 53.3 | 58.9 |
|  | 13,000 |  |  | 1.1 | 3.4 | 8.3 | 11.3 | 16.2 | 22.7 | 29.9 | 40.3 | 51.5 | 57.2 |
|  | 14,000 |  |  |  | 1.9 | 6.4 | 9.8 | 14.6 | 21.1 | 28.3 | 38.8 | 50.1 | 55.9 |
|  | 15,000 |  |  |  |  | 3.3 | 6.7 | 11.7 | 18.8 | 26 | 36.7 | 48.2 | 54.1 |
|  | 17,000 |  |  |  |  |  | 3.5 | 7 | 13.1 | 20.3 | 31.1 | 42.9 | 49.1 |
|  | 19,000 |  |  |  |  |  |  | 2.5 | 7.1 | 14.4 | 25.2 | 36.9 | 43.2 |
|  | 21,000 |  |  |  |  |  |  |  | 3.1 | 9.3 | 20 | 32.1 | 39.1 |
|  | 23,000 |  |  |  |  |  |  |  |  | 5.1 | 14.5 | 25.7 | 32.6 |
|  | 25,000 |  |  |  |  |  |  |  |  |  | 1.8 | 5.2 | 9.4 |
|  | 27,000 |  |  |  |  |  |  |  |  |  |  | 1.4 | 4.4 |
|  | 29,000 |  |  |  |  |  |  |  |  |  |  |  | 1.6 |

Table 11B-17. Percent Redd Dewatered Look-up Table for Late Fall–Run Chinook Salmon with ACID Dam Boards In (the percent of redds dewatered are looked up at the intersection of the “Spawning Flow” columns and “Dewatering Flow” rows)

|  | Spawning Flow |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dewatering Flow |  | 3,500 | 3,750 | 4,000 | 4,250 | 4,500 | 4,750 | 5,000 | 5,250 | 5,500 | 6,000 | 6,500 | 7,000 | 7,500 | 8,000 | 9,000 | 10,000 | 11,000 |
|  | 3,250 | 0.9 | 1.7 | 2.6 | 3.7 | 4.9 | 6.2 | 7.8 | 9.5 | 11.3 | 15.1 | 18.9 | 22.5 | 26 | 29.1 | 34.9 | 39.4 | 42.3 |
|  | 3,500 |  | 0.9 | 1.6 | 2.4 | 3.4 | 4.5 | 5.9 | 7.4 | 9 | 12.5 | 16.1 | 19.6 | 23.1 | 26.1 | 32.3 | 37.1 | 40.1 |
|  | 3,750 |  |  | 0.8 | 1.1 | 2 | 2.9 | 4.1 | 5.5 | 6.9 | 10.1 | 13.4 | 16.7 | 20.1 | 23.1 | 29.5 | 34.6 | 37.8 |
|  | 4,000 |  |  |  | 0.7 | 1.3 | 2 | 3 | 4.2 | 5.4 | 8.4 | 11.5 | 14.7 | 17.9 | 20.9 | 27.3 | 32.7 | 36 |
|  | 4,250 |  |  |  |  | 0.7 | 1.2 | 2 | 3 | 4.1 | 6.7 | 9.6 | 12.6 | 15.8 | 18.7 | 25.2 | 30.8 | 34.2 |
|  | 4,500 |  |  |  |  |  | 0.6 | 1.3 | 2.1 | 3.1 | 5.3 | 7.9 | 10.7 | 13.6 | 16.4 | 22.9 | 28.4 | 32 |
|  | 4,750 |  |  |  |  |  |  | 0.6 | 1.3 | 2.1 | 3.9 | 6 | 8.5 | 11.1 | 13.7 | 19.9 | 25.4 | 29.1 |
|  | 5,000 |  |  |  |  |  |  |  | 0.6 | 1.1 | 2.6 | 4.4 | 6.6 | 8.9 | 11.3 | 17.4 | 22.9 | 26.7 |
|  | 5,250 |  |  |  |  |  |  |  |  | 0.5 | 1.7 | 3.3 | 5.2 | 7.3 | 9.5 | 15.3 | 20.7 | 24.4 |
|  | 5,500 |  |  |  |  |  |  |  |  |  | 1.2 | 2.5 | 4.3 | 6.1 | 8.1 | 13.7 | 19.1 | 22.8 |
|  | 6,000 |  |  |  |  |  |  |  |  |  |  | 0.9 | 2.1 | 3.4 | 5 | 10 | 15.1 | 19 |
|  | 6,500 |  |  |  |  |  |  |  |  |  |  |  | 0.9 | 1.9 | 3.1 | 7.4 | 11.8 | 15.4 |
|  | 7,000 |  |  |  |  |  |  |  |  |  |  |  |  | 0.8 | 1.6 | 5.2 | 9.1 | 12.5 |
|  | 7,500 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.7 | 3.5 | 6.9 | 9.9 |
|  | 8,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 | 4.9 | 7.7 |
|  | 9,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.5 | 3.3 |
|  | 10,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
|  | 11,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 17,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 19,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 21,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 27,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 11B-17 (cont.)

|  | Spawning Flow |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dewatering Flow |  | 12,000 | 13,000 | 14,000 | 15,000 | 17,000 | 19,000 | 21,000 | 23,000 | 25,000 | 27,000 | 29,000 | 31,000 |
|  | 3,250 | 44.6 | 46 | 47.9 | 50.1 | 53.4 | 55.4 | 59.2 | 63.7 | 66.8 | 69.7 | 74.4 | 79.1 |
|  | 3,500 | 42.6 | 44 | 46 | 48.2 | 51.5 | 53.5 | 57.2 | 61.8 | 64.6 | 67.8 | 72.6 | 77.3 |
|  | 3,750 | 40.5 | 42 | 44 | 46.3 | 49.6 | 51.5 | 55.1 | 59.7 | 62.6 | 65.6 | 70.4 | 75.1 |
|  | 4,000 | 38.8 | 40.4 | 42.4 | 44.8 | 48.1 | 50 | 53.6 | 58.3 | 61.1 | 64.3 | 68.9 | 73.5 |
|  | 4,250 | 37.1 | 38.7 | 40.8 | 43.2 | 46.5 | 48.3 | 51.9 | 56.4 | 59 | 62.2 | 66.9 | 71.5 |
|  | 4,500 | 34.9 | 36.5 | 38.6 | 41.1 | 44.4 | 46.1 | 49.6 | 53.9 | 56.3 | 59.2 | 64.1 | 68.7 |
|  | 4,750 | 32.2 | 33.8 | 36 | 38.5 | 41.9 | 43.5 | 46.8 | 50.6 | 53.2 | 55.9 | 60.6 | 65.1 |
|  | 5,000 | 29.8 | 31.4 | 33.5 | 35.9 | 39.1 | 40.5 | 43.6 | 47.5 | 49.3 | 51.9 | 56.3 | 60.6 |
|  | 5,250 | 27.5 | 28.9 | 30.9 | 33.2 | 36.3 | 37.3 | 40.2 | 43.6 | 44.8 | 46.9 | 51.4 | 55.5 |
|  | 5,500 | 25.9 | 27.3 | 28.9 | 31.4 | 34.2 | 35.1 | 37.8 | 41 | 42.1 | 43.9 | 48 | 51.9 |
|  | 6,000 | 22.2 | 23.7 | 25.6 | 27.7 | 30.6 | 31.3 | 33.7 | 36.4 | 37 | 38.6 | 42.4 | 45.9 |
|  | 6,500 | 18.8 | 20.3 | 22.3 | 24.5 | 27.4 | 28.1 | 30.5 | 33 | 33.3 | 34.5 | 37.8 | 40.8 |
|  | 7,000 | 16 | 17.6 | 19.6 | 21.8 | 24.9 | 25.5 | 27.8 | 30.2 | 30.2 | 31.1 | 34.3 | 37.1 |
|  | 7,500 | 13.7 | 15.5 | 17.6 | 20 | 23.1 | 23.8 | 26 | 28.3 | 28.4 | 29.2 | 32.2 | 35.2 |
|  | 8,000 | 11.4 | 13.5 | 15.7 | 18.1 | 21.3 | 21.8 | 24.1 | 26.3 | 26.2 | 27 | 30.1 | 33.1 |
|  | 9,000 | 6.6 | 8.7 | 11.1 | 13.6 | 17 | 17.7 | 20.1 | 22.2 | 22.1 | 22.8 | 25.8 | 28.7 |
|  | 10,000 | 2.7 | 4.6 | 7 | 9.8 | 13.3 | 14.3 | 16.7 | 19.3 | 19 | 19.4 | 22.3 | 25.1 |
|  | 11,000 | 2 | 3.4 | 5.4 | 8.1 | 12 | 12.6 | 16.6 | 17 | 16.7 | 17 | 19.9 | 22.6 |
|  | 12,000 |  | 0.9 | 2.7 | 5.3 | 9.1 | 10 | 12.3 | 15 | 14.7 | 14.9 | 17.7 | 20.5 |
|  | 13,000 |  |  | 0.9 | 2.8 | 6.5 | 7.8 | 10.4 | 13.7 | 13.3 | 13.6 | 16.3 | 19 |
|  | 14,000 |  |  |  | 1.7 | 5.1 | 6.7 | 9.2 | 12.4 | 12.1 | 12.4 | 15 | 17.7 |
|  | 15,000 |  |  |  |  | 2.5 | 4.2 | 6.9 | 10.6 | 10.3 | 10.8 | 13.3 | 16 |
|  | 17,000 |  |  |  |  |  | 2.4 | 4.3 | 7.5 | 7.7 | 8.2 | 10.6 | 13.2 |
|  | 19,000 |  |  |  |  |  |  | 1.7 | 4.2 | 5.1 | 5.8 | 8.1 | 10.5 |
|  | 21,000 |  |  |  |  |  |  |  | 2 | 2.7 | 3.5 | 5.8 | 8.4 |
|  | 23,000 |  |  |  |  |  |  |  |  | 1.1 | 2.1 | 4.3 | 7.4 |
|  | 25,000 |  |  |  |  |  |  |  |  |  | 1.3 | 3.4 | 6.4 |
|  | 27,000 |  |  |  |  |  |  |  |  |  |  | 1.3 | 4 |
|  | 29,000 |  |  |  |  |  |  |  |  |  |  |  | 1.5 |

Table 11B-18. Percent Redd Dewatered Look-up Table for CCV Steelhead with ACID Dam Boards Out (the percent of redds dewatered are looked up at the intersection of the “Spawning Flow” columns and “Dewatering Flow” rows)

|  | Spawning Flow |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dewatering Flow |  | 3,500 | 3,750 | 4,000 | 4,250 | 4,500 | 4,750 | 5,000 | 5,250 | 5,500 | 6,000 | 6,500 | 7,000 | 7,500 | 8,000 | 9,000 | 10,000 | 11,000 |
|  | 3,250 | 1.2 | 2.6 | 3.7 | 4.9 | 6.8 | 8.9 | 10.9 | 13.3 | 15.7 | 19.9 | 23.4 | 26.2 | 28.5 | 31.1 | 37.2 | 43.5 | 49.8 |
|  | 3,500 |  | 1.6 | 2.4 | 3.2 | 4.7 | 6.4 | 8 | 10.2 | 12.4 | 16.5 | 19.9 | 22.8 | 25.1 | 27.7 | 33.8 | 40.1 | 46.2 |
|  | 3,750 |  |  | 0.5 | 1.2 | 2.5 | 3.8 | 5.3 | 7.3 | 9.1 | 12.7 | 15.9 | 18.9 | 21.1 | 23.9 | 30.3 | 36.5 | 42.4 |
|  | 4,000 |  |  |  | 0.8 | 1.9 | 2.9 | 4 | 5.7 | 7.3 | 10.5 | 13.4 | 16 | 18.2 | 20.8 | 27.1 | 33.5 | 39.5 |
|  | 4,250 |  |  |  |  | 1.1 | 2.2 | 3.2 | 4.8 | 6.2 | 9.3 | 12 | 14.6 | 16.7 | 19.1 | 25.3 | 31.5 | 37.3 |
|  | 4,500 |  |  |  |  |  | 1.1 | 1.9 | 3.3 | 4.5 | 7.1 | 9.6 | 12 | 14 | 16.3 | 22.4 | 28.5 | 34.2 |
|  | 4,750 |  |  |  |  |  |  | 0.8 | 2 | 2.8 | 5.1 | 7.4 | 9.7 | 11.6 | 13.8 | 19.8 | 25.8 | 31.4 |
|  | 5,000 |  |  |  |  |  |  |  | 1.1 | 1.8 | 3.7 | 5.8 | 8 | 9.7 | 11.8 | 17.7 | 23.8 | 26.6 |
|  | 5,250 |  |  |  |  |  |  |  |  | 0.8 | 2.4 | 4.2 | 6.2 | 7.7 | 9.4 | 14.9 | 21.1 | 26.8 |
|  | 5,500 |  |  |  |  |  |  |  |  |  | 1.5 | 3.2 | 5 | 6.1 | 7.8 | 13 | 19.1 | 24.6 |
|  | 6,000 |  |  |  |  |  |  |  |  |  |  | 1.3 | 2.7 | 3.8 | 5.3 | 10.2 | 15.9 | 21.2 |
|  | 6,500 |  |  |  |  |  |  |  |  |  |  |  | 2.8 | 1.3 | 2.6 | 6.9 | 12.1 | 17.2 |
|  | 7,000 |  |  |  |  |  |  |  |  |  |  |  |  | 0.5 | 1.3 | 4.8 | 9.4 | 14.3 |
|  | 7,500 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.7 | 3.8 | 8.1 | 12.7 |
|  | 8,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.8 | 6.7 | 10.9 |
|  | 9,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.6 | 5.3 |
|  | 10,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.9 |
|  | 11,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 17,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 19,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 21,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 27,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 11B-18 (cont.)

|  | Spawning Flow |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dewatering Flow |  | 12,000 | 13,000 | 14,000 | 15,000 | 17,000 | 19,000 | 21,000 | 23,000 | 25,000 | 27,000 | 29,000 | 31,000 |
|  | 3,250 | 56.6 | 63.7 | 70.7 | 76.8 | 84.2 | 86.5 | 88.5 | 89.1 | 91 | 91.3 | 93.1 | 94.7 |
|  | 3,500 | 52.9 | 60 | 67.1 | 73.6 | 81.4 | 84 | 86.4 | 87.4 | 89.9 | 90.5 | 92.3 | 94 |
|  | 3,750 | 49 | 55.9 | 63 | 69.7 | 77.8 | 80.9 | 84.3 | 85.9 | 88.9 | 89.7 | 91.7 | 93.8 |
|  | 4,000 | 46 | 52.9 | 60 | 66.8 | 74.9 | 78.2 | 82.1 | 84.1 | 88.1 | 89.4 | 91.6 | 93.7 |
|  | 4,250 | 43.6 | 50.3 | 57.3 | 64.1 | 72.3 | 75.6 | 79.8 | 82 | 86.8 | 88.3 | 91 | 93.3 |
|  | 4,500 | 40.3 | 46.9 | 53.7 | 60.5 | 69.4 | 73.1 | 77.4 | 79.4 | 84.3 | 86.3 | 89.7 | 92.2 |
|  | 4,750 | 37.3 | 43.7 | 50.2 | 57 | 66.1 | 70.1 | 74.6 | 77 | 83.1 | 85.5 | 89.2 | 91.9 |
|  | 5,000 | 35.4 | 41.7 | 48.2 | 55 | 64.1 | 68.2 | 72.8 | 75.2 | 82.1 | 85 | 88.8 | 91.6 |
|  | 5,250 | 32.6 | 38.7 | 45.2 | 51.9 | 61.3 | 66.1 | 70.8 | 73.2 | 79.3 | 82.9 | 88.1 | 90.8 |
|  | 5,500 | 30.1 | 36 | 42.2 | 48.8 | 58.2 | 63.6 | 69.2 | 71.9 | 78.2 | 82.1 | 87.2 | 89.9 |
|  | 6,000 | 26.6 | 32.3 | 38.4 | 44.7 | 53.8 | 58.8 | 64.6 | 67.7 | 74.9 | 79.2 | 84.3 | 86.8 |
|  | 6,500 | 22.9 | 28.7 | 34.5 | 40.4 | 48.6 | 52.6 | 58.2 | 61 | 69.2 | 74 | 79.2 | 81.4 |
|  | 7,000 | 19.9 | 25.7 | 31.6 | 37.5 | 46.2 | 50.2 | 56 | 59.1 | 67.5 | 72.2 | 77.3 | 79.4 |
|  | 7,500 | 18.2 | 24.1 | 30 | 35.8 | 44.4 | 48.2 | 54.1 | 57.3 | 66.2 | 71.1 | 76 | 78.2 |
|  | 8,000 | 16.3 | 22 | 27.7 | 33.4 | 42.1 | 46.4 | 52.7 | 55.9 | 64.6 | 69.5 | 75 | 77.2 |
|  | 9,000 | 9.6 | 14.5 | 19.7 | 25.7 | 35.2 | 40.4 | 47.2 | 50.7 | 60.2 | 65.3 | 71.1 | 73.5 |
|  | 10,000 | 4.6 | 8.9 | 13.4 | 18.9 | 27.7 | 33.7 | 41.4 | 45.6 | 55.7 | 61.7 | 68.3 | 70.8 |
|  | 11,000 | 2.8 | 6.8 | 10.9 | 15.7 | 24.3 | 29.5 | 37.4 | 42 | 52.8 | 58.7 | 65.1 | 67.7 |
|  | 12,000 |  | 3.1 | 6.3 | 10.4 | 18.9 | 25.1 | 33.9 | 38.9 | 50.3 | 56.5 | 63 | 65.7 |
|  | 13,000 |  |  | 5.4 | 5.4 | 12.7 | 19.7 | 29.1 | 36.4 | 48 | 54.6 | 61.2 | 64.1 |
|  | 14,000 |  |  |  | 3 | 9.5 | 15.6 | 25.1 | 32.3 | 44.5 | 51.7 | 58.3 | 61.5 |
|  | 15,000 |  |  |  |  | 5 | 10.9 | 20.7 | 29.5 | 42.1 | 49.3 | 55.8 | 58.8 |
|  | 17,000 |  |  |  |  |  | 4.8 | 13.4 | 20.8 | 34.1 | 42.5 | 49.7 | 53 |
|  | 19,000 |  |  |  |  |  |  | 7.3 | 13.3 | 26.4 | 35.7 | 43.1 | 46.6 |
|  | 21,000 |  |  |  |  |  |  |  | 6.8 | 20 | 29.2 | 36.3 | 39.9 |
|  | 23,000 |  |  |  |  |  |  |  |  | 13.5 | 20.5 | 26.9 | 31.2 |
|  | 25,000 |  |  |  |  |  |  |  |  |  | 4 | 9.3 | 14.6 |
|  | 27,000 |  |  |  |  |  |  |  |  |  |  | 3.9 | 9.2 |
|  | 29,000 |  |  |  |  |  |  |  |  |  |  |  | 5.1 |

Table 11B-19. Percent Redd Dewatered Look-up Table for CCV Steelhead with ACID Dam Boards In (the percent of redds dewatered are looked up at the intersection of the “Spawning Flow” columns and “Dewatering Flow” rows)

|  | Spawning Flow |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dewatering Flow |  | 3,500 | 3,750 | 4,000 | 4,250 | 4,500 | 4,750 | 5,000 | 5,250 | 5,500 | 6,000 | 6,500 | 7,000 | 7,500 | 8,000 | 9,000 | 10,000 | 11,000 |
|  | 3,250 | 1.1 | 2.3 | 3.3 | 4.7 | 6.5 | 8.7 | 11 | 13.6 | 16 | 20.3 | 23.9 | 26.9 | 29.3 | 31.8 | 37.6 | 42.3 | 46.7 |
|  | 3,500 |  | 1.4 | 2.2 | 3.2 | 4.6 | 6.4 | 8.4 | 10.8 | 13 | 17.1 | 20.6 | 23.7 | 26.1 | 28.6 | 34.5 | 39.2 | 43.5 |
|  | 3,750 |  |  | 0.6 | 1.3 | 2.6 | 4.1 | 5.9 | 8.1 | 10 | 13.6 | 17 | 20 | 22.5 | 25.1 | 31.2 | 35.9 | 40.3 |
|  | 4,000 |  |  |  | 0.9 | 2.1 | 3.3 | 4.7 | 6.7 | 8.3 | 11.6 | 14.6 | 17.4 | 19.7 | 22.2 | 28.3 | 33.3 | 37.8 |
|  | 4,250 |  |  |  |  | 1.3 | 2.6 | 4 | 5.8 | 7.2 | 10.3 | 13.2 | 15.9 | 18.1 | 20.5 | 26.5 | 31.3 | 35.7 |
|  | 4,500 |  |  |  |  |  | 1.4 | 2.7 | 4.2 | 5.5 | 8.2 | 10.8 | 13.3 | 15.4 | 17.6 | 23.6 | 28.4 | 32.7 |
|  | 4,750 |  |  |  |  |  |  | 1.5 | 2.9 | 3.8 | 6.2 | 8.5 | 11 | 12.9 | 15.1 | 20.9 | 25.7 | 30 |
|  | 5,000 |  |  |  |  |  |  |  | 1.7 | 2.4 | 4.4 | 6.5 | 8.8 | 10.6 | 12.6 | 18.3 | 23.1 | 27.5 |
|  | 5,250 |  |  |  |  |  |  |  |  | 1.1 | 2.6 | 4.6 | 6.5 | 8 | 9.6 | 15 | 19.7 | 24 |
|  | 5,500 |  |  |  |  |  |  |  |  |  | 1.5 | 3.2 | 4.8 | 6.2 | 7.7 | 12.8 | 17.5 | 21.6 |
|  | 6,000 |  |  |  |  |  |  |  |  |  |  | 1.3 | 2.7 | 3.8 | 5.1 | 9.9 | 14.3 | 18.3 |
|  | 6,500 |  |  |  |  |  |  |  |  |  |  |  | 2.7 | 1.4 | 2.5 | 6.9 | 10.8 | 14.8 |
|  | 7,000 |  |  |  |  |  |  |  |  |  |  |  |  | 0.5 | 1.3 | 4.9 | 8.4 | 12.2 |
|  | 7,500 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.7 | 4 | 7.3 | 10.8 |
|  | 8,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 | 5.9 | 9.2 |
|  | 9,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.2 | 4.4 |
|  | 10,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.6 |
|  | 11,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 17,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 19,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 21,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 27,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29,000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 11B-19 (cont.)

|  | Spawning Flow |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dewatering Flow |  | 12,000 | 13,000 | 14,000 | 15,000 | 17,000 | 19,000 | 21,000 | 23,000 | 25,000 | 27,000 | 29,000 | 31,000 |
|  | 3,250 | 50.5 | 53.5 | 55.6 | 56.3 | 54.1 | 49.5 | 46.8 | 42.3 | 39.1 | 38.3 | 37.7 | 39.2 |
|  | 3,500 | 47.4 | 50.6 | 52.9 | 54.1 | 52.3 | 48.1 | 45.6 | 41.3 | 38.2 | 37.6 | 37 | 38.5 |
|  | 3,750 | 44.2 | 47.4 | 49.9 | 51.4 | 50.6 | 46.3 | 44.4 | 40.4 | 37.6 | 37 | 36.5 | 38.1 |
|  | 4,000 | 41.7 | 45.1 | 47.7 | 49.4 | 48.3 | 44.8 | 43.2 | 39.4 | 37 | 36.5 | 36.2 | 37.8 |
|  | 4,250 | 36.5 | 42.8 | 45.5 | 47.3 | 46.6 | 43.2 | 41.7 | 38.2 | 36 | 35.6 | 35.4 | 37.1 |
|  | 4,500 | 36.6 | 39.8 | 42.6 | 44.6 | 44.5 | 41.5 | 40.1 | 36.5 | 34.2 | 34 | 34 | 35.8 |
|  | 4,750 | 33.7 | 37 | 39.7 | 41.8 | 42.1 | 39.4 | 38.2 | 34.8 | 32.9 | 32.8 | 33 | 34.8 |
|  | 5,000 | 31.2 | 34.4 | 37.2 | 39.4 | 39.8 | 37.2 | 36.2 | 32.8 | 31.1 | 31.1 | 31.1 | 32.8 |
|  | 5,250 | 27.9 | 31.1 | 33.8 | 36.2 | 36.9 | 34.8 | 33.8 | 30.3 | 28.2 | 28.4 | 28.9 | 30.4 |
|  | 5,500 | 25.3 | 28.4 | 31.1 | 33.5 | 34.5 | 32.8 | 32.3 | 28.9 | 26.8 | 27 | 27.3 | 28.8 |
|  | 6,000 | 21.9 | 25.1 | 27.8 | 30.2 | 31.3 | 29.7 | 29.4 | 26.3 | 24.3 | 24.5 | 24.8 | 26 |
|  | 6,500 | 18.7 | 22.1 | 27.8 | 27.1 | 28.1 | 26.2 | 25.9 | 22.9 | 21.2 | 21.5 | 21.7 | 22.8 |
|  | 7,000 | 16.2 | 19.6 | 22.5 | 24.9 | 26.4 | 24.7 | 24.5 | 21.7 | 19.9 | 20.2 | 20.4 | 21.4 |
|  | 7,500 | 14.8 | 18.3 | 21.2 | 23.7 | 25.2 | 23.5 | 23.5 | 20.7 | 19.1 | 19.3 | 19.4 | 20.4 |
|  | 8,000 | 13.1 | 16.6 | 19.5 | 21.9 | 23.7 | 22.2 | 22.5 | 19.7 | 18 | 18.1 | 18.5 | 19.5 |
|  | 9,000 | 7.6 | 10.8 | 13.6 | 16.6 | 19.4 | 18.7 | 19.3 | 16.8 | 15.2 | 15.4 | 15.9 | 17 |
|  | 10,000 | 3.6 | 6.6 | 9.2 | 12.1 | 15.1 | 15.3 | 16.4 | 14.5 | 12.9 | 13.4 | 14.3 | 15.5 |
|  | 11,000 | 2.3 | 5 | 7.5 | 10.1 | 13.1 | 13.1 | 14.5 | 12.8 | 11.5 | 11.9 | 12.8 | 14.1 |
|  | 12,000 |  | 2.2 | 4.3 | 6.7 | 10.1 | 10.9 | 12.9 | 11.4 | 10.4 | 10.9 | 11.9 | 13.2 |
|  | 13,000 |  |  | 3.7 | 3.6 | 6.8 | 8.3 | 10.7 | 10.5 | 9.6 | 10.3 | 11.3 | 12.7 |
|  | 14,000 |  |  |  | 2.1 | 5.1 | 6.6 | 9.1 | 9 | 8.3 | 9.2 | 10.3 | 11.9 |
|  | 15,000 |  |  |  |  | 2.6 | 4.2 | 7.2 | 7.9 | 7.4 | 8.3 | 9.4 | 10.9 |
|  | 17,000 |  |  |  |  |  | 1.9 | 5.1 | 5.8 | 5.6 | 6.8 | 8.3 | 10 |
|  | 19,000 |  |  |  |  |  |  | 3 | 3.7 | 3.8 | 5.1 | 6.7 | 8.4 |
|  | 21,000 |  |  |  |  |  |  |  | 1.4 | 1.8 | 2.9 | 4.4 | 6.3 |
|  | 23,000 |  |  |  |  |  |  |  |  | 0.9 | 2.2 | 3.8 | 5.7 |
|  | 25,000 |  |  |  |  |  |  |  |  |  | 1.7 | 3.4 | 5.4 |
|  | 27,000 |  |  |  |  |  |  |  |  |  |  | 1.8 | 3.8 |
|  | 29,000 |  |  |  |  |  |  |  |  |  |  |  | 2.2 |

### Feather River

Spring-run, fall-run, and steelhead spawn in both the upper Feather River between the Fish Barrier Dam and Thermalito Afterbay Outlet (Low Flow Channel or LFC) and the lower river downstream of the Thermalito Afterbay Outlet (High Flow Channel or HFC). Results of escapement surveys conducted since 2002 (Kindopp pers. comm. 2021a) show that the LFC is preferred for spawning over the HFC and that this preference has increased over time (Figure 11B-11). However, Alternatives 1–3 would have no effect on flow in the LFC, so differences in redd dewatering between the alternatives and the NAA were estimated only for the HFC.

Figure 11B-11. Escapement Population Estimates for Fall-run Chinook Salmon in the Low Flow Channel (LFC) and High Flow Channel (HFC) of the Feather River from 2000 through 2019, from Escapement Surveys of CDWR.

No redd dewatering field data similar to that used for the Sacramento River (USFWS 2006) or the American River (see below) are available for salmon or steelhead in the Feather River; therefore, the reduction in flow from the spawning to the dewatering flow was used directly as a proxy for redd dewatering. The spawning and dewatering flows downstream of the Thermalito Afterbay outlet for each month of spring-run, fall-run, and steelhead spawning, as estimated by CALSIM II, were used to compute the reduction in flow under Alternatives 1–3 and the NAA. Larger reductions are assumed to increase the percent of redds dewatered and, therefore, to have a potentially negative effect on the species’ populations. As previously noted, the use of monthly time-step flow estimates like those obtained from CALSIM II modeling likely underestimates redd dewatering rates. This potential bias is expected to equally affect the NAA and Alternatives 1–3.

### American River

The redd dewatering analysis for the lower American River used relationships between flow, river stage, and redd depth distribution developed by Bratovich et al. (2017). A composite redd depth frequency distribution was developed by combining results from several redd surveys conducted between 1996 and 2016. The stage vs. flow relationship for the river was developed from a combination of field measurements and modeling. CALSIM II flow estimates at the Nimbus Dam location were used to compute stage at the spawning and dewatering flows, and the redd depth frequency distribution was queried to determine the percentage of the redds that occur between those two stages and would therefore be dewatered. The analyses were conducted for fall-run and steelhead spawning and incubation periods for each year of the CALSIM period of record. Based on ranges provided in Bratovich et al. 2017, fall-run and steelhead were estimated to have three-month and two-month incubation periods, respectively. The analysis compared CALSIM II flow estimates below Nimbus Dam for each spawning month with the minimum flow during 2 or 3 months following the spawning month to estimate the percentage of redds dewatered. Absolute differences between Alternatives 1–3 and the NAA in the percentage of redds dewatered were used to compare the alternatives and the NAA. As noted above, the use of monthly time-step flow estimates like those obtained from CALSIM II modeling likely underestimates redd dewatering rates. This potential bias is expected to affect all Project scenarios equally.

## Redd Scour/Entombment

Loss of redds to scouring or entombment occurs when flows are high enough to mobilize sediments, destroying redds and their incubating eggs and alevins, or entombing the redds when sediments are redeposited. Estimates of redd losses resulting from scouring flows in the Sacramento and American Rivers were based on estimates from various sources of the minimum flows required to mobilize sediments and the frequency of occurrence of those flows. Frequency of scouring flows was not estimated for the Feather River because information on minimum flows required to mobilize sediments could not be located for the Feather River.

The probability of flows occurring that would be high enough to mobilize sediments and scour or entomb Chinook salmon and steelhead redds was estimated for the Sacramento and American Rivers. The amount of flow needed to mobilize sediments in these rivers has been little studied (Kondolf et al. 2000; Ayres 2001), but the information available suggests that a minimum of roughly 40,000 cubic feet per second (cfs) of flow is required in both rivers for significant bed movement (scour flow threshold) (Table 11B-20). It should be noted that 40,000 cfs is likely to be a conservative estimate for redd scour because, due to the areas of a streambed that salmonids typically select for redd construction, the flows needed to scour redds may be significantly greater than those that initiate bed mobility (May et al. 2009). A search of the literature found no corresponding estimate of scouring flow for the Feather River.

Table 11B-20. Estimated Bed Mobility Flows for the Sacramento and American Rivers

| River | Approximate flow ranges to initiate mobility (cfs) | References |
| --- | --- | --- |
| Sacramento River | 24,000–50,000 | Kondolf et al. 2000; Cain and Monohan 2008 |
| American River | 26,500–50,000 | Ayres Associates 2001; Fairman 2007 |

For the Sacramento River, the frequency of flows exceeding 40,000 cfs for Alternatives 1–3 and the NAA during the spawning and egg incubation periods of winter-run, spring-run, fall-run and late fall–run Chinook salmon and steelhead were estimated from USRDOM estimates of daily flows.

No estimated daily flows for the American River under Alternatives 1–3 and the NAA are available; only CALSIM II estimates are available. Redd scour can occur at a very small temporal scale (minutes to hours), whereas CALSIM II provides mean monthly flow estimates. In an attempt to overcome this discrepancy in temporal scales, historical monthly and daily flow data during December through April (when scour is most likely to occur) were plotted to determine whether the probability of occurrence of daily flows above the scour flow threshold could be predicted with monthly flow data. The purpose was to find the minimum monthly flow value at which the maximum daily flow in that month would always be greater than the 40,000-cfs scour flow threshold. The actual monthly and daily flow data used in the analysis are from gauge records at Hazel Avenue, and the CALSIM II estimates used to compare probabilities of redd scour for Alternatives 1–3 and the NAA are for the Nimbus Dam location. The Nimbus Dam location is immediately upstream of the Hazel Avenue gauge location. The analysis of the Hazel Avenue gauge data shows that for months with a mean monthly flow of at least 19,350 cfs, the maximum daily flow in that month is always at least 40,000 cfs (Figure 11B-12). Therefore, redd scour probabilities for Alternatives 1–3 and the NAA were evaluated by comparing frequencies of CALSIM II flows greater than 19,350 cfs at Nimbus Dam during the fall-run and steelhead spawning and incubation periods.

Figure 11B-12. Relationship between Mean Monthly Flows and Maximum Daily Flows during December through April, American River Downstream of Hazel Avenue, 1950–2015. Minimum monthly flow is identified in red.

## SALMOD

The SALMOD model was used to evaluate flow- and temperature-related mortality of early life stages and overall production of each race of Chinook salmon in the Sacramento River. Appendix 11H, Salmonid Population Modeling (SALMOD), provides details and outputs of the model and Chapter 11, Aquatic Biological Resources, Impacts FISH-2, FISH-3, and FISH-4 include interpretation of model outputs.

## Rearing Habitat Weighted Usable Area

The availability of rearing habitat was estimated using WUA curves obtained from the literature (U.S. Fish and Wildlife Service 2005b). WUA is an index of the surface area of physical habitat available, weighted by the suitability of that habitat. WUA curves are normally developed as part of instream flow incremental methodology (IFIM) studies. Rearing habitat WUA was estimated only for the Sacramento River because no adequate flow versus rearing WUA curves located for the Feather or American River. The available flow versus rearing WUA information for these rivers is old, limited, and potentially unreliable (Thomas R. Payne & Associates 2002).

### Sacramento River

The rearing habitat WUA curves used for Chinook salmon rearing habitat in the Sacramento River were obtained from a U.S. Fish and Wildlife Service report (U.S. Fish and Wildlife Service 2005b). As noted above for spawning habitat, WUA is computed as the surface area of physical habitat available weighted by its suitability. Modeling assumptions used to derive rearing WUA curves include that the suitability of physical habitat for salmon and steelhead rearing is largely a function of water depth, flow velocity, and the availability and type of cover. The race- or species-specific suitability of the habitat with respect to these variables is determined by observing the fish and is used to develop HSC for each race or species. Hydraulic modeling is then used to estimate the amount of rearing habitat available for different HSC levels at different river flows, and the results are used to develop rearing habitat WUA curves and tables (Bovee et al. 1998). These curves and tables are used to look up the amount of rearing WUA available at different flows.

U.S. Fish and Wildlife Service (2005b) provides WUA curves and tables for rearing winter-run, fall-run, and late fall–run Chinook salmon for three segments of the Sacramento River encompassing the reach from Keswick Dam to Battle Creek (Figure 11B-1). Separate curves were developed for fry and juveniles, with fry defined as fish less than 60 millimeters and juveniles defined as greater than 60 millimeters. No WUA curves were developed for spring-run Chinook salmon or steelhead, but as discussed later, the fall-run curves were used to quantify spring-run rearing habitat and the late fall–run curves were used for steelhead. Figure 11B-13 through Figure 11B-18 show the flow versus rearing WUA results for fry and juvenile winter-run, fall-run, and late fall–run Chinook salmon in the three river segments (Segment 6 = Keswick to ACID Dam, Segment 5 = ACID Dam to Cow Creek, and Segment 4 = Cow Creek to Battle Creek) as provided in U.S. Fish and Wildlife Service (2006). Note that for Segment 6, separate WUA curves were developed for periods when the ACID Dam boards are installed (April through October) and for when the boards are out because installation of the boards affects water depths and velocities for some of the sampling transects used to develop the curves. All rearing WUA analyses were limited to juveniles less than a year old.

Because tributaries enter the Sacramento River between Keswick Dam and Battle Creek, flows are generally different among the three segments. For the U.S. Fish and Wildlife Service studies, flows were measured directly at the sampling transects and were estimated as the sum of Keswick flow releases and tributary gage readings upstream of the transects. To estimate WUA for the effects analysis, the segment flows were estimated using Sacramento River CALSIM II flows at Keswick Dam, the Clear Creek confluence, and Battle Creek for Segments 6, 5, and 4, respectively. Keswick Dam flows were used for Segment 5 upstream of the Clear Creek confluence. For Segment 6, the WUA curves for the months when the ACID Dam boards are installed (April through October) were used with the flows for those months and the WUA curves for the months when the ACID Dam boards are out were used with the flows for the rest of the year.

Although fall-run rearing WUA curves were used as surrogates for spring-run rearing, CALSIM II flows for the months of spring-run rearing, not those of fall-run rearing, were used to compute the spring-run WUA results. This caveat applies as well to the use of the late fall–run rearing WUA curves to compute steelhead WUA results.

Figure 11B-13. Rearing WUA curves for Winter-Run Chinook Salmon Fry in the Sacramento River, Segments 4 to 6. ACID = Anderson-Cottonwood Irrigation District.

Figure 11B-14. Rearing WUA curves for Winter-Run Chinook Salmon Juveniles in the Sacramento River, Segments 4 to 6. ACID = Anderson-Cottonwood Irrigation District.

Figure 11B-15. Rearing WUA Curves for Fall-Run Chinook Salmon Fry in the Sacramento River, Segments 4 to 6. (The fall-run curves were used to quantify spring-run Chinook salmon WUA, as discussed in the text.) ACID = Anderson-Cottonwood Irrigation District.

Figure 11B-16. Rearing WUA Curves for Fall-Run Chinook Salmon Juveniles in the Sacramento River, Segments 4 to 6. (The fall-run curves were used to quantify spring-run Chinook salmon WUA, as discussed in the text.) ACID = Anderson-Cottonwood Irrigation District.

Figure 11B-17. Rearing WUA Curves for Late Fall–Run Chinook Salmon Fry in the Sacramento River, Segments 4 to 6. (The late fall–run rearing curves were used to quantify steelhead rearing WUA, as discussed in the text.) ACID = Anderson-Cottonwood Irrigation District.

Figure 11B-18. Rearing WUA Curves for Late Fall–Run Chinook Salmon Juveniles in the Sacramento River, Segments 4 to 6. (The late fall–run rearing curves were used to quantify steelhead rearing WUA, as discussed in the text.) ACID = Anderson-Cottonwood Irrigation District.

As noted above, there are no spring-run or steelhead rearing WUA curves in the U.S. Fish and Wildlife Service documentation, so the fall-run and late fall–run Chinook salmon rearing WUA curves were used as surrogates to model rearing habitat for spring-run and steelhead, respectively. Mark Gard, who led the U.S. Fish and Wildlife Service studies that produced the Sacramento River WUA curves, has endorsed this practice for both spring-run Chinook salmon and steelhead (Gard pers. comm.). The use of these substitutions has been adopted in subsequent studies. For instance, the SacEFT model, which produces spawning and rearing WUA outputs for CV spring-run Chinook salmon and steelhead, derives the spring-run spawning and rearing WUA results using the fall-run WUA curves as surrogates and the steelhead rearing WUA results using the late fall–run WUA curves as surrogates (ESSA 2011; Robinson pers. comm.). It should be noted that this practice introduces additional uncertainty to the spring-run and steelhead results.

A potential limitation of the WUA curves presented above, as of all such habitat-based studies, is that they assume the channel characteristics of the river during the time of field data collection by U.S. Fish and Wildlife Service (1995–1999), such as proportions of mesohabitat types, have remained in dynamic equilibrium to the present time and would continue to do so through the life of the Project. If the channel characteristics substantially change, the shape of the curves may no longer be applicable. A further limitation is that the curves were developed for the Sacramento River upstream of Battle Creek, but all races of Chinook salmon and steelhead spend time rearing downstream of this part of the river.

Differences in the mean rearing WUA under Alternatives 1–3 and the NAA were examined for the months of the fry and juvenile rearing period under each water year type and all water year types combined. Differences in mean rearing WUA of greater than 5% were flagged in the results tables to highlight results with the largest differences.

## Juvenile Stranding

Juvenile stranding occurs when water level drops and the juveniles become isolated from suitable habitat. Juvenile salmon typically rest in shallow slow-moving water between feeding forays into swifter water, which makes them particularly susceptible to stranding during rapid reductions in flow (Jarrett and Killam 2015).

A juvenile stranding analysis for the Sacramento River was developed using a functional relationship developed in field studies by USFWS (2006). The juvenile stranding analysis is conceptually similar to the redd dewatering analysis in that both compare water elevation at an initial flow with that at the minimum flow during the following period. A period of 3 months is used for the juvenile stranding analysis in this report because the juveniles are presumed to be most vulnerable to stranding during their first 3 months (i.e., fry stage). The juvenile stranding analysis (USFWS 2006) computes the area of salmonid rearing habitat inundated at the initial flow that is dewatered at the minimum (stranding) flow and converts this area to number of stranded juveniles using estimates of habitat capacity based on field study observations. The estimates of area of rearing habitat available at different flows are based on a combination of field observations and modeling (USFWS 2006).

Juvenile stranding is computed using USRDOM daily flow estimates for Alternatives 1–3 and the NAA at three locations in the upper Sacramento River: Keswick Dam, Clear Creek, and Battle Creek. Separate tables for converting initial and stranding flows to number of juveniles stranded were developed for periods when the ACID Dam boards are in and when they are out (Table 11B-21 and Table 11B-22). Both tables are used for all the salmonid species and races.

Table 11B-21. Juvenile Stranding Look-up Table for Chinook Salmon and Steelhead in the Sacramento River with ACID Dam Boards Out (numbers of juveniles stranded are looked up at the intersection of the “Initial Flow” columns and “Stranding Flow” rows)

|  | Initial Flow |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stranding Flow |  | 3,500 | 3,750 | 4,000 | 4,250 | 4,500 | 4,750 | 5,000 | 5,250 | 5,500 | 6,000 | 6,500 |
|  | 3,250 | 1,097 | 11,227 | 11,895 | 13,095 | 14,598 | 16,654 | 16,819 | 16,939 | 17,494 | 20,250 | 20,860 |
|  | 3,500 |  | 10,130 | 10,798 | 11,998 | 13,501 | 15,557 | 15,722 | 15,842 | 16,397 | 19,153 | 19,763 |
|  | 3,750 |  |  | 668 | 1,868 | 3,371 | 5,427 | 5,592 | 5,712 | 6,267 | 9,023 | 9,633 |
|  | 4,000 |  |  |  | 1,200 | 2,703 | 4,759 | 4,925 | 5,044 | 5,599 | 8,355 | 8,965 |
|  | 4,250 |  |  |  |  | 1,503 | 3,559 | 3,725 | 3,844 | 4,399 | 7,155 | 7,765 |
|  | 4,500 |  |  |  |  |  | 2,056 | 2,222 | 2,341 | 2,896 | 5,652 | 6,262 |
|  | 4,750 |  |  |  |  |  |  | 185 | 304 | 859 | 3,615 | 4,225 |
|  | 5,000 |  |  |  |  |  |  |  | 139 | 694 | 3,450 | 4,060 |
|  | 5,250 |  |  |  |  |  |  |  |  | 574 | 3,330 | 3,940 |
|  | 5,500 |  |  |  |  |  |  |  |  |  | 2,775 | 3,385 |
|  | 6,000 |  |  |  |  |  |  |  |  |  |  | 629 |
|  | 6,500 |  |  |  |  |  |  |  |  |  |  |  |
|  | 7,000 |  |  |  |  |  |  |  |  |  |  |  |
|  | 7,500 |  |  |  |  |  |  |  |  |  |  |  |
|  | 8,000 |  |  |  |  |  |  |  |  |  |  |  |
|  | 9,000 |  |  |  |  |  |  |  |  |  |  |  |
|  | 10,000 |  |  |  |  |  |  |  |  |  |  |  |
|  | 11,000 |  |  |  |  |  |  |  |  |  |  |  |
|  | 12,000 |  |  |  |  |  |  |  |  |  |  |  |
|  | 13,000 |  |  |  |  |  |  |  |  |  |  |  |
|  | 14,000 |  |  |  |  |  |  |  |  |  |  |  |

Table 11B-21 (cont.)

|  |  | Initial Flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stranding Flow |  | 7,000 | 7,500 | 8,000 | 9,000 | 10,000 | 11,000 | 12,000 | 13,000 | 14,000 | 15,000 |
|  | 3,250 | 20,954 | 21,024 | 21,953 | 22,764 | 23,084 | 23,193 | 23,230 | 23,239 | 23,253 | 23,420 |
|  | 3,500 | 19,857 | 19,927 | 20,856 | 21,668 | 21,987 | 22,096 | 22,133 | 22,142 | 22,156 | 22,323 |
|  | 3,750 | 9,727 | 9,797 | 10,726 | 11,538 | 11,857 | 11,966 | 12,003 | 12,012 | 12,026 | 12,193 |
|  | 4,000 | 9,059 | 9,129 | 10,059 | 10,870 | 11,189 | 11,298 | 11,335 | 11,344 | 11,358 | 11,525 |
|  | 4,250 | 7,859 | 7,929 | 8,858 | 9,670 | 9,989 | 10,098 | 10,135 | 10,144 | 10,158 | 10,325 |
|  | 4,500 | 6,356 | 6,426 | 7,355 | 8,167 | 8,486 | 8,595 | 8,632 | 8,641 | 8,655 | 8,822 |
|  | 4,750 | 4,319 | 4,389 | 5,319 | 6,130 | 6,449 | 6,558 | 6,595 | 6,604 | 6,618 | 6,785 |
|  | 5,000 | 4,154 | 4,224 | 5,153 | 5,964 | 6,284 | 6,393 | 6,430 | 6,439 | 6,453 | 6,620 |
|  | 5,250 | 4,034 | 4,104 | 5,033 | 5,845 | 6,164 | 6,273 | 6,310 | 6,319 | 6,333 | 6,500 |
|  | 5,500 | 3,479 | 3,549 | 4,479 | 5,290 | 5,609 | 5,718 | 5,755 | 5,764 | 5,778 | 5,945 |
|  | 6,000 | 723 | 793 | 1,723 | 2,534 | 2,853 | 2,962 | 2,999 | 3,008 | 3,022 | 3,189 |
|  | 6,500 | 114 | 183 | 1,113 | 1,924 | 2,243 | 2,353 | 2,390 | 2,399 | 2,413 | 2,579 |
|  | 7,000 |  | 89 | 1,018 | 1,830 | 2,149 | 2,258 | 2,295 | 2,304 | 2,318 | 2,485 |
|  | 7,500 |  |  | 949 | 1,760 | 2,079 | 2,188 | 2,226 | 2,234 | 2,249 | 2,415 |
|  | 8,000 |  |  |  | 811 | 1,131 | 1,240 | 1,277 | 1,286 | 1,300 | 1,466 |
|  | 9,000 |  |  |  |  | 319 | 428 | 466 | 474 | 489 | 655 |
|  | 10,000 |  |  |  |  |  | 109 | 146 | 155 | 169 | 336 |
|  | 11,000 |  |  |  |  |  |  | 37 | 46 | 60 | 227 |
|  | 12,000 |  |  |  |  |  |  |  | 9 | 23 | 190 |
|  | 13,000 |  |  |  |  |  |  |  |  | 14 | 181 |
|  | 14,000 |  |  |  |  |  |  |  |  |  | 167 |

Table 11B-22. Juvenile Stranding Look-up Table for Chinook Salmon and Steelhead in the Sacramento River with ACID Dam Boards In (numbers of juveniles stranded are looked up at the intersection of the “Initial Flow” columns and “Stranding Flow” rows)

|  | Initial Flow |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stranding Flow |  | 3,500 | 3,750 | 4,000 | 4,250 | 4,500 | 4,750 | 5,000 | 5,250 | 5,500 | 6,000 | 6,500 |
|  | 3,250 | 1,097 | 11,227 | 11,895 | 13,095 | 14,598 | 16,671 | 17,441 | 17,847 | 18,402 | 21,158 | 21,768 |
|  | 3,500 |  | 10,130 | 10,798 | 11,998 | 13,501 | 15,574 | 16,344 | 16,750 | 17,305 | 20,061 | 20,671 |
|  | 3,750 |  |  | 668 | 1,868 | 3,371 | 5,444 | 6,214 | 6,620 | 7,175 | 9,931 | 10,541 |
|  | 4,000 |  |  |  | 1,200 | 2,703 | 4,776 | 5,546 | 5,953 | 6,507 | 9,264 | 9,873 |
|  | 4,250 |  |  |  |  | 1,503 | 3,576 | 4,346 | 4,753 | 5,307 | 8,063 | 8,673 |
|  | 4,500 |  |  |  |  |  | 2,073 | 2,843 | 3,249 | 3,804 | 6,560 | 7,170 |
|  | 4,750 |  |  |  |  |  |  | 789 | 1,196 | 1,751 | 4,507 | 5,116 |
|  | 5,000 |  |  |  |  |  |  |  | 426 | 981 | 3,737 | 4,346 |
|  | 5,250 |  |  |  |  |  |  |  |  | 574 | 3,330 | 3,940 |
|  | 5,500 |  |  |  |  |  |  |  |  |  | 2,775 | 3,385 |
|  | 6,000 |  |  |  |  |  |  |  |  |  |  | 629 |
|  | 6,500 |  |  |  |  |  |  |  |  |  |  |  |
|  | 7,000 |  |  |  |  |  |  |  |  |  |  |  |
|  | 7,500 |  |  |  |  |  |  |  |  |  |  |  |
|  | 8,000 |  |  |  |  |  |  |  |  |  |  |  |
|  | 9,000 |  |  |  |  |  |  |  |  |  |  |  |
|  | 10,000 |  |  |  |  |  |  |  |  |  |  |  |
|  | 11,000 |  |  |  |  |  |  |  |  |  |  |  |
|  | 12,000 |  |  |  |  |  |  |  |  |  |  |  |
|  | 13,000 |  |  |  |  |  |  |  |  |  |  |  |
|  | 14,000 |  |  |  |  |  |  |  |  |  |  |  |

Table 11B-22 (cont.)

|  |  | Initial Flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stranding Flow |  | 7,000 | 7,500 | 8,000 | 9,000 | 10,000 | 11,000 | 12,000 | 13,000 | 14,000 | 15,000 |
|  | 3,250 | 21,893 | 21,932 | 22,861 | 23,823 | 23,602 | 23,711 | 23,753 | 23,757 | 23,771 | 23,938 |
|  | 3,500 | 20,796 | 20,835 | 21,765 | 22,186 | 22,505 | 22,614 | 22,656 | 22,660 | 22,675 | 22,841 |
|  | 3,750 | 10,666 | 10,705 | 11,635 | 12,056 | 12,375 | 12,485 | 12,526 | 12,531 | 12,545 | 12,711 |
|  | 4,000 | 9,998 | 10,037 | 10,967 | 11,388 | 11,708 | 11,817 | 11,858 | 11,863 | 11,877 | 12,044 |
|  | 4,250 | 8,798 | 8,837 | 9,767 | 10,188 | 10,508 | 10,617 | 10,658 | 10,663 | 10,677 | 10,843 |
|  | 4,500 | 7,295 | 7,334 | 8,264 | 8,685 | 9,004 | 9,114 | 9,155 | 9,160 | 9,174 | 9,340 |
|  | 4,750 | 5,241 | 5,281 | 6,210 | 6,631 | 6,951 | 7,060 | 7,101 | 7,106 | 7,120 | 7,287 |
|  | 5,000 | 4,471 | 4,510 | 5,440 | 5,861 | 6,181 | 6,290 | 6,331 | 6,336 | 6,350 | 6,517 |
|  | 5,250 | 4,065 | 4,104 | 5,033 | 5,455 | 5,774 | 5,883 | 5,925 | 5,929 | 5,943 | 6,110 |
|  | 5,500 | 3,510 | 3,549 | 4,479 | 4,900 | 5,219 | 5,329 | 5,370 | 5,375 | 5,389 | 5,555 |
|  | 6,000 | 754 | 793 | 1,723 | 2,144 | 2,463 | 2,572 | 2,614 | 2,618 | 2,633 | 2,799 |
|  | 6,500 | 144 | 183 | 1,113 | 1,534 | 1,854 | 1,963 | 2,004 | 2,009 | 2,023 | 2,190 |
|  | 7,000 |  | 58 | 988 | 1,409 | 1,729 | 1,838 | 1,879 | 1,884 | 1,898 | 2,065 |
|  | 7,500 |  |  | 949 | 1,370 | 1,690 | 1,799 | 1,840 | 1,845 | 1,859 | 2,025 |
|  | 8,000 |  |  |  | 421 | 741 | 850 | 891 | 896 | 910 | 1,077 |
|  | 9,000 |  |  |  |  | 319 | 428 | 470 | 474 | 489 | 655 |
|  | 10,000 |  |  |  |  |  | 109 | 151 | 155 | 169 | 336 |
|  | 11,000 |  |  |  |  |  |  | 41 | 46 | 60 | 227 |
|  | 12,000 |  |  |  |  |  |  |  | 5 | 19 | 185 |
|  | 13,000 |  |  |  |  |  |  |  |  | 14 | 181 |
|  | 14,000 |  |  |  |  |  |  |  |  |  | 167 |

As noted above, fry are likely the most vulnerable juveniles to the stranding flows and, therefore, stranding flows are assumed to cause the greatest mortality to the salmon runs or steelhead during the months that the fry are present. The seasonal presence of fry and early juveniles of each of the salmonid races and species was estimated from information on the spawning, incubation, and fry emergence periods in Appendix 11A, Aquatic Species Life Histories, and results of the USFWS RBDD Rotary Screw Trap and Sacramento River Beach Seine sampling efforts. The estimated fry and early juvenile rearing periods are: (1) July through October for winter-run, (2) November through February for spring-run, (3) December through March for fall-run, (4) March to June for late fall–run, and (5) February through May for steelhead).

## Upstream Migration of Salmon and Sturgeon Adults

Most potential effects of flow on upstream migrating anadromous fish were not analyzed because adequate analysis procedures were unavailable. The only exception is effects of very low flow that may result in passage barriers, which is discussed below. Other flow effects on adult salmon migrations are primarily related to flow velocities in the river channel and the bioenergetics of the fish (Enders et al. 2003; Liao 2007; Martin et al. 2015). While effects of uniform flow velocities on adult salmon bioenergetics are reasonably well understood (Enders et al. 2003), many natural settings, such as the river channel of the upstream reaches of the lower Sacramento, Feather and American Rivers, have turbulent flow with complex flow velocity fields. These velocity fields vary greatly with discharge. Research has demonstrated that upstream migrating salmon intersperse periods of rest in flow velocity refuges, including complex flow vortices, with spurts of high energy expenditure through channel sections with higher flow velocities (Liao 2007). The bioenergetics of the salmon, which are highly complex, depend on the path through the flow velocity fields selected by the salmon, which change with the amount of flow in the river (Liao 2007; Martin et al. 2015). Adding to the complexity, the effects of temperature on the salmon bioenergetics interact with those of the flow velocities (Martin et al. 2015). Ultimately, what is needed to assess the effects of different flows on upstream migrating salmon is a model that integrates river channel hydraulics, water temperatures, migratory behaviors of the salmon, and their bioenergetics for different combinations of flow and temperature. No such model is currently available.

Effects on upstream migrating adult salmon or sturgeon may occur if reduced flows result in inadequate water depth or flow over barriers for passage of adult fish. Note that if periods of low flow last only a few days and are not very frequent, they probably have little effect on the fish because the fish can wait in deeper water until passage conditions improve. The specific flow level at which passage difficulties for migrating adults first appear is not known for the Sacramento, Feather, or American Rivers. Therefore, threshold flows were selected based on the expert judgment of biologists who have long experience from observing fish in these rivers at many different flows.

The required minimum flow release from Keswick Dam is 3,250 CFS (NCWA 2019). This flow was selected as the threshold for potential obstruction of upstream passage for the Sacramento River because the river’s flow rarely drops below this level and salmonid adults have not been observed experiencing any passage difficulties at flows approaching this level (Killam pers. comm.). As such, it represents a conservative minimum flow above which fish do not experience migration difficulties. However, there have not been opportunities to observe whether fish experience passage difficulties below this level. A 1,000 cfs flow threshold was selected for the American River in this analysis because this is the approximate flow at which adult fall-run Chinook salmon have been first observed to delay upstream movement to spawning grounds (Kundargi pers. comm.).

A threshold flow for upstream passage in the lower Feather River was more difficult to determine. The primary Feather River passage obstruction is a boulder weir at the Sunset Pumps in the Feather River at Live Oak (Thom 2017; Seesholtz pers. comm). This weir creates a partial barrier to the only confirmed spawning location of green sturgeon in the Feather River (Seesholtz et al. 2014). USFWS (2016) indicates that the boulder weir is a barrier to upstream passage of green sturgeon when Feather River flow is less than 6,000 cfs. Given the absence of information indicating passage at lower flows, 6,000 cfs flow was selected as the threshold flow for upstream passage of sturgeon in the Feather River. Adult salmonids are able to pass above the Sunset Pumps weir at 1,500 cfs or less (Kindopp pers. comm. 2021), so 1,500 cfs was selected as the threshold flow for upstream passage of salmonids. The recovery plan for the southern DPS of green sturgeon lists removal or modification of the Sunset Pumps boulder weir as a high priority recovery action (Thom 2017), but it is not clear when such measures would be implemented (Seesholtz pers. comm.).

Three locations in the Sacramento River (Keswick Dam, RBDD, and Wilkins Slough), one location in the Feather River (below Thermalito Afterbay outlet), and one location in the American River (below Nimbus Dam) were selected for this analysis of low flows. For the Sacramento River, USRDOM daily flow estimates at Keswick Dam and the RBDD and CALSIM II monthly flow estimates at Wilkins Slough were used. CALSIM II monthly flow estimates were also used for the locations on the Feather and American Rivers. For each species and location, the percent of days (Keswick Dam and RBDD) or months (other locations) during the adult immigration period that modeled flows were lower than the minimum flow thresholds were calculated for Alternatives 1–3 and the NAA, and the differences in these percentages between Alternatives 1–3 and the NAA were determined.

## References

Yellow = Missing full reference

### Printed References

Adams, B.L., Zaugg, W.S., and McLain, L.R. 1975. Inhibition of salt water survival and Na-K-ATPase elevation in steelhead trout (Salmo gairdneri) by moderate water temperatures. Transactions of the American Fisheries Society. 104(4):766–769.

Alderdice, D. F., and F. P. J. Velsen. 1978. Relation between temperature and incubation time for eggs of chinook salmon (Oncorhynchus tshawytscha). Journal of the Fisheries Research Board of Canada 35:60–75.

Anderson, J. J. 2018. Using river temperature to optimize fish incubation metabolism and survival: a case for mechanistic models. bioRxiv:257154. Available: http://dx.doi.org/10.1101/257154. Accessed: July 3, 2019.

Ayres Associates, 2001. Two-dimensional modeling and analysis of spawning bed mobilization Lower American River: Report prepared for Sacramento District Army Corps of Engineers. City, State.

Bartholow, J.M. 2004. Modeling Chinook salmon with SALMOD on the Sacramento River, California. Hydroécologie Appliquée. 14(1):193-219.

Beakes, M. P., J. W. Moore, S. A. Hayes, and S. M. Sogard. 2014. Wildfire and the effects of shifting stream temperature on salmonids. Ecosphere 5(5):63.

Bell, C. E., and B. Kynard. 1986. Mortality of adult American shad passing through a 17-megawatt kaplan turbine at a low-head hydroelectric dam. North American Journal of Fisheries Management 5:33–38.

Bovee, K. D., B. L. Lamb, J. M. Bartholow, C. B. Stalnaker, J. Taylor, and J. Henriksen. 1998. Stream Habitat Analysis Using the Instream Flow Incremental Methodology. U.S. Geological Survey, Biological Resources Division Information and Technology Report USGS/BRD-1998-0004. viii + 131 pp.

Cain, J. and C. Monohan. 2008. Estimating ecologically based flow targets for the Sacramento and Feather Rivers. The Natural Heritage Institute (NHI). 74pp. Available: http://www.n-h-i.org/uploads/tx_rtgfiles/FINAL_Sacramento_and_Feather_Env_Flows_Doc.pdf. Accessed: 6/1/2015.

Cech, J.J., Mitchell, S.J., Castleberry, D.T. et al. Distribution of California stream fishes: influence of environmental temperature and hypoxia. Environ Biol Fish 29, 95–105 (1990).

Enders, Eva & Boisclair, Daniel & Roy, André. (2003). The Effect of Turbulence on the Cost of Swimming for Juvenile Atlantic Salmon. Canadian Journal of Fisheries and Aquatic Sciences. 60. 10.1139/f03-101.

Erickson, D. L., J. A. North, J. E. Hightower, J. Weber, and L. Lauck. 2002. Movement and habitat use of green sturgeon Acipenser medirostris in the Rogue River, Oregon, USA. Journal of Applied Ichthyology 18:565–569.

ESSA Technologies Ltd. 2011. Sacramento River Ecological Flows Tool (SacEFT): Record of Design (v.2.00). Prepared for The Nature Conservancy. Chico, CA.

Fairman, D. 2007. A Gravel Budget for the Lower American River. Master’s Thesis, Sacramento State University. Sacramento, CA. xvi +158pp. Available: http://www.csus.edu/indiv/h/hornert/Dave%20Fairman%20thesis%20gravel%20budget%20American%20River.pdf. Accessed: 6/1/2015

Grabowski SJ. 1973. Effects of fluctuating and constant temperatures on some hematological characteristics, tissue glycogen levels, and growth of steelhead trout (Salmo gairdneri). Ph.D. dissertation. University of Idaho. 77 pp.

Hoar, W.S. 1988. The physiology of smolting salmonids. Pages 275-326 in W.S. Hoar and J.R. Randall, editors. Fish Physiology, Volume XIB, Academic Press, New York.

Hokanson, K. E. F., C. F. Kleiner, and T. W. Thorsland. 1977. Effects on constant temperature and diel fluctuation on growth, mortality, and yield of juvenile rainbow trout, Salmo gairdneri (Richardson). Journal of the Fisheries Research Board of Canada 34:639-648.

Houston, J. J. 1988. Status of green sturgeon, Acipenser medirostris, in Canada. Canadian Field-Naturalist 102:286–290.

ICF International. 2016. Biological Assessment for the California WaterFix. July. (ICF 00237.15.) Sacramento, CA. Prepared for U.S. Department of the Interior, Bureau of Reclamation, Sacramento, CA.

Jarrett, D., and D. Killam. 2014. Redd Dewatering and Juvenile Stranding in the Upper Sacramento River, Year 2013-2014. RBFO Technical Report No. 01-2014. California Department of Fish and Wildlife. Sacramento, California.

Jarrett, D., and D. Killam. 2015. Redd Dewatering and Juvenile Stranding in the Upper Sacramento River, Year 2014-2015. RBFO Technical Report No. 02-2015. California Department of Fish and Wildlife. Sacramento, California.

King, R., Morgan, B., Gimenez, O. and Brooks, S. 2010. Bayesian analysis for population ecology. CRC Press, Boca Raton, FL.

Kondolf, G. M., T. Griggs, E. W. Larsen, S. McBain, M. Tompkins, J. G., Williams, and J. Vick. 2000. Flow regime requirements for habitat restoration along the Sacramento River between Colusa and Red Bluff. CALFED Bay-Delta Program, Integrated Storage Investigation, Sacramento, CA. Available: http://www.sacramentoriver.org/forum/index.php?id=library&rec=288. Accessed: 6/1/2015.

Leggett, William & WHITNEY, RICHARD. (1972). Water temperature and the migration of American shad. Fish. Bull. 70.

Martin, B., R. Nisbet, A. Pike, C. Michel, and E. Danner. (2015). Sport science for salmon and other species: Ecological consequences of metabolic power constraints. Ecology Letters 18. 10.1111/ele.12433.

Martin, B. T., A. Pike, S. N. John, N. Hamda, J. Roberts, S. T. Lindley, and E. M. Danner. 2017. Phenomenological vs. biophysical models of thermal stress in aquatic eggs. Ecology Letters 20(1):50-59.

May, C. L., B. Pryor, T. E. Lisle, and M. Lang. 2009. Coupling hydrodynamic modeling and empirical measures of bed mobility to predict the risk of scout and fill of salmon redds in a larger regulated river. Water Resources Research 45. Available: http://www.fs.fed.us/psw/publications/lisle/psw_2009_lisle(may)001.pdf. Accessed: 6/1/2015.

Mayfield, Ryan & Jr, Joseph. (2004). Temperature Effects on Green Sturgeon Bioenergetics. Transactions of The American Fisheries Society - TRANS AMER FISH SOC. 133. 961-970.

Meeuwig, Michael & H, Michael. (2002). Identification of Larval Pacific Lampreys (Lampetra Tridentata), River Lampreys (L. Ayresi) and Western Brook Lampreys (L. Richardson) and Thermal Requirements of Early Life History Stages of Lampreys: Annual Report 2002. 10.2172/821798.

Meeuwig, Michael & Bayer, Jennifer & Seelye, James. (2005). Effects of Temperature on Survival and Development of Early Life Stage Pacific and Western Brook Lampreys. Transactions of The American Fisheries Society - TRANS AMER FISH SOC. 134. 19-27. 10.1577/FT03-206.1.

McCullough D. A., S. Spalding, D. Sturdevant, M. Hicks. 2001. EPA Issue Paper 5: Summary of Technical Literature Examining the Physiological Effects of Temperature on Salmonids. EPA-910-D-01-005.

Moyle, P. B. 2002. Inland Fishes of California, Revised and Expanded. Berkeley: University of California Press.

Moyle, P. B, Baxter, R. D, Sommer, T., Foin, T. C, & Matern, S. A. (2004). Biology and Population Dynamics of Sacramento Splittail (Pogonichthys macrolepidotus) in the San Francisco Estuary: A Review. San Francisco Estuary and Watershed Science, 2(2).

Myrick, C.A and J.J. Cech, Jr. 2005. Temperature effects on juvenile anadromous salmonids in California’s central valley: what don’t we know? Review in Fish Biology and Fisheries 14:113-123.

National Marine Fisheries Service. 2009. Biological and Conference Opinion on the Long-Term Operations of the Central Valley Project and State Water Project. June 4. Southwest Region.

National Marine Fisheries Service (NMFS). 2016. Central Valley Recovery Domain 5-Year Review: Summary and Evaluation of California Central Valley Steelhead Distinct Population Segment. National Marine Fisheries Service, West Coast Region.

Thom, B. A. (2017). Endangered Species Act (ESA) Section 7 (a)(2) Biological Opinion and Magnuson-Stevens Fishery Conservation and Management Act Essential Fish Habitat (EFH) Consultation. Northern California Water Association. 2019. Re-managed Instream Flows in the Sacramento River Basin. Available: https://norcalwater.org/wp-content/uploads/2012/01/Re-managed-Instream-Flows-in-the-Sac-River-Basin.pdf. Accessed: May 20, 2021. Updated 2019.

Painter, R. L., L. Wixom, and L. Meinz. 1979. American Shad Management Plan for the Sacramento River Drainage. Anadromous Fish Conservation Act Project AFS-17, Job 5. Sacramento, CA: California Department of Fish and Game.

Painter, R. E., L. Wixom, and M. Meinz. 1980. Management Plan for American Shad (Alosa sapidissima) in Central California. Final Report Job No. 3, Anadromous Fish Conservation Act Project No. AFS-17. CDFG. (Cited by USBR, 1997).

Richter A. and S. A. Kolmes. 2005. Maximum Temperature Limits for Chinook, Coho, and Chum Salmon, and Steelhead Trout in the Pacific Northwest. Reviews in Fisheries Science, 13:1,23–49.

Seesholtz, Alicia & Manuel, Matt & Van Eenennaam, Joel. 2014. First documented spawning and associated habitat conditions for green sturgeon in the Feather River, California. Environmental Biology of Fishes. 98. 10.1007/s10641-014-0325-9.

Sites Project Authority and US Bureau of Reclamation. 2017. Draft Environmental Impact Report/Draft Environmental Impact Statement. State Clearinghouse #2001112009.

Sommer, T. R., W. C. Harrell, and M. L. Nobriga. 2005. Habitat Use and Stranding Risk of Juvenile Chinook Salmon on a Seasonal Floodplain. North American Journal of Fisheries Management 25:1493–1504.

Southern California Edison Company. 2007. ATTACHMENT H LIFE HISTORY AND HABITAT REQUIREMENTS OF FISH SPECIES IN THE PROJECT AREA. February 2007. Rosemead, CA.

Spiegelhalter D. J., N. G. Best, B. P. Carlin, A. van der Linde. 2002. Bayesian Measures of Complexity and Fit. Journal of the Royal Statistical Society SeriesB, 64, 583–639.

Sullivan K., D.J. Martin, R.D. Cardwell, J.E. Toll, and S. Duke. 2000. An analysis of the effects of temperature on salmonids of the Pacific Northwest with implications for selecting temperature criteria. Sustainable Ecosystems Institute. Portland, OR. 147 pp.

U.S. Environmental Protection Agency. 2003. EPA Region 10 Guidance for Pacific Northwest. State and Tribal Temperature Water Quality Standards. EPA 910-B-03-002. Region 10 Office of Water, Seattle, WA. 49 pp.

U.S. Fish and Wildlife Service. 1999. Effects of Temperature on Early-Life Survival of Sacramento River Fall-Run and Winter-Run Chinook Salmon. Final Report. Shepherdstown, WV.

U.S. Fish and Wildlife Service. 2003a. Flow-Habitat Relationships for steelhead and fall, late-fall, and winter-run Chinook salmon spawning in the Sacramento River between Keswick Dam and Battle Creek. February 4, 2003. Sacramento, CA. Available: http://www.fws.gov/sacramento/fisheries/Instream-Flow/Documents/Sacramento%20River%20Spawning%20Final%20Report%20Feb%204,%202003.pdf. Accessed: 6/1/2015.

U.S. Fish and Wildlife Service. 2003b. Comparison of PHABSIM and 2-D Modeling of Habitat for Steelhead and Fall-run Chinook Salmon Spawning in the Lower American River. February 4, 2003. Sacramento, CA. Available: http://www.fws.gov/sacramento/fisheries/Instream-Flow/Documents/American%20River%20PHABSIM%202D%20Final%20Report.pdf. Accessed: 6/1/15.

U.S. Fish and Wildlife Service. 2005b. Flow-Habitat Relationships for Chinook Salmon Rearing in the Sacramento River between Keswick Dam and Battle Creek. Available: http://www.fws.gov/sacramento/fisheries/Instream-Flow/Documents/Sacramento%20River%20Keswick%20Dam%20to%20Battle%20Creek%20Rearing%20Final%20Report.pdf. Accessed: 6/1/2015.

U.S. Fish and Wildlife Service. 2006. Sacramento River (Keswick Dam to Battle Creek) redd dewatering and juvenile stranding. June 22, 2006. Sacramento, CA. Available: http://www.fws.gov/sacramento/Fisheries/Instream-Flow/Documents/Sacramento%20River%20Keswick%20Dam%20to%20Battle%20Creek%20-%20redd%20dewatering%20and%20juvenle%20stranding%20Final%20Report%20.pdf. Accessed: 6/1/2015.

Van Eenennaam, J.P., Linares-Casenave, J., Deng, X., and Doroshov, S.I. 2005. Effect of incubation temperature on green sturgeon embryos, Acipenser medirostris. Environmental Biology of Fishes 72(2):145-154.

Wang, J. C. (1986). Fishes of the Sacramento-San Joaquin estuary and adjacent waters, California: A guide to the early life histories (Vol. 9). The Department.

Wurtsbaugh, W.A. and G.E. Davis. 1977. Effects of temperature and ration level on the growth and food conversion efficiency of rainbow trout, Salmo gairdneri, Richardson. Journal of Fish Biology, 11:99-104.

Zaugg, W.S. and H.H. Wagner. 1973. Gill ATPase activity related to parr-smolt transformation and migration in steelhead trout (Salmo gairdneri): Influence of photo-period and temperature. Comparative Biochemistry and Physiology Part B: Comparative Biochemistry, 45:955-965.

Zaugg, W. S. 1981. Advanced photoperiod and water temperature effects on gill Na+-K+ adenosine triphosphatase activity and migration of juvenile steelhead (Salmo gairdneri). Can. 5. Fisheries and Aquatic Science. 38:758-764.

Zeug, S., P. Bergman, B. Cavallo, and K. Jones. 2012. Application of a Life Cycle Simulation Model to Evaluate Impacts of Water Management and Conservation Actions on an Endangered Population of Chinook Salmon. Environmental Modeling and Assessment 17(5):455-467.

### Personal Communications

Gard, M. 2015. Personal communication by email. July 5, 2015—Email.

Kindopp pers comm

Kundargi K. August 12, 2015—Verbal explanation of flows in the American River below which migration issues occur for migrating adult salmonids.

Robinson D. June 16, 2015—Email from Don Robinson, ESSA Technologies to Clint Alexander, ESSA Technologies about the use of surrogate species in the SacEFT model.

Rich, A.A. 1987. Water temperatures which optimize growth and survival of the anadromous fishery resources of the lower American River. A.A. Rich and Associates. San Rafael, CA.

Seesholtz pers. comm.

Swank D., July 24, 2015—Email regarding a significant temperature increment for steelhead eggs and juveniles at which mortality increases quickly.

Thayer, Reed. Water Resources Engineer, Jacobs, Sacramento, CA. May 29, 2019—Excel file <Henderson_USRDOM_2030_With&WitouthProj.xlsx> sent by email to Marin Greenwood, Aquatic Ecologist, ICF, Sacramento, CA.