# Through-Delta Survival of Juvenile Salmonids

## Introduction

This appendix describes the through-Delta survival analysis of juvenile salmonids based on the model of Perry et al. (2018). The analysis was conducted through a spreadsheet implementation of the model that was provided by Perry (pers. comm.) which reproduces the mean response of the STARS (Survival, Travel time, And Routing Simulation) model estimating through-Delta survival of juvenile Chinook salmon as a function of Sacramento River flow at Freeport (Perry et al. 2019).

## Methods

The original Excel file <North Delta Routing Management Tool v2.1.xlsx> (Perry pers. comm.) was adapted to make through-Delta survival calculations for the No Action Alternative (NAA) and Alternatives 1, 2, and 3 CALSIM modeling outputs for the Sacramento River at Freeport. A lookup table of through-Delta survival in 1,000-cubic-feet-per-second increments was calculated (plotted in Figure 11J-1), with estimates of through-Delta survival for each month under each alternative being calculated based on the CALSIM modeling applied to the lookup table (with interpolation as necessary) for the months of September through June.

Notes: DCC = Delta Cross Channel; cfs = cubic feet per second.

Figure 11J-1. Through-Delta Survival Function Based on Perry et al. (2018).

The analysis was undertaken with the assumption that no Georgiana Slough Migratory Barrier was in place, and included a sensitivity analysis that assumed the presence of the barrier with a 50% reduction in entry into Georgiana Slough based on the approximate lower reduction observed during 2 years of effectiveness studies completed by California Department of Water Resources (2012, 2015).

## Results

The results of the analyses are discussed in Chapter 11, Aquatic Biological Resources. Table 11J-1 provides the results of the sensitivity analysis including 50% reduction into Georgiana Slough assumed to occur as a result of the Georgiana Slough Migratory Barrier implementation.

Table 11J-1. Probability of Juvenile Chinook Salmon Through-Delta Survival, Averaged by Month and Water Year Type, Based on Perry et al. (2018), Including 50% Reduction in Georgiana Slough Entry Assumed for Georgiana Slough Migratory Barrier.

| Month | Water Year Type | NAA | Alt 1A | Alt 1B | Alt 2 | Alt 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Sep | Wet | 0.42 | 0.42 (0%) | 0.42 (0%) | 0.42 (0%) | 0.42 (0%) |
| Sep | Above Normal | 0.42 | 0.42 (0%) | 0.42 (0%) | 0.42 (0%) | 0.42 (1%) |
| Sep | Below Normal | 0.32 | 0.33 (1%) | 0.33 (2%) | 0.33 (1%) | 0.33 (2%) |
| Sep | Dry | 0.29 | 0.30 (4%) | 0.30 (4%) | 0.30 (4%) | 0.30 (3%) |
| Sep | Critically Dry | 0.33 | 0.33 (0%) | 0.34 (3%) | 0.33 (-1%) | 0.34 (2%) |
| Oct | Wet | 0.40 | 0.40 (0%) | 0.40 (-1%) | 0.40 (0%) | 0.40 (0%) |
| Oct | Above Normal | 0.34 | 0.35 (1%) | 0.35 (1%) | 0.35 (1%) | 0.36 (5%) |
| Oct | Below Normal | 0.34 | 0.34 (2%) | 0.34 (2%) | 0.34 (2%) | 0.35 (4%) |
| Oct | Dry | 0.31 | 0.33 (5%) | 0.33 (5%) | 0.32 (4%) | 0.31 (-1%) |
| Oct | Critically Dry | 0.27 | 0.28 (6%) | 0.28 (5%) | 0.28 (5%) | 0.27 (2%) |
| Nov | Wet | 0.44 | 0.43 (-2%) | 0.43 (-2%) | 0.43 (-2%) | 0.43 (-2%) |
| Nov | Above Normal | 0.42 | 0.42 (0%) | 0.42 (0%) | 0.42 (0%) | 0.42 (1%) |
| Nov | Below Normal | 0.42 | 0.43 (2%) | 0.43 (2%) | 0.43 (2%) | 0.43 (2%) |
| Nov | Dry | 0.38 | 0.39 (3%) | 0.39 (2%) | 0.39 (1%) | 0.38 (1%) |
| Nov | Critically Dry | 0.32 | 0.32 (-1%) | 0.32 (0%) | 0.32 (0%) | 0.32 (0%) |
| Dec | Wet | 0.51 | 0.51 (0%) | 0.51 (0%) | 0.51 (0%) | 0.51 (0%) |
| Dec | Above Normal | 0.51 | 0.50 (-1%) | 0.50 (0%) | 0.50 (-1%) | 0.51 (0%) |
| Dec | Below Normal | 0.51 | 0.51 (0%) | 0.51 (0%) | 0.51 (0%) | 0.51 (1%) |
| Dec | Dry | 0.47 | 0.47 (0%) | 0.47 (0%) | 0.47 (0%) | 0.48 (0%) |
| Dec | Critically Dry | 0.42 | 0.42 (-1%) | 0.42 (-1%) | 0.42 (-1%) | 0.42 (-1%) |
| Jan | Wet | 0.63 | 0.63 (0%) | 0.63 (0%) | 0.63 (0%) | 0.63 (0%) |
| Jan | Above Normal | 0.59 | 0.59 (0%) | 0.59 (0%) | 0.59 (0%) | 0.59 (0%) |
| Jan | Below Normal | 0.53 | 0.52 (-1%) | 0.52 (-1%) | 0.52 (-1%) | 0.53 (0%) |
| Jan | Dry | 0.48 | 0.48 (0%) | 0.48 (0%) | 0.48 (0%) | 0.48 (0%) |
| Jan | Critically Dry | 0.47 | 0.47 (0%) | 0.46 (0%) | 0.47 (0%) | 0.46 (-1%) |
| Feb | Wet | 0.66 | 0.66 (0%) | 0.66 (0%) | 0.66 (0%) | 0.66 (0%) |
| Feb | Above Normal | 0.62 | 0.62 (0%) | 0.62 (0%) | 0.62 (0%) | 0.62 (0%) |
| Feb | Below Normal | 0.58 | 0.58 (-1%) | 0.58 (-1%) | 0.58 (-1%) | 0.58 (-1%) |
| Feb | Dry | 0.53 | 0.53 (0%) | 0.53 (0%) | 0.53 (0%) | 0.53 (0%) |
| Feb | Critically Dry | 0.49 | 0.49 (0%) | 0.49 (0%) | 0.49 (0%) | 0.49 (0%) |
| Mar | Wet | 0.63 | 0.63 (0%) | 0.63 (0%) | 0.63 (0%) | 0.63 (0%) |
| Mar | Above Normal | 0.62 | 0.62 (-1%) | 0.62 (-1%) | 0.62 (-1%) | 0.62 (-1%) |
| Mar | Below Normal | 0.53 | 0.52 (-2%) | 0.52 (-1%) | 0.52 (-1%) | 0.52 (-2%) |
| Mar | Dry | 0.51 | 0.51 (-1%) | 0.51 (-1%) | 0.51 (-1%) | 0.51 (-1%) |
| Mar | Critically Dry | 0.47 | 0.47 (0%) | 0.47 (0%) | 0.47 (0%) | 0.47 (0%) |
| Apr | Wet | 0.61 | 0.60 (0%) | 0.60 (0%) | 0.60 (0%) | 0.60 (0%) |
| Apr | Above Normal | 0.56 | 0.56 (0%) | 0.56 (0%) | 0.56 (0%) | 0.56 (0%) |
| Apr | Below Normal | 0.51 | 0.51 (0%) | 0.51 (0%) | 0.51 (0%) | 0.51 (0%) |
| Apr | Dry | 0.47 | 0.47 (0%) | 0.47 (0%) | 0.47 (0%) | 0.47 (0%) |
| Apr | Critically Dry | 0.44 | 0.44 (0%) | 0.44 (0%) | 0.44 (0%) | 0.44 (0%) |
| May | Wet | 0.58 | 0.58 (0%) | 0.58 (0%) | 0.58 (0%) | 0.58 (0%) |
| May | Above Normal | 0.53 | 0.53 (0%) | 0.53 (0%) | 0.53 (0%) | 0.53 (0%) |
| May | Below Normal | 0.49 | 0.49 (0%) | 0.49 (0%) | 0.49 (0%) | 0.49 (0%) |
| May | Dry | 0.45 | 0.45 (0%) | 0.45 (0%) | 0.45 (0%) | 0.45 (0%) |
| May | Critically Dry | 0.42 | 0.42 (0%) | 0.42 (0%) | 0.42 (0%) | 0.42 (0%) |
| Jun | Wet | 0.46 | 0.46 (0%) | 0.46 (-1%) | 0.46 (0%) | 0.46 (-1%) |
| Jun | Above Normal | 0.39 | 0.39 (-2%) | 0.39 (-2%) | 0.39 (-2%) | 0.39 (-2%) |
| Jun | Below Normal | 0.36 | 0.36 (-1%) | 0.36 (-1%) | 0.36 (-1%) | 0.36 (-1%) |
| Jun | Dry | 0.36 | 0.36 (0%) | 0.36 (0%) | 0.36 (0%) | 0.36 (0%) |
| Jun | Critically Dry | 0.32 | 0.32 (0%) | 0.32 (0%) | 0.32 (0%) | 0.32 (0%) |

Note: Percentage values in parentheses indicate differences of alternatives compared to NAA.

## References

### Printed References

California Department of Water Resources. 2012. 2011 Georgiana Slough Non-Physical Barrier Performance Evaluation Project Report. Sacramento, CA: Bay-Delta Office, California Department of Water Resources.

California Department of Water Resources. 2015. 2012 Georgiana Slough Non-Physical Barrier Performance Evaluation Project Report. Sacramento, CA: Bay-Delta Office, California Department of Water Resources.

Perry, R. W., A. C. Pope, J. G. Romine, P. L. Brandes, J. R. Burau, A. R. Blake, A. J. Ammann, C. J. Michel. 2018. Flow-Mediated Effects on Travel Time, Routing, and Survival of Juvenile Chinook Salmon in a Spatially Complex, Tidally Forced River Delta. Canadian Journal of Fisheries and Aquatic Sciences 75(11):1886–1901.

Perry, R. W., A. C. Hansen, S. D. Evans, and T. J. Kock. 2019. Using the STARS Model to Evaluate the Effects of Two Proposed Projects for the Long-Term Operation of the State Water Project Incidental Take Permit Application and CEQA Compliance. Open-File Report 2019-1127. Version 2.0. February. U. S. Geological Survey, Reston, VA.

### Personal Communications

Perry, Russell. Research Fisheries Biologist, Quantitative Fisheries Ecology Section, USGS Western Fisheries Research Center, Columbia River Research Laboratory, Cook, WA. June 18, 2019—Email containing Excel file <North Delta Routing Management Tool v2.1.xlsx> sent to Marin Greenwood, Aquatic Ecologist, ICF, Sacramento, CA.

The higher reduction (2011) was ~67%.