# Soil Survey Map Unit Soil Properties

Table 12B-1. Soil Survey Map Unit Soil Properties

| Map Symbol | Map Unit Name | Erosion Hazard (Off-Road, Off-Trail) | Linear Extensibility | Corrison of Concrete | Corrosion of Steel | Septic Tank Absorption Fields |
| --- | --- | --- | --- | --- | --- | --- |
| 100 | Water and Fluvents, 0 to 2 percent slopes | Not rated | No Data | No Data | No Data | Not rated |
| 102 | Aiken family-Dumps, mine tailings association, 2 to 30 percent slopes. | Moderate | 2.7 | Moderate | High | Very limited |
| 128 | Gilligan-Chawanakee families association, 30 to 90 percent slopes. | Severe | 1.4 | Moderate | Low | Very limited |
| 144 | Holland-Skalan families association, 30 to 70 percent slopes. | Severe | 3.9 | Moderate | Moderate | Very limited |
| 145 | Inville family, 15 to 50 percent slopes. | Moderate | 1.3 | Moderate | Low | Very limited |
| 147 | Inville-Wintoner families association, 30 to 50 percent slopes. | Moderate | 1.3 | Moderate | Low | Very limited |
| 155 | Lithic Mollic Haploxeralfs-Dubakella family association, 15 to 70 percent slopes. | Moderate | 1.2 | Low | Low | Very limited |
| 200 | Orset sandy loam, 0 to 9 percent slopes | Slight | 1.5 | Moderate | Low | Very limited |
| 204 | Pinehurst variant very stony loam, 0 to 15 percent slopes | Slight | 2.7 | Low | Low | Very limited |
| 205 | Pinehurst variant very stony loam, 15 to 65 percent slopes | Slight | 2.7 | Low | Low | Very limited |
| 206 | Pit clay | Slight | 6.4 | Moderate | High | Very limited |
| 210 | Skalan-Goldridge-Clallam families association, deep, 20 to 70 percent slopes | Severe | 3.6 | Moderate | Moderate | Very limited |
| 211 | Clallam family, moderately deep, 30 to 50 percent slopes | Severe | 1.3 | Moderate | Low | Very limited |
| 213 | Rock outcrop-Dubakella complex, 30 to 50 percent slopes | Not rated | No Data | No Data | No Data | Not rated |
| 215 | Clallam family, moderately deep, unstable, 50 to 70 percent slopes | Very Severe | 1.2 | Moderate | Low | Very limited |
| 216 | Rock outcrop | Not rated | No Data | No Data | No Data | Not rated |
| 218 | Salisbury clay loam, 2 to 9 percent slopes | Slight | 3.5 | Low | High | Very limited |
| 220 | Ferndale, 0 to 2 percent slopes | Slight | 4 | Low | Moderate | Very limited |
| 221 | Canalschool, 0 to 2 percent slopes | Slight | 5 | Low | High | Very limited |
| 230 | Skalan-Goldridge families complex, deep, 20 to 65 percent slopes | Severe | 3.6 | Moderate | Moderate | Very limited |
| 241 | Tullycreek, 15 to 30 percent slopes | Severe | 4 | Moderate | High | Very limited |
| 255 | Carlotta, 0 to 2 percent slopes | Slight | 1.5 | Moderate | High | Very limited |
| 257 | Lepoil-Candymountain complex, 2 to 15 percent slopes | Moderate | 2.2 | High | Moderate | Very limited |
| 261 | Tillas, 2 to 9 percent slopes | Slight | 3.6 | Moderate | High | Very limited |
| 300 | Rock outcrop-Xerorthents complex,metaigneous, 60 to 90 percent slopes | Not rated | No Data | No Data | No Data | Not rated |
| 305 | Ashtre-Nutzan-Ashdos association | Moderate | 1.4 | Low | Moderate | Very limited |
| 315 | Bombadil-Chime association | Slight | 2.3 | Low | Low | Very limited |
| 316 | Aiken-Holland families complex, deep, 10 to 40 percent slopes | Very Severe | 4 | Moderate | High | Very limited |
| 320 | Hugo family, moderately deep-Maymen family complex, 30 to 50 percent slopes | Severe | 1.3 | High | Moderate | Very limited |
| 329 | Bucklake-Rock outcrop-Corral association | Moderate | 4.2 | Low | Moderate | Very limited |
| 330 | Bucklake-Softscrabble-Devada association | Slight | 4.3 | Low | Moderate | Very limited |
| 331 | Clallam family, moderately deep-Skalan family, deep association, 35 to 75 percent slopes | Severe | 1.3 | Moderate | Low | Very limited |
| 332 | Bullump very stony loam, 5 to 30 percent slopes | Slight | 1.5 | Low | Low | Very limited |
| 334 | Buntingville ashy loam, 2 to 5 percent slopes | Slight | 4.3 | Low | Moderate | Very limited |
| 652 | Schott-Rock outcrop, 30 to 50 percent slopes | Moderate | 3.6 | Low | Moderate | Very limited |
| AaA | Altamont clay, 0 to 3 percent slopes | Slight | 5.4 | Low | High | Very limited |
| AaC | Aiken loam, 8 to 15 percent slopes | Moderate | 4.1 | Moderate | High | Very limited |
| AdD | Altamont soils, 15 to 30 percent slopes | Severe | 5.8 | Low | High | Very limited |
| AdE | Altamont soils, 30 to 65 percent slopes | Severe | 5.8 | Low | High | Very limited |
| AfsD | Altamont-Gullied land complex, shallow, 10 to 30 percent slopes | Not rated | 3.5 | No Data | No Data | Not rated |
| AfsE | Altamont-Gullied land complex, shallow, 30 to 65 percent slopes | Not rated | 3.5 | No Data | No Data | Not rated |
| AgE | Altamont-Rocky gullied land complex, 15 to 45 percent slopes | Severe | 4.5 | Low | High | Very limited |
| AhC | Altamont-Contra Costa clays, 8 to 15 percent slopes | Moderate | 5.4 | Low | High | Very limited |
| AhD | Altamont-Contra Costa clays, 15 to 30 percent slopes | Moderate | 5.4 | Low | High | Very limited |
| AkE3 | Millsholm-Altamont-Rock outcrop complex, 10 to 55 percent slopes, severely eroded, MLRA 15 | Severe | 1 | Low | Low | Very limited |
| AmC | Altamont-Nacimiento association, 3 to 15 percent slopes | Slight | 5.4 | Low | High | Very limited |
| BrA | Bidwell sandy loam, 0 to 2 percent slopes | Slight | 2.5 | Moderate | High | Very limited |
| Ca | Capy silty clay, 0 percent slopes, MLRA 17 | Slight | 8.9 | Low | High | Very limited |
| CaA | Calpine coarse sandy loam, 0 to 2 percent slopes | Slight | 1.5 | Moderate | High | Not limited |
| CaB | Calpine coarse sandy loam, 2 to 5 percent slopes | Slight | 1.5 | Moderate | High | Not limited |
| Cb | Chummy soils, 0 to 3 percent slopes | Slight | 4 | Moderate | High | Very limited |
| CeB | Churn gravelly loam, 3 to 8 percent slopes | Slight | 3.8 | Moderate | Moderate | Very limited |
| Ck | Cobbly alluvial land, frequently flooded | Not rated | 1.5 | No Data | No Data | Very limited |
| HgA | Honn fine sandy loam, 0 to 3 percent slopes | Slight | 3.1 | Low | Moderate | Very limited |
| HgB | Honn fine sandy loam, 3 to 8 percent slopes | Moderate | 3.1 | Low | Moderate | Very limited |
| Hl | Hillgate silt loam, 0 to 3 percent slopes | Slight | 4.7 | Low | Moderate | Very limited |
| KmA | Kimball gravelly loam, 0 to 2 percent slopes | Slight | 4.2 | Low | Moderate | Very limited |
| KmB | Kimball gravelly loam, 2 to 10 percent slopes | Slight | 4.2 | Low | Moderate | Very limited |
| MmD | Millsholm-Rock outcrop-Gullied land complex, 5 to 30 percent slopes, MLRA 15 | Moderate | 1.7 | Low | Low | Very limited |
| MmE | McCarthy stony sandy loam, 30 to 50 percent slopes | Moderate | 1.3 | Moderate | High | Very limited |
| MnD | Millsholm silt loam, 10 to 30 percent slopes, MLRA 15 | Severe | 2.9 | Low | Moderate | Very limited |
| MoD | Millsholm silt loam, 3 to 25 percent slopes, MLRA 15 | Moderate | 2.9 | Low | Moderate | Very limited |
| MtD | Millsholm loam, 10 to 30 percent slopes, MLRA 15 | Severe | 2.3 | Low | Moderate | Very limited |
| MuE | Millsholm very gravelly coarse sandy loam, 30 to 50 percent slopes, MLRA 15 | Slight | 0.8 | Moderate | Low | Very limited |
| MzrA | Myers clay, 0 to 1 percent slopes, MLRA 17 | Slight | 10.3 | Low | High | Very limited |
| MzrB | Myers clay, 3 to 10 percent slopes | Slight | 7.5 | Low | High | Very limited |
| MzxB | Myers-Gullied land complex, 3 to 10 percent slopes | Not rated | 7.5 | No Data | No Data | Not rated |
| MzyA | Myers clay loam, 0 to 3 percent slopes | Slight | 7.5 | Low | High | Very limited |
| MzyB | Myers clay loam, 3 to 8 percent slopes | Slight | 7.5 | Low | High | Very limited |
| NaC | Nacimiento clay, 3 to 15 percent slopes | Moderate | 5.1 | Low | High | Very limited |
| NcD | Nacimiento-Altamont complex, 10 to 30 percent slopes | Severe | 2.9 | Low | Moderate | Very limited |
| NcE | Nacimiento soils, 30 to 50 percent slopes | Severe | 5.1 | Low | High | Very limited |
| NdE | Neuns very stony loam, 8 to 50 percent slopes | Moderate | 1.3 | Moderate | Moderate | Very limited |
| NhC | Nacimiento-Contra Costa association, 3 to 15 percent slopes | Moderate | 4.5 | Low | High | Very limited |
| NhD | Nacimiento-Newville complex, 10 to 30 percent slopes | Severe | 2.9 | Low | Moderate | Very limited |
| NhE | Nacimiento-Newville complex, 30 to 50 percent slopes | Severe | 2.9 | Low | Moderate | Very limited |
| Pg | Plaza silty clay loam | Slight | 4.5 | Moderate | High | Very limited |
| Rg | Red Bluff gravelly loam, 0 to 3 percent slopes | Slight | 3.5 | Moderate | High | Very limited |
| Rma | Riz silt loam, slightly saline-alkali | Slight | 5.4 | Moderate | High | Very limited |
| Rmb | Riz silt loam, moderately saline-alkali | Slight | 5.4 | Moderate | High | Very limited |
| Rnc | Riz silty clay loam, strongly saline-alkali | Slight | 5.8 | High | High | Very limited |
| SbC | Sehorn soils, 3 to 15 percent slopes | Moderate | 5 | Low | High | Very limited |
| SbE | Sehorn soils, 30 to 65 percent slopes | Severe | 5 | Low | High | Very limited |
| Sc | Sacramento clay, 0 to 2 percent slopes, MLRA 17 | Slight | 12.1 | Moderate | High | Very limited |
| ScE | Sehorn silty clay, 30 to 50 percent slopes | Very Severe | 6.7 | Low | High | Very limited |
| SdD | Sehorn-Millsholm association, 3 to 30 percent slopes, MLRA 15 | Moderate | 6.1 | Low | High | Very limited |
| SdE | Sehorn-Millsholm association, 15 to 50 percent slopes, MLRA 15 | Severe | 6 | Low | High | Very limited |
| SeD | Sehorn silty clay, moderately deep, 8 to 30 percent slopes | Severe | 6.3 | Low | High | Very limited |
| SmD | Sierra sandy loam, 15 to 30 percent slopes, LRU 15XF | Severe | 2.4 | Moderate | Moderate | Very limited |
| SmE2 | Sehorn-Balcom complex, 15 to 30 percent slopes, eroded | Moderate | 4.8 | Low | High | Very limited |
| Ss | Stockton clay, moderately deep, frequent overflow | Slight | 3.8 | Low | High | Very limited |
| Su | Sycamore complex | Slight | 4.5 | Low | High | Very limited |
| Sv | Sycamore complex, drained | Slight | 4.5 | Low | Moderate | Very limited |
| TaA | Tehama loam, 0 to 3 percent slopes, MLRA 17 | Slight | 2 | Moderate | Low | Very limited |
| Tb | Tehama gravelly loam, 0 to 3 percent slopes, MLRA 17 | Slight | 2.6 | Low | Moderate | Very limited |
| TcB | Tehama clay loam, 2 to 10 percent slopes | Slight | 4.5 | Low | Moderate | Very limited |
| Tm | Tehama silt loam, 0 to 3 percent slopes, MLRA 17 | Slight | 3.7 | Moderate | Moderate | Very limited |
| W | Water | Not rated | No Data | No Data | No Data | Not rated |
| Wca | Willows clay, slightly saline-alkali | Slight | 7.5 | High | High | Very limited |
| Wcb | Willows clay, moderately saline-alkali | Slight | 7.5 | High | High | Very limited |
| Ya | Yolo silt loam, 0 to 2 percent slopes, MLRA 17 | Slight | 3.2 | Low | Low | Very limited |
| Yd | Yolo clay loam, moderately deep over clay | Slight | 5.9 | Low | High | Very limited |
| Yh | Yolo clay loam, shallow over clay | Slight | 6.5 | Low | High | Very limited |
| Yma | Yolo clay loam, slightly saline-alkali | Slight | 6.5 | Moderate | High | Somewhat limited |
| ZbA | Zamora silty clay loam, 0 to 3 percent slopes, MLRA 17 | Slight | 5.7 | Low | Moderate | Very limited |
| ZbB | Zamora silty clay loam, 2 to 8 percent slopes | Slight | 4.5 | Low | Moderate | Somewhat limited |

Source: Natural Resources Conservation Service 2020